| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0336 |
| --- | --- |

| Title: | Non-SCE1 : Cross check of supplementary results on category 2 (M0311) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Junghye Min Elena Alshina | Email: | jh643.min@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification of experiment 6, 8 and 9 from the proposal M0311 (Non-SCE1: supplementary results on category 2). The source code was reviewed and it was confirmed that implementation is consistent with algorithm descriptions in the proposal. BD-rate and PSNR results from our tests exactly match the results provided by proponents. All three combinations provide performance gains larger than -1.0% in luma component .Encoding/Decoding time increment is about 65%/14%.

# Introduction

LG, Sony and Qualcomm proposed several intra prediction methods using difference samples. In M0311, test results including possible combinations of these tools were presented.

Three experiments verified in this reports are combinations of following three intra tools with difference samples.

- Modified textureRL : Boundary process is performed to inter-layer texture prediction. Boundary process is performed using difference between samples of base layer and enhancement layer.

- Modified Planar : Right-top and bottom-left reference samples are set to zero for planar prediction.

- Modified DC : Similar to Modified textureRL. Only offset values for boundary process are different.

Combinations verified in this report are as follows.

|  | Expt. No. | textureRL | Conventional intra prediction |  | Intra prediction based on difference samples |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PL | DC | PL | DC |
| explicit signaling | 6 | △ | √ | √ | √ | √ |
|  | 8 | √ | √ | √ | √ | △ |
|  | 9 | △ | √ | √ | △ | △ |
| √ : old prediction method |  |  |  |  |  |  |
| △ : modified prediction method |  |  |  |  |  |  |

# Test results

The tests were performed under the conditions defined in JCTVC-L1101 [2] with AI 2x and AI 1.5x configurations. The bit rate and PSNR of our test results match with results proponents provided. Encoding/Decoding run times are similar with our results.  Simulation results are summarised in following tables.

Table 1 results of experiment 6

Table 2 results of experiment 8

Table 3 results of experiment 9

# Conclusion

Three experiments from combinations of several intra tools with difference samples were verified. All three combinations provide performance gains larger than -1.0% in luma component .Encoding time increment is about 65% where decoding time is increased by about 14%.

# References

[1] J. Park, J. Kim, B. Jeon, C. Auyeung, A.Tabatabai, K. Rapaka, J.Chen, X.Li, M.Karczewicz, Non-SCE1 : supplementary results on category 2, JCTVC-M0311, 2013

[2] A. Tabatabai, K. Rapaka, A. Saxena and S. Liu, Description of Core Experiment SCE 1: Intra Prediction Improvements in SHVC, JCTVC-L1101, 2013

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).