| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0337 |
| --- | --- |

| Title: | SCE4: cross-check for SCE4: Results of Test 4.2.1 on High Frequency Pass Inter Layer Sample Adaptive Offset Filter (JCTVC-M0265) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification of test 4.2.1 in SCE4 which is high-frequency pass inter-layer SAO. Gain from verified tool is higher in SNR test: 1,8%  Luma and 1,2% (IntraBL)/1,5% (RefIdx) Chroma BD-rate reduction. In spatial scalability tests average gain is 0,5% Luma and 0,9% (IntraBL)/0,7% (RefIdx) Chroma BD-rate reduction. This gain is accomplished by memory access increment 11%-12% in MC tests and 48%-63% in “all-intra” tests. Decoding time increment is 11-16% (‘all-intra”) and 21-34% (MC tests).

# Introduction

Originally high-frequency pass inter-layer SAO was proposed in [1] and was included into SCE4. Compare to original proposal and SCE description [2] method was extended for Chroma (which actually makes comparison with other SCE4 tools difficult).

The key idea of algorithm is application SAO for high-frequency component of inter-layer prediction.  So additionally to up-sampling filtration stage low-pass filtering is needed for separation of high-frequency components. Inter-layer SAO parameters are signalled per picture. Since only one SAO type per picture is not enough for efficient correction in verified implementation 2 SAO processes (with different types) are cascaded. After high frequency component of inter-layer texture prediction is processes then low and high frequency components are merged again. So for each inter-layer texture prediction block at least 6 passes are needed (compare to 2 passes: horizontal and vertical up-sampling, in SHM1.0).

# Test results

The tests were performed under the conditions defined in [3] for both IntraBL and refIdx frameworks for all mandatory and optional test scenarios. Performance test summary is shown in Table 1. More details can be found in excel spreadsheets attached to this report.

Gain from verified tool is higher in SNR test: 1,8%  Luma and 1,2% (IntraBL)/1,5% (RefIdx) Chroma BD-rate reduction. In spatial scalability tests average gain is 0,5% Luma and 0,9% (IntraBL)/0,7% (RefIdx) Chroma BD-rate reduction. This gain is accomplished by memory access increment 11%-12% in MC tests and 48%-63% in “all-intra” tests. Decoding time increment is 11-16% (‘all-intra”) and 21-34% (MC tests).

Gain is more or less the same for both IntraBL and RefIdx frameworks.

Table 1. Performance test summary.

| Tool | AI Luma | AI Chroma | MC Luma | MC Chroma | SNR Luma | SNR Chroma | Enc. Time | Dec. Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraBL framework |  |  |  |  |  |  |  |  |
| 4.2.1 | -0,22% | -0,50% | -0,64% | -1,10% | -1,76% | -1,18% | 99% | 125% |
| RefIdx framework |  |  |  |  |  |  |  |  |
| 4.2.1 | -0,22% | -0,41% | -0,61% | -0,88% | -1,78% | -1,47% | 100% | 127% |

According to [2] complexity assessment was done using methodology [4]. Results of complexity assessment are summarized in Table 2. More details can be found in excel spreadsheets attached to this report.

Memory access according to [4] was measured for 3 different memory architectures: pure access, DDR-2, DDR-3. Number of multiplications is also reported. Average memory access relatively to anchor is 111-112% in MC test scenarios and 145-163% in Intra Slices.

Table 2. Complexity assessment for IntraBL framework.

|  | All Intra tests |  |  |  | Test scenarios with MC (RA, LD-P) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tool | Pure | DDR-2 | DDR-3 | Mults | Pure | DDR-2 | DDR-3 | Mults |
| IntraBL framework |  |  |  |  |  |  |  |  |
| 4.2.1 | 145% | 152% | 162% | 127% | 111% | 112% | 112% | 105% |
| RefIdx framework |  |  |  |  |  |  |  |  |
| 4.2.1 | 148% | 155% | 163% | 127% | 111% | 112% | 112% | 105% |

# Conclusion

Performance and complexity assessment for high-frequency pass inter-layer SAO is verified. Gain from verified tool is higher in SNR test: 1,8%  Luma and 1,2% (IntraBL)/1,5% (RefIdx) Chroma BD-rate reduction. In spatial scalability tests average gain is 0,5% Luma and 0,9% (IntraBL)/0,7% (RefIdx) Chroma BD-rate reduction. This gain is accomplished by memory access increment 11%-12% in MC tests and 48%-63% in “all-intra” tests. Decoding time increment is 11-12% (‘all-intra”) and 21-34% (MC tests).

# References

[1] W. Pu, J. Chen, K. Rapaka, M. Karczewicz (Qualcomm), High Frequency Pass Inter Layer Sample Adaptive Offset Filter, Document of Joint Collaborative Team on Video Coding, JCTVC-L0234, January, 2013

[2] J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, and J. Park, “Description of Tool Experiment B4: Inter-layer filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1104, Oct. 2012

[3] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013.

[4] E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440 document, January, 2013.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).