| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0338 |
| --- | --- |

| Title: | SCE4: cross-check for Chroma Enhancement (JCTVC-M0183) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification of test 4.2.4 in SCE4 which is so-called chroma enhancement. An average gain from verified tool is 0,4%  Luma and 7,6% (IntraBL)/8,1% (RefIdx) Chroma BD-rate reduction. This gain is accomplished by very small memory access increment 1% in “MC tests” and up to 7-8% in “all-intra” tests. Amount of multiplications increases by 7% in “MC tests” and up to 40% in “all-intra” tests. Decoding time increment is 8% in average.

# Introduction

Originally cross-color inter-layer filtering (proponents call it also “Chroma enhancement”) was proposed by Inter Digital in [JCTVC-L0059]. This tool was included into SCE4.

Tool works both for IntraBL and RefIdx frameworks and requires access to only reconstructed collocated base-layer picture (which is any-way needed for inter-layer texture prediction).

Cross-color inter-layer filtering is implemented as 2D no-separable 4x3 filter, which refers to up-sampled Luma signal in order to reduce the difference between up-sampled and original Chroma signal (as shown on Figure 1).

Fig. 1. Cross-color inter-layer filter: circles – Luma, stars –Chroma, red star – current Chroma sample, red dots surround Luma samples taken into account for current Chroma sample processing.

Separable filters are very friendly for SIMD implementation. 2D non-separable filter from Fig. 1 is equivalent to 3 times application of 1D 4 taps filter. So compare to 2D separable up-sampling filter in SHM1.0 (which is 4 taps) amount of operations will be ~1.5 times higher.

Variable filter coefficients are not friendly for implementation. But this tools was proposed as additional inter-layer filter (not up-sampling filter which is key part of SHVC design), which could be switched on and off in SPS.

Samples processing is performed as follows

. (1)

Values ,  and  are signaled in slice-header for Cb and Cr (separately). Additionally tool can be switched On and Off Cb and Cr separately (with corresponding flag encoding in slice header).

Since value  is signaled in slice-header implementation of (1) requires variable de-scaling shift, which is not friendly for implementation.

If slice level on/off flag for this tool is false then Chroma samples will not be modified. The same will happen if  in (1) is equal to 0. So there is some syntax redundancy for verified tool.

# Test results

The tests were performed under the conditions defined in [3] for both IntraBL and refIdx frameworks for all mandatory and optional test scenarios. Performance test summary is shown in Table 1. More details can be found in excel spreadsheets attached to this report.

Luma gain from verified tool is higher in “all-intra” test.

Table 1. Performance test summary.

| Tool | ALL Luma | ALL Chroma | AI Luma | AI Chroma | MC Luma | MC Chroma | SNR Luma | SNR Chroma |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraBL framework |  |  |  |  |  |  |  |  |
| 4.2.4 v1 | -0,38% | -7,57% | -0,77% | -7,63% | -0,23% | -8,23% | -0,27% | -6,18% |
| RefIdx framework |  |  |  |  |  |  |  |  |
| 4.2.4 v1 | -0,40% | -8,14% | -0,79% | -8,31% | -0,25% | -8,73% | -0,28% | -6,79% |

According to [2] complexity assessment was done using methodology [4]. Results of complexity assessment are summarized in Table 2. More details can be found in excel spreadsheets attached to this report.

Both gain and complexity are more or less the same for both IntraBL and RefIdx frameworks.

Table 2. Complexity assessment for IntraBL framework.

|  | All Intra tests |  |  |  | Test scenarios with MC (RA, LD-P) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tool | Pure | DDR-2 | DDR-3 | Mults | Pure | DDR-2 | DDR-3 | Mults |
| IntraBL framework |  |  |  |  |  |  |  |  |
| 4.2.1 | 108% | 101% | 101% | 140% | 101% | 101% | 101% | 107% |
| RefIdx framework |  |  |  |  |  |  |  |  |
| 4.2.1 | 107 % | 101% | 101% | 140% | 101% | 101% | 101% | 107% |

In this report we verify performance and complexity of so-called “version 1” of test 4.2.4. It was release first. Later proponent release another version of tool, which has only encoder modifications compare to verified test.

# Conclusion

Performance and complexity assessment fo test 4.2.4 in SCE4 ( so-called chroma enhancement) are verified. An average gain from verified tool is 0,4%  Luma and 7,6% (IntraBL)/8,1% (RefIdx) Chroma BD-rate reduction. This gain is accomplished by very small memory access increment 1% in “MC tests” and up to 7-8% in “all-intra” tests. Amount of multiplications increases by 7% in “MC tests” and up to 40% in “all-intra” tests. Decoding time increment is 8% in averageReferences

[1] J. Dong, Y. He, Y. Ye, Chroma enhancement for ILR picture , Document of Joint Collaborative Team on Video Coding, JCTVC-L0059 document, January, 2013

[2] J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, and J. Park, “Description of Tool Experiment B4: Inter-layer filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1104, Oct. 2012

[3] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013.

[4] E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440 document, January, 2013.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).