| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0339 |
| --- | --- |

| Title: | SCE3: cross-check for SCE3: Results of Test 3.3 on Generalized Residual Prediction (case 2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin | Email: | elena_a.alshina@samsung.com alexander_b.alshin@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification of test 3.3 in SCE3 which is so-called Generalized Residual Prediction. An average BD-rate gain from verified tool is 3,9%(Luma)/4,9%(Chroma). This gain is accomplished by 30-32% increment of memory access in average. In the worst case the memory access is 6-8% higher than HEVC worst case (despite restrictions of GRP application). In our measurements run time increment is 27% for Encoder and 32% for Decoder.

# Introduction

Generalized Residual Prediction is the term used tools where combined prediction is constructed as follows:

Pred’EL = PredEL + ω (BaseCol – PredBL);                 (1)

Here

- PredEL- prediction from enhancement layer (MC prediction in verified test);

- BaseCol – collocated base-layer block (processed by up-sampling filter and additional filtration for int-pels which has de-noising effect)

- PredBL- prediction from base layer (with the MV from EL);

Compare to SHM1.0 which uses PredEL and  BaseCol (but w/o int-pel processing) these is new term PredBL which requires access to the base-layer reference frames. This is significant change of design. It is easy to implement GRP in reference s/w, but in actual implementations BL reference frames are not mandatory available during EL decoding. In terms of number of reference frames we may say that SHM1.0 enhancement layer decoding uses 5 reference frame (4 for EL MC and 1 is collocated BL picture for inter-layer texture prediction). For GRP the number of reference frames goes to 9. So comparison with anchor is not 100% fair.

Depending on weight ω there are several GRP modes switched on CU level.

In verified test (1) was implemented w/o additional MC on BL: first reference BL block is up-sampled (with extension necessary for further MC) and then (RefEL - ω RefBL) is processed by MC interpolation filter (MV is the same). It should be mentioned that the input of this MC process (RefEL - ω RefBL) in the worst case has twice wide diapason compare to regular MC and the system of de-scaling shifts, clips needs special design for this input processing.

In order to reduce complexity of up-sampling and MC process in GRP modes up-sampling filter length is shorter (4 taps) and MC filter applied for references difference is bi-linear.

Up-sampling filter applied for both Luma and Chroma in GRP mode is 4 taps but different from Chroma 4-taps up-sampling filter in SHM1.0 (including bit-depth of filter coefficients). So this is pure addition of new processing module to the SHVC. Even additional filters are short (4 and 2 taps) total LUT sizes for filter coefficients storage increases 46% with GRP.

GRP up-sampling filter has essentially different spectral characteristics from inter-layer texture prediction filter and applied for int-pel positions. At least for SNR test it was shown that application of de-noising filter for int-pel positions provides 1,4% avg. gain ([4]).

Switching between inter-layer texture prediction and GRP CUs is a kind of CU-base adaptive up-sampling filter. Adding CU on/off option to [4] provided 0,7% additional gain. Shorter MC filter in GRP mode also works as kind of adaptive MC filter.

So not small part of GRP gain in verified test comes not from combined prediction (1) but from CU-based adaptive up-sampling and MC filters. This part of GRP gain can be achieved without accessing BL reference frames.

As mentioned about (1) requires access to at least 3 difference storages: RefEL, RefBL and BaseCol. In order to limit memory access in the worst case GRP application is restricted in verified to for small blocks (restriction is different for uni- and bi-directional MC prediction s in (1)). Despite these restrictions the worst case memory access for GRP exceeds HEVC the worst case. We would suggest not breaking this limit and restricting GRP more (keep the worst case within HEVC limit).

Even in a worst case GRP memory access is only 6-8% higher compare to HEVC MC the actual memory access due to GRP (was measured using JCTVC-L0440 s/w module) increment is 30-32%.

As was shown in [3] currently memory access in SHM1.0 is 4-7% lower than for HEVC simulcast single layer coding. So, with GRP SHVC would become more complicated (from view point of memory access) compare to HEVC simulcast.

# Test results

Source code for verified test from 3 companies (Qualcomm, Canon and Nokia) arrived before SCE3 dead-line. Since tool requires low-level changes (CU base signaling) it was tested vs IntraBL SHM1.0 branch.

The tests were performed under the conditions defined in [1]. Performance test summary is shown in Table 1. More details can be found in excel spreadsheets attached to this report. Additionally several tools presented in SCE3 and non-SCE3 contributions are listed in Table 1. No of these tools requires access BL reference. No of these tools use adaptive switching of up-sampling or MC filters.

Table 1. Performance test summary.

| Tool | ALL Luma | ALL Chroma | MC Luma | MC Chroma | SNR Luma | SNR Chroma | Enc. Time | Dec. Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GRP case 2 | -3,91% | -4,87% | -3,19% | -4,03% | -4,63% | -5,70% | 127% | 132% |
| SCE3 -Test 3.2 [6] | -1,24% | -5,16% | -0,85% | -3,94% | -1,63% | -6,37% | 114% | 100% |
| JCTVC-M0090 [5] | -1,50% | -6,88% | -1,22% | -6,35% | -1,79% | -7,42% | 103% | 101% |

There are several observations.

GRP is used for Chroma, but other 2 tools mentioned in Table 1 – not. But both 3 show impressive Chroma BD-rate gain. Even Chor gain from Luma-only tools is higher than Chroma gain from GRP.

All tools in Table 1 have de-noising effect so gain in SNR test is higher them all (positive effect of de-noising for SNR scalability was studied in SCE4-test 4.1.1 and 2,13% Luma BD-rate gain was demonstrated).

The complexity assessment was done using methodology [2]. Results of complexity assessment are summarized in Table 2 (average complexity which is price we pay for the gain we have) and Table 3 (the worst case which is implementation cost measure). More details can be found in excel spreadsheets attached to this report.

Table 2. Complexity assessment summary.

| Test | PureMemory | DDR2 | DDR3 | Mults |
| --- | --- | --- | --- | --- |
| GRP case 2 | 130% | 132% | 132% | 122% |
| SCE3 -Test 3.2 [6] | 106% | 105% | 105% | 106% |
| JCTVC-M0090 [5] | 108% | 106% | 106% | 112% |

Table 3. The worst scenario complexity for combined mode compare to HEVC worst case.

| Tool | MemBand 4x2 | MemBand 8x2 | Mults |
| --- | --- | --- | --- |
| GRP case 2 | 106% | 108% | 76% |
| SCE3 -Test 3.2 [6] | 133% | 133% | 145% |
| JCTVC-M0090 [5] | 100% | 100% | 134% |

From this complexity assessment and performance summary we observe that tools

- not accessing Base Layer references

- with worst case complexity within HEVC limit

show smaller than GRP but essential BD-rate gain.

# Conclusion

Verified tool test 3.3 (case 2) in SCE3 (so-called GRP) shows nice performance improvement which could be achieved with low-level changes (not-possible in HLS only framework). Performance and complexity assessment for GRP (case 2) are verified. An average gain from verified tool is 3,9%(Luma)/4,9%(Chroma). This gain is accomplished by 30-32% increment of memory access in average. In the worst case the memory access is 6-8% higher than HEVC worst case. In our measurements run time increment is 27% for Encoder and 32% for Decoder.

# References

[1] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013.

[2] E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440 document, January, 2013.

[3] E. Alshina, A. Alshin AHG-17: complexity and performance analysis of SHM1.0 compare to HM8.1 simulcast, JCTVC-M0086 document, April, 2013.

[4] E. Alshina, A. Alshin SCE4: De-noising filter for SNR scalability, JCTVC-M0087 document, April, 2013.

[5] E. Alshina, A. Alshin non SCE3: Low-pass filter for Combined Inter Prediction, JCTVC-M0090 document, April, 2013.

[6] V. Seregin, J. Chen, M. Karczewicz SCE3: Combined inter mode (test 3.2), JCTVC-M0294 document, April, 2013.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).