| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0340 |
| --- | --- |

| Title: | AHG13: Cross-check for Low Complexity Resampling Filters For SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification of alternative up-sampling filter design for SHVC. Proposed filter for Luma is shorter compare to SHM1.0 (6 taps instead 8 taps). Due to the this change there is 0,7% (×2) and 1,1% (×1,5) Luma BD-rate drop in “all-intra” tests. In motion compensation test scenario drop is 0,2% (×2) and 1,0% (×1,5). Results presented compare to SHM1.0 RefIdx framework.

# Introduction

In verified contribution Luma up-sampling filter length was reduced to 6 taps (SHM1.0 uses 8-taps filter) as shown in Table 1.

Table 1. Luma up-sampling filters.

| Phase shift | 8 taps Luma up-sampling filter in SHM1.0 | 6 taps Luma up-sampling filter in verified test |
| --- | --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 |
| 1/3 | {-1, 4, -11, 52, 26, -8, 3, -1}/64 | { 4, -12, 54, 24, -8, 2}/64 |
| 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1}/64 | {2, -8, 38, 38, -8, 2}/64 |
| 2/3 | {-1, 3, -8, 26, 52, -11, 4, -1}/64 | {2, -8, 24, 54, -12, 4}/64 |

Compare to SHM1.0 filter bit-depth of verified filter coefficients is 1 bit lower.

Frequency responses for ½ and 1/3 phase shift filters are shown on Figures 1-3. Additionally to verified and SHM1.0 filters 6-taps DCTIF is shown.

Due to lower coefficients precision of FastVDO’s filter deviation from ideal frequency response is noticeably higher compare to DCTIF. Especially in phase domain low-frequency part of spectrum will be shifted by ~0,28 instead 1/3. Error accumulation due to the bomb in amplitude frequency response for FastVDO’s filter is higher.

| Figure 1. Frequency response in amplitude domain for 1/3 filters: DCTIF 6 taps SHM1.0 8-taps filter 6-taps filter proposed by FastVDO (5 bits) | Figure 2. Frequency response in phase domain for 1/3 filters: DCTIF 6 taps SHM1.0 8-taps filter 6-taps filter proposed by FastVDO (5 bits) |
| --- | --- |
| Figure 3. Frequency response in amplitude domain for ½ filters: DCTIF 6 taps SHM1.0 8-taps filter 6-taps filter proposed by FastVDO (5 bits) |  |

# Test results

Source code for verified test arrived in time enough for study. It was requested by proponent run test only using RefIdx framework. Complexity assessment module was not integrated, but the worst case is the same DCTIF 6 taps Luma up-sampling filter [1]. And average memory access is expected to be very similar to results presented in [1].

Performance summary is presented in Table 2. Results for [1] (6 taps DCTIF) are presented for comparison as well. It should be mentioned that 6 taps DCTIF was presented for the first time in July, 2011 (before starting scalable HEVC extension development) [2]. Since DCTIF design principle is to keep minimal deviation from ideal frequency response it performs better than up-sampling filter from verified test. The difference is especially noticeable in ×1.5 tests: 0,7%...0,8% in terms of Luma BD-rate.

Table 2. Performance test summary.

|  | “all-intra” tests |  |  |  | “MC” tests |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ×2 |  | ×1.5 |  | ×2 |  | ×1.5 |  |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| FastVDO filter | 0,67% | 0,85% | 1,10% | 1,30% | 0,22% | 0,61% | 1,01% | 0,87% |
| DCTIF | 0,30% | 0,33% | 0,39% | 0,46% | 0,06% | 0,09% | 0,24% | 0,13% |

More details about test results and comparison with DCTIF can be found in [1] and excel spreadsheet attached to this contribution.

# Conclusion

Performance of 6-taps Luma up-sampling filter is verified in this report. Compare to 8-taps Luma filter in SHM1.0 there is 0,7% (×2) and 1,1% (×1,5) Luma BD-rate drop in “all-intra” tests. In motion compensation test scenario drop is 0,2% (×2) and 1,0% (×1,5).

# References

[1] E. Alshina, A. Alshin AHG-17,13: complexity and performance analysis of different length up-sampling filters in SHM1.0, JCTVC-M0088 document, April, 2013.

[2] E. Alshina, A.Alshin, CE3: DCT derived interpolation filter test by Samsung., JCTVC-F247 document, July, 2011.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).