| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0341 |
| --- | --- |

| Title: | SCE2: Cross-check for test 1.1 on skipped inter-layer texture prediction (ILTP) block signalling in Inter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification for SCE2 test which is about IntraBL skip mode which is introduced for Inter-slices. In average verified tool provides 0,1% Luma BD-rate gain accomplished by 0,1% Chroma BD-rate drop. There is no noticeable change for encoding or decoding time due to verified IntraBL skip mode.

# Introduction

Verified IntraBL skip mode for Inter-slices was proposed originally proposed in [1] and included into SCE2 study.

Inter-layer texture prediction signalling is modified compare to SHM1.0 in verified test. Modification is applied in Inter-slices only. In Inter-slice if both skip-flag and Inter-layer texture prediction flag are true then residual and merge index is not encoded.

# Test results

Source code for verified test arrived in time enough for study. Performance test was conducted vs SHM1.0 IntraBL framework. Since tools works only for Inter-slices only MC test were done.

Performance test summary is presented in Table 1.

In average verified tool provide 0,1% Luma BD-rate gain, but leads to 0,1% Chroma BD-rate drop.

We didn’t observe noticeable change of run-time.

Table 1. Performance test summary.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,3% | -0,3% |  |  |  | 0,0% | -0,3% | -0,3% |
| Class B | 0,0% | 0,1% | 0,2% | -0,1% | 0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Test vs Ref) | 0,0% | 0,0% | 0,0% | -0,1% | 0,1% | 0,0% | 0,0% | -0,1% | -0,1% |
| Overall (Test vs single layer) | 19,3% | 32,0% | 33,0% | 16,5% | 28,6% | 30,9% | 14,8% | 28,7% | 32,6% |
| EL only (Test vs Ref) | -0,4% | -0,3% | -0,3% | -1,2% | -1,0% | -1,0% | -0,4% | -0,5% | -0,4% |
| Enc Time[%] | 99,9% |  |  | 96,6% |  |  | 100,6% |  |  |
| Dec Time[%] | 99,8% |  |  | 100,4% |  |  | 99,6% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,1% | -0,2% | -0,2% |  |  |  | -0,1% | -0,2% | -0,3% |
| Class B | -0,1% | 0,4% | 0,3% | -0,3% | 0,4% | 0,2% | -0,1% | 0,2% | 0,1% |
| Overall (Test vs Ref) | -0,1% | 0,2% | 0,2% | -0,3% | 0,4% | 0,2% | -0,1% | 0,1% | 0,0% |
| Overall (Test vs single layer) | 25,9% | 36,7% | 38,4% | 22,2% | 33,6% | 36,2% | 22,3% | 34,5% | 39,1% |
| EL only (Test vs Ref) | -0,5% | -0,1% | -0,2% | -1,7% | -0,9% | -1,0% | -0,4% | -0,2% | -0,3% |
| Enc Time[%] | 100,1% |  |  | 96,1% |  |  | 100,9% |  |  |
| Dec Time[%] | 99,7% |  |  | 100,2% |  |  | 99,4% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -0,10% | 0,10% | 0,06% |  |  |  |  |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,1% | -0,2% | -0,2% |  |  |  | -0,1% | -0,3% | -0,3% |
| Class B | -0,1% | 0,3% | 0,2% | -0,3% | 0,3% | 0,3% | -0,1% | 0,1% | 0,1% |
| Overall (Test vs Ref) | -0,1% | 0,1% | 0,1% | -0,3% | 0,3% | 0,3% | -0,1% | 0,0% | 0,0% |
| Overall (Test vs single layer) | 28,1% | 37,5% | 38,6% | 24,3% | 33,4% | 35,5% | 23,7% | 33,4% | 37,1% |
| EL only (Test vs Ref) | -0,5% | -0,2% | -0,3% | -1,6% | -0,9% | -0,9% | -0,4% | -0,3% | -0,3% |
| Enc Time[%] | 100,6% |  |  | 97,5% |  |  | 101,4% |  |  |
| Dec Time[%] | 100,0% |  |  | 99,0% |  |  | 99,9% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

More details about test results can be found in excel spreadsheet attached to this contribution.

# Conclusion

Performance of SCE2 test 1.1 is verified in this report. In average verified tool provide 0,1% Luma BD-rate gain, but leads to 0,1% Chroma BD-rate drop. There is no noticeable change of encoding or decoding time compare to SHM1.0 I(ntraBL framework) anchor.

# References

[1] D.-K. Kwon, M. Budagavi, M. Zhou Non-TE2: Improved inter-layer texture prediction signaling JCTVC-L0230 document, January, 2013.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).