| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0342 |
| --- | --- |

| Title: | Non-SCE4 Cross-check for 6-Tap Adaptive Re-Sampling Filter (JCTVC-M0271) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification for non-SCE4 test from Qualcomm which is about adaptive up-sampling filter. In opposite to test presented in SCE4 in verified proposal Qualcomm uses 6-taps Luma and 4-taps Chroma filter with variable coefficients. In averaged verified tool provides following BD-rate gain: 0,3%(Luma)/0,5%(Chroma) in spatial scalability tests, 2,2%(Luma)/0,9%(Chroma intraBL)/1,4%(Chroma RefIdx) in SNR tests. Average gain is almost the same for IntraBL and RefIdx frameworks except Chroma gain in SNR test (higher for RefIdx).

# Introduction

Adaptive up-sampling filter was used in Qualcomm’s CfP response (JCTVC-K0036). It was already studies once in TEB4 and included in to SCE4 again. Design is kept almost un-changed and analysed in detail in our previous cross-check JCTVC-L0092 [1] for this tool.

The difference between previous design and verified test is

- Adaptive up-sampling filter is applied for Chroma. Default Chroma filter is the same with SHM1.0 (5 bits precision for coefficients magnitude). Adaptive one uses 6-bits precision.

- Default Luma filter is 8-taps SHM1.0 filter. If encoder chooses to use alternative one then 6-taps filter with variable coefficients is applied. The difference between default 8-taps filter and adaptive 6-taps is signalled:

{f0,f1+d0,f2+d1,f3+d2,f4+d3,f5+d4,f6+d5,f7}

here

{fi} are SHM1.0 up-sampling filter coefficients ,

{di} is difference between adaptive and default filter .

During {di} signaling it is assumed that the sum of the filter coefficients is 64 (last difference d5 is not signaled).

- Both for Luma and Chroma int-pel (zero phase) positions are filtered with not-constant filter coefficients. For the zero phase filter is symmetric.

# Test results

Source code for verified test arrived in time enough for study. Performance test was conducted vs SHM1.0 (both IntraBL and RefIdx frameworks). Results perfectly match with proponent’s data.

Performance test summary is presented in Table 1. For comparison we show results of JCTVC-M0087 [2] which is de-noising filter for SNR scalability tested in SCE4:

- M0271 uses variable filter coefficients, applied for Luma and Chroma;

- M0087 uses predetermined filter coefficients and applied for Luma only.

|  | All-Intra |  | MC |  | SNR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| IntraBL |  |  |  |  |  |  |
| M0271 | -0,20% | -0,48% | -0,39% | -0,57% | -2,18% | -0,87% |
| M0087 | NA | NA | NA | NA | -2,13% | 0,00% |
| RefIdx |  |  |  |  |  |  |
| M0271 | -0,21% | -0,39% | -0,40% | -0,54% | -2,21% | -1,37% |
| M0087 | NA | NA | NA | NA | -1,44% | -0,68% |

More details about tests can be found in excel spread-sheets attached to this contribution.

Complexity assessment for verified tool was not provided by proponent. But shorter Luma up-sampling filter (6-taps, fixed coefficients) combined with 4-taps Chroma filter (SHM1.0 variant) was studied in [3]. Average memory access reduction due to this change was 1-2% only. There is no reduction for the worst case memory band-width due to usage of shorter up-sampling filter since the worst case is still bi-pred 8x8 motion compensation.

# Conclusion

Performance of adaptive 6-taps (Luma) and 4-tpas (Chroma) up-sampling filter is verified. In averaged verified tool provides following BD-rate gain: 0,3%(Luma)/0,5%(Chroma) in spatial scalability tests, 2,2%(Luma)/0,9%(Chroma intraBL)/1,4%(Chroma RefIdx) in SNR tests.

# References

[1] E. Alshina, A. Alshin, J.H.Park TEB4: Cross-check report for adaptive up-sampling filter (test 4.2.1) JCTVC-L0092 document, January, 2013.

[2] E. Alshina, A. Alshin SCE4: De-noising filter for SNR scalability JCTVC-M0087 document, April, 2013

[3] E. Alshina, A. Alshin AHG-17,13: complexity and performance analysis of different length up-sampling filters in SHM1.0 JCTVC-M0088 document, April, 2013

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).