| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0343 |
| --- | --- |

| Title: | Cross-check for Encoder Optimization for SHVC: Enhancement Layer Lambda Refinement in RA Configurations (JCTVC-M0259) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification for Qualcomm’s proposal on lambda modification for random access configuration. Verified algorithm provides in average 0,8% (Luma) and  1,1 (Chroma) BD-rate gain for both IntraBL and RefIdx frameworks. Algorithm works in random access configuration and applied for only enhancement layer encoding.

# Introduction

In verified algorithm Lagrange multiplier in rate-distortion module of enhancement layer is refined based on temporal level, resolution ratio and QP gap between base and enhancement layer. As result the bit-size increases ~6% in average. PSNR of y component increases by 0,1dB due suggested change.

Only encoder part and only for enhancement layer was modified in verified test.

# Test results

Source code for verified test arrived in time enough for study. Performance test was conducted vs SHM1.0 (both IntraBL and RefIdx frameworks). Results perfectly match with proponent’s data and shown in Table 1 (IntraBL) and Table 2 (RefIdx).

Small decoding time increment 1-2% observed in our test can be explained by 6% large bit-streams (more bits to parse).

Table 1. Performance test summary for IntraBL framework.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0,5% | -0,5% | -0,3% |  |  |  |
| Class B | -0,6% | -2,1% | -2,2% | -0,8% | -1,2% | -1,4% |
| Overall (Test vs Ref) | -0,6% | -1,6% | -1,6% | -0,8% | -1,2% | -1,4% |
| Overall (Test vs single layer) | 18,6% | 29,9% | 30,9% | 15,6% | 26,9% | 29,1% |
| EL only (Test vs Ref) | 0,5% | -0,6% | -0,6% | 3,4% | 3,0% | 2,8% |
| Enc Time[%] | 100,6% |  |  | 100,8% |  |  |
| Dec Time[%] | 101,3% |  |  | 101,1% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0,5% | 1,4% | 1,7% |
| Class B | -1,3% | -1,1% | -1,4% |
| Overall (Test vs Ref) | -1,1% | -0,4% | -0,5% |
| Overall (Test vs single layer) | 13,5% | 28,4% | 32,1% |
| EL only (Test vs Ref) | 1,5% | 2,3% | 2,1% |
| Enc Time[%] | 101,0% |  |  |
| Dec Time[%] | 102,2% |  |  |
| Enc Mem[%] | 100,0% |  |  |
| BL Match | Matched |  |  |

Table 2. Performance test summary for RefIdx framework.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0,5% | -0,6% | -0,4% |  |  |  |
| Class B | -0,7% | -2,1% | -2,1% | -0,8% | -1,1% | -1,2% |
| Overall (Test vs Ref) | -0,6% | -1,7% | -1,6% | -0,8% | -1,1% | -1,2% |
| Overall (Test vs single layer) | 19,4% | 31,0% | 32,0% | 16,3% | 26,6% | 29,0% |
| EL only (Test vs Ref) | 0,5% | -0,6% | -0,6% | 3,5% | 3,3% | 3,1% |
| Enc Time[%] | 101,6% |  |  | 100,2% |  |  |
| Dec Time[%] | 101,1% |  |  | 101,3% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0,5% | 1,2% | 1,6% |
| Class B | -1,3% | -1,2% | -1,5% |
| Overall (Test vs Ref) | -1,1% | -0,5% | -0,6% |
| Overall (Test vs single layer) | 14,5% | 29,4% | 33,9% |
| EL only (Test vs Ref) | 1,4% | 2,1% | 1,9% |
| Enc Time[%] | 101,1% |  |  |
| Dec Time[%] | 101,8% |  |  |
| Enc Mem[%] | 100,0% |  |  |
| BL Match | Matched |  |  |

# Conclusion

Interesting performance improvement due to the no-normative change proposed by Qualcomm for enhancement layer coding in random access configuration is verified in this report. Verified algorithm provides in average 0,8% (Luma) and  1,1 (Chroma) BD-rate gain for both IntraBL and RefIdx frameworks.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).