| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0344 |
| --- | --- |

| Title: | Crosscheck of Removal of the offset quantization in SAO from MediaTek( |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Seungsoo Jeong Junghye Min Elena Alshina | Tel: Email: | ss00.jeong@samsung.com jh643.min@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung |  |  |

_____________________________

# Abstract

The document reports the verification results of Removal of the offset quantization in SAO described in JCTVC-M0118. It is reported that the BD-rate and PSNR results exactly match those provided by the proponent ( MediaTek ). The source code was examined and it was confirmed that implementation is consistent with descriptions in the proposal. The proposed method provides performance gains of -0.1%, -0.2%,-0.2%,-0.3%,-0.2%,-0.3% and -0.2% for in AI Main-tier, AI High-tier, AI Super High-tier, RA Main-tier, RA High-tier, LB Main-tier, and LB High-tier, respectively.

# Introduction

In HEVC, a quantization process is applied to the absolute values of the offsets when the sample bit depth is larger than 10

In JCTVC-M0118, it was proposed to remove the quantization process of offsets in SAO for sample bit depths larger than 10.

Magnitudes of correcting offsets in SAO (sao_offset_abs ) are limited according to internal bit-depth as shown in table 1below In HEVC for bit-depth higher than 10 SAO magnitudes are limited by 31. In verified contribution this limitation was removed in order to allow wider range of sao_offset_abs and so improve SAO performance.

Internal bit-depth and Maximum allowed SAO magnitude value.

|  | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- |
| HEVC | 7 | 15 | 31 | 31 | 31 |
| Proposed | 7 | 15 | 31 | 63 | 127 |

# Test results

The test is performed under the conditions defined in [1] with AI 2x and AI 1.5x configurations. The bit rate and PSNR results from our test with MediaTek’s software and the results provided by MediaTek are same. Encoding/Dncoding run times in M0118 are similar with our results.

Table 1 Coding performance, Removal of offset quantization

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:4:4 | -0.2% | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.3% | -0.3% | -0.4% |
| YCbCr 4:2:2 | -0.2% | 0.2% | 0.3% | -0.2% | 0.0% | 0.2% | -0.2% | -0.1% | 0.0% |
| Overall | -0.1% | 0.0% | 0.1% | -0.2% | -0.1% | 0.0% | -0.2% | -0.2% | -0.2% |
|  | -0.2% | 0.0% | 0.1% | -0.2% | -0.1% | 0.0% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.1% | 0.6% | -0.1% | 0.1% | 0.3% | 0.1% |  |  |  |
| YCbCr 4:4:4 | -0.5% | 1.0% | 0.3% | -0.3% | 0.7% | 0.4% |  |  |  |
| YCbCr 4:2:2 | -0.4% | 1.0% | 1.0% | -0.4% | 0.8% | 1.0% |  |  |  |
| Overall | -0.3% | 0.9% | 0.4% | -0.2% | 0.6% | 0.5% |  |  |  |
|  | -0.3% | 1.3% | 0.4% | -0.2% | -0.9% | 0.5% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.4% | 0.1% | 0.2% | 0.3% | 0.2% |  |  |  |
| YCbCr 4:4:4 | -0.4% | 0.5% | 0.3% | -0.4% | 0.1% | 0.2% |  |  |  |
| YCbCr 4:2:2 | -0.7% | 0.2% | 0.4% | -0.5% | 0.0% | -0.1% |  |  |  |
| Overall | -0.3% | 0.4% | 0.3% | -0.2% | 0.1% | 0.1% |  |  |  |
|  | -0.3% | 0.6% | 0.3% | -0.3% | 0.3% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |

# Conclusion

It was verified that the simulation results matched exactly those reported by the proponent and the description in the proposed Removal of the offset quantization in SAO matches the implementation in the software. The proposed method provides performance gains of -0.1%, -0.2%,-0.2%,-0.3%,-0.2%,-0.3% and -0.2% for in AI Main-tier, AI High-tier, AI Super High-tier, RA Main-tier, RA High-tier, LB Main-tier, and LB High-tier, respectively.

# Reference

[1] David Flynn, Chris Rosewarne , " Common test conditions and software reference configurations for HEVC range extensions ", JCTVC-L1006, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

[2] C-W Hsu,C-Y Chen,C-Y Tsai, Y-W Huang, S Lei, " Removal of the offset quantization in SAO ", JCTVC-M0118, 13th Meeting, Incheon, KR, 18–26 Apr. 2013