| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0345 |
| --- | --- |

| Title: | Cross-check for Bandwidth reduction for range extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunil Lee Junghye Min Elena Alshina | Email: | sunil.lee@samsung.com jh643.min@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification for Qualcomm’s proposal on memory band-width reduction for range extension by restriction of motion compensation in small blocks.

# Introduction

Motion compensation in 4:4:4 format deals with full-size Chroma blocks, and so memory access is noticeably higher compare to 4:2:0 case. The goal of verified contribution was to reduce the worst case memory band-width by restriction of motion compensation in small blocks.

Two methods were studied:

- Following HEVC bandwidth reduction for 8×4 and 4×8 PUs, where bi-directional motion vector is converted to uni-directional of the RefPicList0, the same conversion is done for 8×8 PUs.

- Bi-directional MV is converted to uni-directional of the RefPicList0 at motion compensation stage for both Chroma components only, while Luma component is still bi-predicted.

Both lead to the same reduction of the memory band-width in a worst case for 4:4:4 format. But still the worst case memory band-width for 4:4:4 will be slightly (9...16%) higher than for 4:2:0.

Both restrictions 1 and 2 lead to some performance degradation. Method 2 shows interesting possibility, since performance degradation is smaller.

Memory band-width assessment in verified contribution was done using JCTVC-L0440 methodology, which essentially assumes that Cb&Cr components are storied interleaved in 4:2:0 format. Proponents modified JCTVC-L0440 correctly for 4:4:4 format and keep assumption about interleaved storage of Cb&Cr component.

# Test results

Source code for verified test arrived in time enough for study. Performance test was conducted based on the test condition described in [1]. Test results are summarized in Tables 1 -2 below.

Table 1. Experimental results for method 1.

Table 2. Experimental results for method 2.

# Conclusion

Quite important problem was raised in Qualcomm’s contribution about memory band-width for HEVC range extension (4:4:4). Interesting solution for this problem was proposed in verified contribution. Solution 2 leads to smaller (0,3-0,4%) BD-rate performance degradation, but still memory band-width in worst case exceeds HEVC (4:2:0).

# References

[1] D. Flynn, and K. Sharman “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, Geneva, CH, Jan. 2013.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).