| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0346 |
| --- | --- |

| Title: | Cross-check for Combined Scalable + Multiview HEVC coding results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yongjin Cho Elena Alshina | Email: | yongjin9.cho@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification for TI’s proposal on combined Scalable + Multiview HEVC performance evaluation. Two layers for SHVC (D0 and D1) and two views for MV-HEVC (V0 and V1) were combined and tested with SNR scalability. From test results we may conclude that IntraBL framework outperforms RefIdx when SHVC is combined with MV-HEVC. Performance difference in these combined tests (2,0%) is higher when compared to SHVC SNR tests (0,9%).

# Introduction

In verified contribution, inter-view prediction is implemented in the SHM-1.0 software and a total of 4 layers are coded and several different prediction configurations (i.e. Simulcast, inter-view prediction only and Inter-view + Inter-layer prediction) are compared to each other. Inter-view prediction works with both IntraBL-based and RefIdx-based approaches. So their performances in the context of SHVC + MV-HEVC are also compared. Only SNR scalability was currently tested.

# Test results

# Currently no common software or common test conditions exist for combined scalable + multi-view coding. We just follow test conditions suggested by proponent. Run time is not reliable in our tests.

# Test results are summarized in Tables 1-3.

Table 1. BD-rate gain of ‘IntraBL SHVC + MV-HEVC” compared to Simulcast.

|  | AI |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL D +BL D) | -40.5% | -41.0% | -41.1% |
| Overhead over Enhancement D Layer | -8.3% | -8.8% | -9.0% |
| Overall (EL) | -53.0% | -53.3% | -53.4% |

|  | RA |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL D+BL D) | -30.7% | -24.3% | -25.2% |
| Overhead over Enhancement D Layer | 3.2% | 13.2% | 11.7% |
| Overall (EL) | -39.7% | -33.7% | -34.6% |

|  | LD-P |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (ELD +BL D) | -18.1% | -11.8% | -12.5% |
| Overhead over Enhancement D Layer | 18.8% | 28.4% | 27.2% |
| Overall (EL) | -23.4% | -17.2% | -17.8% |

Table 2. BD-rate gain of ‘IntraBL SHVC + MV-HEVC” compared to ‘MV-HEVC simulcast’.

|  | AI |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL D +BL D) | -24.7% | -23.2% | -23.5% |
| Overhead over Enhancement D Layer | 14.0% | 16.5% | 16.1% |
| Overall (EL) | -41.5% | -40.2% | -40.5% |

|  | RA |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL D+BL D) | -18.0% | -10.7% | -11.6% |
| Overhead over Enhancement D Layer | 20.4% | 31.6% | 30.3% |
| Overall (EL) | -29.5% | -22.7% | -23.5% |

|  | LD-P |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (ELD +BL D) | -10.7% | -3.8% | -4.9% |
| Overhead over Enhancement D Layer | 28.7% | 39.2% | 37.5% |
| Overall (EL) | -16.9% | -9.9% | -11.0% |

Table 3. BD-rate gain of ‘IntraBL SHVC + MV-HEVC” compared to ‘RefIdx SHVC + MV-HEVC’.

|  | AI |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL D +BL D) | -0.4% | -1.2% | -1.1% |
| Overall (EL) | -0.4% | -1.2% | -1.2% |
| Enc Time[%] | 106.7% |  |  |
| Dec Time[%] | 94.0% |  |  |

|  | RA |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL D+BL D) | -2.0% | -3.3% | -3.0% |
| Overall (EL) | -3.5% | -4.9% | -4.6% |

|  | LD-P |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (ELD +BL D) | -2.0% | -2.1% | -2.0% |
| Overall (EL) | -3.0% | -3.1% | -3.1% |

# Conclusion

Performance of combined Scalable + Multiview HEVC (SHVC + MV-HEVC) coding was verified in this report. Experimental results show that inter-layer prediction still provides significant gain even with inter-view predictions. And it was observed that ‘IntraBL SHVC + MV-HEVC’ improves luma BD-rate by 0.4% ~ 2.0%, when compared to ‘RefIdx SHVC + MV-HEVC’. This difference is higher performance gap between IntraBL vs RefIdx than that in SHM1.0.