| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0347 |
| --- | --- |

| Title: | Non-SCE3: Cross-check for bandwidth reduction for combined inter modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification for Qualcomm’s proposal on memory band-width reduction for test3.2 in SCE3. In order to keep memory band-width within HEVC limit this contribution, combined mode usage is restricted for 8x8 coding unit with non 2Nx2N partition mode, with additional restriction of bi-prediction for 16x16 coding unit with non 2Nx2N partition mode. This is so-called method 1, which is verified by this report.

# Introduction

Combined prediction tested in SCE3 (test 3.2 [1] and 3.1) provides nice gain without complicated access to the base layer reference frames (as GRP does). But the worst case memory band-width for the variant of combined prediction tested in SCE3 is 33% higher compare to the HEVC. In order to keep the worst case memory band-width within HEVC limit in verified contribution proponent apply following restrictions:

Method 1:

- combined mode is disallowed for 8x8 non 2Nx2N CUs,

- for 8x8 and 16x16 CUs combination with bi-prediction is not allowed

Method 2:

- combined mode is disallowed for 8x8 non 2Nx2N CUs,

- combination with bi-prediction is not allowed all CU sizes

With these restrictions the worst case memory band-width of combined mode is exactly equal to HEVC the worst case (8x8 bi-directional MC).

The difference in a combined mode application for original SCE3 test 3.2 [1] and verified restrictions is shown in Tables 1-3. Alternative variant for combined mode restrictions was proposed in JCTVC-M0092 [4]. This variant is described by Table4.

Table 1. SCE3 test 3.2 [1] original design.

| CU | 8x8 , 2Nx2N | 8x8 , non-2Nx2N | 16x16 , 2Nx2N | 16x16 , non-2Nx2N | >16x16 , 2Nx2N | >16x16 , non-2Nx2N |
| --- | --- | --- | --- | --- | --- | --- |
| Combined with uni-direction MC | yes | yes | yes | yes | yes | yes |
| Combined with bi-direction MC | yes | yes | yes | yes | yes | yes |

Table 2. Restricted SCE3 test 3.2 (JCTVC-M0297 method 1).

| CU | 8x8 , 2Nx2N | 8x8 , non-2Nx2N | 16x16 , 2Nx2N | 16x16 , non-2Nx2N | >16x16 , 2Nx2N | >16x16 , non-2Nx2N |
| --- | --- | --- | --- | --- | --- | --- |
| Combined with uni-direction MC | yes | no | yes | yes | yes | yes |
| Combined with bi-direction MC | no | no | yes | no | yes | yes |

Table 3. Restricted SCE3 test 3.2 (JCTVC-M0297 method 2).

| CU | 8x8 , 2Nx2N | 8x8 , non-2Nx2N | 16x16 , 2Nx2N | 16x16 , non-2Nx2N | >16x16 , 2Nx2N | >16x16 , non-2Nx2N |
| --- | --- | --- | --- | --- | --- | --- |
| Combined with uni-direction MC | yes | no | yes | yes | yes | yes |
| Combined with bi-direction MC | no | no | no | no | no | no |

Table 4. Restricted SCE3 test 3.2 (JCTVC-M0092).

| CU | 8x8 , 2Nx2N | 8x8 , non-2Nx2N | 16x16 , 2Nx2N | 16x16 , non-2Nx2N | >16x16 , 2Nx2N | >16x16 , non-2Nx2N |
| --- | --- | --- | --- | --- | --- | --- |
| Combined with uni-direction MC | yes | no | yes | no | yes | no |
| Combined with bi-direction MC | no | no | yes | no | yes | no |

# Test results

Source code for verified test arrived in time enough for study. Performance test was conducted using SCE3 recommendation [2]. Test results are summarized in Table 4. In addition to verified test results of original contribution [1] and alternative way to resolve memory band-width problem are included into summary. It has to be mentioned that the worst case memory band-width is the same (equal to HEVC the worst case). Memory access assessment was done according to [3], results are also included into the Table 4.

Table 4. Performance and memory access assessment summary.

| Test | Avg. BD-rate |  | Run Time |  | Memory access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Enc. | Dec. | Pure | DDR2 | DDR3 |
| SCE3 test3.2 | -1,24% | -5,16% | 114% | 100% | 106% | 105% | 105% |
| JCTVC-M0297_m1 | -1,21% | -5,00% | 114% | 100% | 106% | 104% | 104% |
| JCTVC-M0297_m2 | -1,13% | -4,29% | 112% | 101% | 104% | 103% | 103% |
| JCTVC-M0092 | -0,95% | -3,95% | 104% | 100% | 105% | 103% | 103% |

Our observation is following. So-called method 1 verified here leads to the minimal performance degradation. Method 2 verified by MediaTek memory access 2% in average. The restriction of combined mode in all non-2Nx2N CUs (JCTVC-M0092) essentially reduced encoding time.

# Conclusion

Performance and complexity assessment for combined mode restriction are verified in this report. If combined prediction is disallowed for 8x8 non-2Nx2N CUs and combination is allowed only with uni-directional MC prediction in 8x8 and 16x16 CUS then the worst case memory access doesn’t exceed HEVC limit, but BD-rate performance degradation from this restriction is only 0,03% (Luma)/0,16%(Chroma).

# References

[1] V. Seregin, J. Chen, and M. Karczewicz “SCE5: Simplified base layer MV candidate for merge mode (test 5.1.1)”, JCTVC-M0295, Incheon, KR, Apr. 2013.

[2] E. François, A. Tabatabai, and E. Alshina “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L0440, Geneva, CH, Jan. 2013.

[3] X. Li, E. François, P. Lai, D. Kwon, and A. Saxena “Description of Scalable Extensions Core Experiment 3 (SCE3): Combined Inter and Inter-Layer Prediction in SHVC”, JCTVC-L1103, Geneva, CH, Jan. 2013.

[4] E.Alshina, A.Alshin, “Non SCE3: Simplified design for combined prediction (test 3.2)”, JCTVC-M0092, Incheon, KR, Apr. 2013.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).