| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0350 |
| --- | --- |

| Title: | AHG8: Video coding using Intra motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-567-0630 madhukar@ti.com +1-214-479-1797 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Applications such as wireless displays, automotive infotainment, remote desktop, remote gaming, cloud computing etc. are becoming popular. Video in these applications often has mixed content consisting of natural video, text, graphics etc. In text and graphics regions, patterns (e.g. text characters, icons, lines etc.) can repeat within a picture. This contribution proposes a CU-level intra motion compensation tool to remove this redundancy to reportedly achieve coding gain. When intra motion compensation is enabled for a CU, either horizontal motion or vertical motion is signaled. The proposed method is tested with two different search ranges in encoder. In the first test, horizontal and vertical motions are limited to a range of 0 ~ -63. In the second test, the vertical motion is further limited so that displaced block does not go beyond LCU boundary. The second test is also combined with Sample adaptive angular Intra prediction in horizontal and vertical direction only (SAPHV of JCTVC-M0056) and coefficient level cRiceParam update of JCTVC-M0316. The following bit rate savings under RCE2 common conditions (lossless coding) and on RCE2 screen content sequences (RGB 4:4:4) are reported:

- First test: Intra MC horizontal only or vertical only, Search range of 0 ~ -63: AI: 24.0%, RA: 18.9% and LDB: 15.7%.

- Second test: Intra MC horizontal only or vertical only, Search range of 0 ~ -63, Vertical motion limited to be within LCU: AI: 21.0%, RA: 16.6% and LDB: 14.6%.

- Second test + Sample adaptive angular Intra prediction in horizontal and vertical direction only (SAPHV) of JCTVC-M0056: AI: 29.1, RA: 22.6 and LDB: 19.8%.

- Second test + SAPHV + cRiceParam update (JCTVC-M0316): AI: 32.8, RA: 25.9 and LDB: 22.9%.

Further improvements to this technique including extensions to handle 4:2:0 and 4:2:2 content are being currently studied.

# Introduction

Applications such as wireless displays, automotive infotainment, remote desktop, remote gaming, cloud computing etc. are becoming popular. Video in these applications often has mixed content consisting of natural video, text, graphics etc. In text and graphics regions, patterns (e.g. text characters, icons, lines etc.) can repeat within a picture as highlighted in Figure 1. We propose use of intra picture motion compensation to exploit this redundancy and achieve bit rate savings.

Figure 1: Example video picture with mixed content consisting of natural scene, text, and graphics from sc_map. Red highlights are overlaid on original picture.

JCTVC-L0303 [1] is another recent contribution that exploits this redundancy to reduce bits using a dictionary based coding system. The advantage of our contribution is that it uses NxN blocks (i.e. coding units) and existing tools such as motion compensation and is therefore better tuned to existing video codec architectures and implementations when compared to dictionary coding.

# Proposed technique

A CU level flag is used to signal whether Intra motion compensation is used or not. When Intra motion compensation is used, a motion vector is transmitted. If Intra motion compensation is not used, then normal HEVC prediction is carried out. Motion compensation is currently restricted to blocks displaced exactly to the left or blocks displaced exactly on the top of the current block. So only the x- or the y-motion vector component is signaled. Motion vectors are coded directly without prediction using fixed length coding in bypass mode. The Intra motion compensation method is tested with two different search ranges in encoder and it is also combined with the methods proposed in [2] and [3] as described below.

2.1 Horizontal and vertical motion vectors in a range of 0 to -63

Horizontal and vertical motion vectors are limited to 0 to -63 as shown in Figure 2. When LCU size is 64x64, it requires 3 rows of LCU to be stored.

Figure 2. Horizontal and vertical Intra MV search ranges in a range of 0 to -63

2.2 Vertical motion vector within LCU

Horizontal motion vector is limited to 0 to -63 as in Section 2.1 and vertical motion vector is further restricted so that displaced block does not go beyond LCU boundary as shown in Figure 3. Therefore, only two left LCUs need to be stored for motion compensation.

Figure 3. Vertical Intra MV search range within LCU

2.3 Vertical motion vector within LCU + SAPHV

The second test is combined with sample adaptive angular Intra prediction in horizontal and vertical direction only (SAPHV) [2].

2.4 Vertical motion vector within LCU + SAPHV + coefficient level cRiceParam update

The second test is combined with coefficient level cRiceParam update [3] as well as SAPHV [2].

# Simulation results

Software based on HM RExt was used in simulations. RCE2 common conditions (lossless coding) was used on RCE2 screen content sequences. Bit-rate savings by the methods in Sections 2.1 ~ 2.4 are shown in Tables 1 ~ 4, respectively, when compared to RCE2 anchor.

Table 1. Bit-rate saving by IntraMC using horizontal and vertical motions in a range of 0 to -63

| Sequence | Bit-rate savings |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LDB |
| wordEditing | -17.44% | -10.11% | -9.62% |
| map | -2.55% | -1.09% | -0.61% |
| programmming | -20.11% | -4.22% | -3.33% |
| VenueVu | -0.05% | -0.01% | -0.01% |
| cad-waveform | -49.50% | -48.28% | -45.25% |
| pcb-layout | -45.85% | -41.48% | -33.38% |
| ppt-doc-xls | -30.86% | -30.57% | -26.88% |
| vc-doc-sharing | -37.80% | -26.04% | -17.43% |
| web-browsing | -26.47% | -24.08% | -17.46% |
| cg_twist_tunnel | -9.24% | -3.57% | -2.98% |
| Average | -24.0% | -18.9% | -15.7% |

Table 2. Bit-rate saving by IntraMC using horizontal motion in a range of 0 to -63 and vertical motion within LCU

| Sequence | Bit-rate savings |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LDB |
| wordEditing | -16.33% | -9.40% | -8.64% |
| map | -2.02% | -0.88% | -0.53% |
| programmming | -17.33% | -3.61% | -2.88% |
| VenueVu | -0.04% | -0.01% | -0.01% |
| cad-waveform | -43.28% | -41.91% | -44.73% |
| pcb-layout | -40.37% | -36.55% | -28.32% |
| ppt-doc-xls | -28.69% | -28.41% | -24.56% |
| vc-doc-sharing | -27.50% | -18.73% | -11.89% |
| web-browsing | -25.62% | -23.27% | -16.65% |
| cg_twist_tunnel | -8.38% | -3.15% | -2.67% |
| Average | -21.0% | -16.6% | -14.1% |

Table 3. Bit-rate saving by IntraMC using horizontal motion in a range of 0 to -63 and vertical motion within LCU + SAPHV [1]

| Sequence | Bit-rate savings |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LDB |
| wordEditing | -24.03% | -13.79% | -13.27% |
| map | -10.40% | -5.98% | -3.72% |
| programmming | -27.04% | -9.30% | -7.93% |
| VenueVu | -8.26% | -3.31% | -2.79% |
| cad-waveform | -47.56% | -46.09% | -48.75% |
| pcb-layout | -52.37% | -49.53% | -43.06% |
| ppt-doc-xls | -34.49% | -34.27% | -31.04% |
| vc-doc-sharing | -34.27% | -22.38% | -13.17% |
| web-browsing | -32.33% | -28.85% | -23.14% |
| cg_twist_tunnel | -20.29% | -12.33% | -11.49% |
| Average | -29.1% | -22.6% | -19.8% |

Table 4. Bit-rate saving by IntraMC using horizontal motion in a range of 0 to -63 and vertical motion within LCU + SAPHV [1] + coefficient level cRiceParam update

| Sequence | Bit-rate savings |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LDB |
| wordEditing | -27.40% | -9.40% | -8.64% |
| map | -12.15% | -0.88% | -0.53% |
| programmming | -28.46% | -3.61% | -2.88% |
| VenueVu | -8.15% | -0.01% | -0.01% |
| cad-waveform | -53.51% | -41.91% | -44.73% |
| pcb-layout | -56.19% | -36.55% | -28.32% |
| ppt-doc-xls | -40.64% | -28.41% | -24.56% |
| vc-doc-sharing | -38.87% | -18.73% | -11.89% |
| web-browsing | -37.01% | -23.27% | -16.65% |
| cg_twist_tunnel | -25.72% | -3.15% | -2.67% |
| Average | -32.8% | -25.9% | -22.9% |

# References

1. Tao Lin, Shuhui Wang, Peijun Zhang, Kailun Zhou, “AHG8: P2M based dual-coder extension of HEVC,” JCTVC-L0303.

2. M. Zhou, M. Budagavi, “RCE2: Experimental results on Test 3 and Test 4”, JCTVC-M0056.

3. M. Budagavi, “AHG8: Coefficient level cRiceParam updates for screen content coding”, JCTVC-M0316.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).