| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0351 |
| --- | --- |

| Title: | AHG8: Residual DPCM for visually lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA 92121 | Email: Tel: | rajanj@qti.qualcomm.com +1-858-658-4511 |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Residual DPCM is being evaluated for lossless coding in Test 1 of RCE2. Extensions of residual DPCM have been proposed in JCTVC-M0288. In this document, the use of residual DPCM is proposed for lossy coding (targeting visually lossless coding). The method uses residual DPCM for certain intra modes when transform is skipped. Simulations results are presented for super-high tier QPs (12, 17, 22, and 27) using the test set and conditions recommended by AHG8. For the All-Intra configuration, it is reported that BD-rates of -2.1%, -2.1%, and -2.1% are achieved for the 3 components, respectively. For RGB 4:4:4 Screen Content sequences BD-rates of -3.7%, -3.5%, and -3.4% are reported.

# Introduction

Residual DPCM [1] is being evaluated for lossless coding in Test 1 of RCE2 [2]. Extensions of residual DPCM have been proposed in JCTVC-M0288 [3]. In this document, a modification to the extended residual DPCM method is proposed to account for the quantization operation, so that the same prediction method is applied when the transform is skipped irrespective of whether lossless or lossy coding is used. When a CU is coded in intra mode and when the transform is skipped, the reconstructed values of (quantized) samples in the causal neighborhood are available. Thus, the concept of residual DPCM can be extended to lossy coding by replacing original residuals by their quantized versions. Simulations results are presented for super-high tier QPs (12, 17, 22, and 27, targeting visually lossless scenarios) using the test set and conditions recommended by AHG8.

# Residual DPCM for visually lossless coding

Consider a block of size N (rows) × N (cols). Let ri,j, 0≤i≤N−1, 0≤j≤N−1 be the prediction residual after performing intra prediction as specified in the HEVC specification. Assume that the transform is skipped and let Q(ri,j), 0≤i≤N−1, 0≤j≤N−1 denote the quantized version of the residual ri,j. The residual DPCM is applied to the residual samples as follows. The modified N × N array R with elements ri,j is obtained as follows when the intra prediction mode is near-vertical (between 22 and 30, inclusive):

ri,j=ri,j,i=0, 0≤j≤(N−1)ri,j−Q(r(i−1),j),1≤i≤(N−1), 0≤j≤(N−1).

The modified residual sample, ri,j is quantized to produce Q(ri,j). Then, when the mode is near-vertical, is calculated as

Q(ri,j)=Q(ri,j),i=0, 0≤j≤(N−1),Q(ri,j)+Q(ri−1,j)1≤i≤(N−1), 0≤j≤(N−1).

The quantized modified residual samples Q(ri,j) are sent to the decoder. On the decoder side, the above calculations are repeated to produce Q(ri,j), 0≤i≤N−1, 0≤j≤N−1. The quantized residuals are added to the original prediction values to produce reconstructed sample values.

This can be straightforwardly extended to near-horizontal modes (between 6 and 14, inclusive) and diagonal modes (2, 18 and 34).

# Experimental results

In this section, coding results for the proposed method are presented according to the test sequences and conditions recommended by AHG8. The anchor is HM 10.0-RExt-2.0. Since the desired goal is visually lossless quality, super high tier QP range (12, 17, 22, and 27) is used. The gradient filtering is not changed and residual DPCM is applied for near-horizontal and near-vertical modes as proposed in [3]. Table 1 presents the coding results for the proposed method. For the All-Intra configuration, the overall BD-rates are -2.1%, -2.1%, and -2.1% for the 3 components, respectively. For class F and RGB 4:4:4 SC sequences, transform skip is used often. For these sequence the BD-rate performance is -1.1%, -1.8%, and -1.8% for class F and -3.7%, -3.5% and -3.4% for RGB 4:4:4 SC.

| Sequence Class | Y | U | V |
| --- | --- | --- | --- |
| Class F | -1.1% | -1.8% | -1.8% |
| Class B | 0.0% | -0.1% | 0.0% |
| RGB 4:4:4 SC | -3.7% | -3.5% | -3.4% |
| YCbCr 4:2:2 | 0.1% | -0.1% | 0.0% |
| YCbCr 4:4:4 | 0.1% | 0.0% | 0.0% |
| Overall | -2.1% | -2.1% | -2.1% |

Table 1: BD-rate performance of the proposed method for the All-Intra configuration (super-high tier)

BD-rate results for other configurations and range of QPs are provided in Table 2. It can be seen that similar BD-rate savings are achieved for other intra tiers as well.

Table 2: BD-rate performance of the proposed method for main tier and high tier QPs

# Conclusion

In this document, the use of extended residual DPCM is proposed for visually lossless coding. The basic idea is to use residual DPCM for certain intra modes when transform is skipped. Simulations results are presented for super-high tier QPs (12, 17, 22, and 27) using the test set and conditions recommended by AHG8. For the All-Intra configuration, it is reported that BD-rates of -2.1%, -2.1%, and -2.1% are achieved for the 3 components, respectively. For RGB 4:4:4 Screen Content sequences BD-rates of -3.7%, -3.5%, and -3.4% are reported.

# References

- S. Lee, I.-K. Kim, and C. Kim, “RCE2: Test 1 – Residual DPCM for HEVC lossless coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0079, April 2013.

- W. Gao, M. Zhou, P. Amon, and S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1122, Jan. 2013.

- R. Joshi, J. Sole, and M. Karczewicz, “AHG8: Extension of residual DPCM for lossless coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0288, April 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).