| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0352 |
| --- | --- |

| Title: | Report of issue in specifications of video and image transfer characteristics (gamma) functions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Request for study and corrective action as appropriate |  |  |
| Author(s) or Contact(s): | David Singer & Jay Zipnick Apple 1 Infinite Loop Cupertino CA 95014 Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98008 USA | Tel: Email: | 408 996 1010 singer@apple.com |
| Source: | Apple, Microsoft |  |  |

_____________________________

# Abstract

This contribution reports that there may be an issue with the precision of expression of the transfer characteristics functions specified in various video-related standards of SG16 and MPEG (including HEVC, AVC, and the "Coding-independent media code-points" of MPEG-B Part 8), and that this may have resulted in a small error in some equations obtained from (and expressed in) ITU-R BT.2020. It is suggested for the topic to be studied and for ITU-R WP6C to be consulted to determine the appropriate action.

# Introduction / Problem Statement

The transfer characteristics ("gamma") function of several video colour-space-related standards has been expressed in an equivalent of the following form for a linear signal strength input x:

f(x)=(1+a)⋅xk−a;t<=x<=1m⋅x;0<=x<=t

where t is a threshold value that defines the transition point between the two curve segments. (For convenience, here we assume that the lower bound of the range of x is 0, although this is not always the case in such specifications.)

Thus, the shape of the curve is controlled by four constants: m, k, a, and t. The value of these constants have been specified directly in the text of various standards. For example, Recs. ITU-R BT.601-6, BT.709-5, BT.1358, BT.1700, BT.2020, and SMPTE 170M all seem to include the following variation:

m = 4.5  	(the slope of the linear curve segment)

k = 0.45  	(the exponent of the exponential curve segment)

a = 0.099	(a constant offset)

t = 0.018	(a transition threshold)

It has been suggested that the design intent behind this formulation is for the values of m and k to be selected as design parameters and for the values of a and t to be derived as the values necessary such that two desirable properties are achieved: function and slope continuity at the value x = t.

It has thus been suggested that, rather than being intended as specific design parameters, the values of a and t that are specified in BT.709 are only two-digit approximations of the infinite-precision parameters necessary to achieve the design intent rather than being intended as exact value specifications. For BT.709, it is suggested that this would mean that the intent is to use a = 0.0992968268… and t = 0.01805396851… Note that if the two-significant-digit values are taken literally, there is some discontinuity at the value x = t, which is suggested to be unintended.

The difference between the two interpretations may not be so critical for 8-bit systems. However, differences below the second significant digit of precision may become important for higher bit depths.

It is noted that ITU-R BT.2020 specifies the following:

m = 4.5

k = 0.45

a = 0.099; for 10-bit system  (the same as in ITU-R BT.709-5)0.0993; for 12-bit system

t = 0.018; for 10-bit system  (the same as in ITU-R BT.709-5)0.0181; for 12-bit system

The distinction between 10 and 12 bit systems seems surprising if the intent is as described above, since:

- It perpetuates and encourages the use of low precision truncated constants with only two significant digits; expressing the function in this form seems ill-advised, assuming the intent is for the actual function shape to be the same for the two variants.

- It may not make much sense since often the calculation is performed at a different stage that is independent from the quantization (e.g. to 10 or 12 bits), so some systems may not have a practical way to implement this.

- Additionally, the Y'CbCr constant luminance matrix transforms are a function of a and t. So if there are two different sets of constants for the transfer function (e.g. the 10-bit set and the 12-bit set) there should be two different (but similar) sets of Y'CbCr constant luminance matrix transforms – yet ITU-R BT.2020 only defines one set of these transforms.

We note that the difference of interpretation not only affects the specification of the transfer characteristics function itself, but causes differences in the values given for the derivation of the chroma components of the "constant luminance" variation specified in Rec. ITU-R BT.2020. As currently specified, the chroma components seem to be derived from the "lower-precision" values of the constants in the transfer characteristics function. For example, using the a full-saturation blue signal input { R, G, B } = { 0, 0, 1 } would seem to produce an E′PB signal greater than 0.5, which does not seem intended. Using higher-precision representations of a and t would result in some values of other constants found in the "constant luminance" chroma equations of BT.2020 to be somewhat different.

In particular, it is suggested that if the following equations from BT.2020 are re-derived:

E′PB = ( E′B − E′Y ) ÷ 1.9404	for −0.9702  <=  E′B − E′Y  <=  0

E′PB = ( E′B − E′Y ) ÷ 1.5816	for 0 < E′B − E′Y  <=  0.7908

E′PR = ( E′R − E′Y ) ÷ 1.7184	for −0.8592  <=  E′R − E′Y   <=  0

E′PR = ( E′R − E′Y ) ÷ 0.9936	for 0 < E′R − E′Y   <=  0.4968

the result, with the same number of digits of precision, would be different – such as the following (see section 3.2).

E′PB = ( E′B − E′Y ) ÷ 1.9403	for −0.9702  <=  E′B − E′Y  <=  0

E′PB = ( E′B − E′Y ) ÷ 1.5820	for 0 < E′B − E′Y  <=  0.7910

E′PR = ( E′R − E′Y ) ÷ 1.7182	for −0.8591  <=  E′R − E′Y   <=  0

E′PR = ( E′R − E′Y ) ÷ 0.9938	for 0 < E′R − E′Y   <=  0.4969

We suggest liaison communication with ITU-R WP6C to determine the appropriate action.

# Proposal

If, after coordination through liaison communication, the interpretation described herein is confirmed, it is proposed that this discrepancy be corrected as follows:

- Only use one set of transfer characteristics, eliminating the two-significant digits version (this could be for transfer characteristics 1 and 6 as well as 14 and 15, and some other entries such as 11, 12, and 13 could be adjusted similarly).  Specifically, for values 1, 6, 14, and 15, the following formulation could be supplied:

VC = 1.0993 × LC (0.4500) − 0.0993		for 0.01805 ≤ LC ≤ 1

VC = 4.500 × LC				for 0 ≤ LC < 0.01805

(i.e.  α = 1.0993 and β = 0.01805, using the symbols used in BT.2020)

- Add remarks to the standard to clarify that the values provided for a and t (or α and β, using the symbols used in BT.2020) are only provided as approximations and that the ideal values of these constants are the values that achieve function and slope continuity across the transition point between the two curve segments.

- Redefine the constant luminance colour-difference signals in terms of the more precise transfer characteristics.

- Correct ISO/IEC 23001-8 Coding-Independent Code Points, which copy these equations in an enumerated form.

- Amend the various video coding standards to refer to 23001-8 for these code points and their definition.

# Detailed Derivation

## ITU-R BT.709 Transfer Characteristics and Precision

A couple of decades ago, a small discrepancy was noticed in the transfer characteristics for SMPTE 170M (which has the same opto-electronic transfer characteristics as ITU-R BT.709):

From section 5.1 Opto-electronic transfer characteristic of reference camera

VC = 1.099 × LC (0.4500) − 0.099		for 0.018≤ LC≤ 1

VC = 4.500 × LC				for 0≤ LC < 0.018

From section 5.2 Electro-optical transfer characteristic of reference reproducer

LT = [(Vr + 0.099)/1.099](1/0.4500)		for 0.0812 ≤ Vr ≤ 1

LT = Vr/4.500				for 0 ≤ Vr < 0.0812

At first glance these are inverse functions, except that 0.018 × 4.5 = 0.081, not 0.0812, and conversely, 0.0812/4.5 = 0.0180444… So it appeared that the first equation simply used a truncated value for the transition point of the piecewise function. A little bit of math showed that these functions were nearly continuous, and nearly continuous in the first derivative, and that the only reason for the tangent discontinuity was because two constants 0.099 and 0.018 were truncated to two significant digits. Just as one can truncate π to a value of 3.14, it is typically done for brevity, and not to suggest that π = 3.14 exactly. Nor should it preclude one from using a more precise value for π, if higher precision results are desired.

Charles Poynton independently described the values of transfer function parameters being chosen “to maintain function and tangent continuity”.

With this in mind, one can conclude the intent of the Rec. 709 opto-electronic transfer functions is intended to be:

VC = (1+a) × LC (0.4500) − a		for t ≤ LC ≤ 1

VC = 4.500 × LC			for 0 ≤ LC < t

where:

a ≈ 0.0992968…

t ≈ 0.01805397…

The math to derive a and t is in section 3.3.

However, many never do the math to derive these equations, and simply copy truncated, low-precision constants with two significant digits that unfortunately propagate in various standards documents, causing small discontinuities and increased error. This is akin to assuming π = 3.1. This may be fine for some approximations, but why intentionally artificially truncate precision to this extent?

Still, others have extended the precision of this same transfer function beyond two significant digits and have expressed the opto-electronic transfer function (OETF) as follows:

VC = 1.0993 × LC (0.4500) − 0.0993	for 0.01805 ≤ LC ≤ 1

VC = 4.500 × LC			for 0 ≤ LC < 0.01805

One might argue that this is “sufficient precision for most purposes”.

Regardless of how much precision is considered “sufficient”, using only two significant digits when commonly encoding digital video in 8, 10, or more bits, can be considered to be “insufficient precision”. Therefore, it is recommended that the OETF as shown above, with at least four significant digits be used in place of lower-precision functions.

## Deriving ITU-R BT.2020 colour-difference signal equations for constant luminance

ITU-R BT.2020 expresses the luminance coefficients to four significant digits:

Y = 0.2627⋅R + 0.6780⋅G + 0.0593⋅B

These coefficients can be derived from the colour primaries and white point as per SMPTE RP-177.

ITU-R BT.2020 then expresses constant luminance, Y′C, as:

Y′C = (0.2627⋅R + 0.6780⋅G + 0.0593⋅B)′

and the constant luminance colour-difference signals are expressed in terms of B′ − Y′C and R′ − Y′C scaled to have a nominal range from −0.5 to +0.5. Thus if we know the minimum and maximum values, m, of each colour-difference signal, the scale factors are simply 0.5 / |m|.

The prime notation, applies the opto-electronic transfer characteristic, as described in section 3.3, which to four significant digits can be expressed as:

VC = 1.0993 × LC (0.4500) − 0.0993		for 0.01805≤ LC≤ 1

VC = 4.500 × LC			for 0≤ LC < 0.01805

The 100% cyan, yellow, red and blue colour bars provide the full range (positive and negative extremes) for both colour difference signals. Thus for:

cyan	= (0,1,1)

yellow	= (1,1,0)

red		= (1,0,0)

blue	= (0,0,1)

yellow:		B′ − Y′C = (0.0)′ − (0.2627 + 0.6780)′ = −(0.9407)′

B′ − Y′C ≈ −0.9702   [B′ − Y′C negative extreme]

scale factor for B′ − Y′C negative extreme ≈ 1 / 1.9403

blue:		B′ − Y′C = (1.0)′ − (0.0593)′ = 1.0 − (0.0593)′

B′ − Y′C ≈ 0.7910   [B′ − Y′C positive extreme]

scale factor for B′ − Y′C negative extreme ≈ 1 / 1.5820

Thus:

C′BC = ( B′ − Y′C ) ÷ 1.9403        for −0.9702 <= ( B′ − Y′C ) <= 0

C′BC = ( B′ − Y′C ) ÷ 1.5820        for 	  0 <   ( B′ − Y′C ) <= 0.7910

cyan:		R′ − Y′C = (0.0)′ − (0.6780 + 0.0593)′ = −(0.7373)′

R′ − Y′C ≈ −0.8591   [R′ − Y′C negative extreme]

scale factor for R′ − Y′C negative extreme ≈ 1 / 1.7182

red:		R′ − Y′C = (1.0)′ − (0.2627)′ = 1.0 − (0.2627)′

R′ − Y′C ≈ 0.4969   [R′ − Y′C positive extreme]

scale factor for B′ − Y′C positive extreme ≈ 1 / 0.9938

Thus:

C′RC = ( R′ − Y′C ) ÷ 1.7182        for −0.8591 <= ( R′ − Y′C ) <= 0

C′RC = ( R′ − Y′C ) ÷ 0.9938        for 	  0 <   ( R′ − Y′C ) <= 0.4969

The least significant digits coloured red are those which differ from the currently published ITU-R BT.2020 standard.

## Deriving Rec. 709 transfer function parameters to maintain function and tangent continuity (math for pedantic video engineers)

The Rec. 709 transfer function is often expressed in the following form:

y = 4.5 x				0 <= x <= 0.018

y = 1.099 x0.45 − 0.099		x >  0.018

These equations contain truncated values, so this is not continuous, and the first derivative of the power function at x=0.018 is not exactly 4.5.

The intent of Rec. 709 transfer function can be expressed as:

y = 4.5 x				0 <= x <= t

y = (1 + a) x0.45 − 0.099		x > t

Where:

t ≈ 0.01805396851

a ≈ 0.0992968268

One can derive a more precise piecewise function that maintains function and tangent continuity using the following parametric form:

VC = (1+a) × LCk − a		for t ≤ LC ≤ 1

VC = m × LC			for 0 ≤ LC < t

where:

k = 0.45

m = 4.5

The point, t, where the two functions meet is at:

EQ 01	(1 + a) t 0.45 − a = 4.5 t

The point where the first derivatives are the same, is where the derivative of the power function is equal to the “toe” slope of 4.5:

EQ 02	0.45 (1 + a) t −0.55 = 4.5

Simplify:

EQ 03	(1+a) t −0.55 = 10

divide EQ 1 by t:

EQ 04	(1 + a) t −0.55 − a/t = 4.5

substitute EQ 3 into EQ 4:

EQ 05	10 − a / t = 4.5

EQ 06	5.5 = a / t

EQ 07	a = 5.5 t

substitute for a in EQ 1:

EQ 08	(1 + 5.5 t ) t 0.45 − 5.5 t = 4.5 t

EQ 09	t 0.45 + 5.5 t 1.45 = 10 t

EQ 10	t −0.55 + 5.5 t 0.45 = 10

solving for t and a:

EQ 11 → t = 0.01805396851...

EQ 12 → a = 0.0992968268...