| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0356 |
| --- | --- |

| Title: | Cross-check for JCTVC-M0322 signaling of phase offset in up-sampling process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye 9710 Scranton Rd, Ste 250 San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document is the cross-check report on Qualcomm’s proposal JCTVC-M0322 on signaling of phase offset in upsampling process. The proposal has two versions, uploaded on Apr. 9 and 15, respectively. The implementation and experimental results of the first version are cross-checked and confirmed in this document. The second version is carefully reviewed and studied by the cross-checker. However, due to the lack of explanation and software, the cross-checker is not able to confirm its technical correctness.

# Introduction

In SHVC Test Model 1 [1], upsampling for inter layer prediction is design based on an assumption that the sampling grids of BL and EL pictures have no phase shift. To satisfy this assumption, JCT-VC provides the downsampler in [2], which downsamples EL sequences and generates the zero-phase shifted BL sequences. However, the assumption doesn’t always hold, because the downsampler in [2] is not mandated, and an SHVC codec is expected to be able to take input BL and EL sequences with non-zero phase shift. When such is the case, using the current upsampling process in [1], the sampling grids between the EL and the upsampled BL pictures are misaligned, which causes inaccurate inter layer prediction and thus loss of scalable coding performance.

The first version of proposal JCTVC-M0322 tries to address the aforementioned problem. The proponents assume two possible types of relative positions between BL and EL sequences. One is so-called “aligned” position, which is generated by the downsampler in [2] and used as CTC sequences. The other is so-called “symmetric”, of which the BL sampling grid is aligned with that generated by the downsampler in [3] and has non-zero phase shift with the EL sampling grid. One flag luma_phase_flag is proposed to be added to SPS extension to signal which type of the relative position the BL and EL sequences have. The relative BL and EL positions other than the aforementioned two are not addressed in the proposal.

Two types of experiments were performed, given BL and EL sequences with “aligned” and “symmetric” relative positions, respectively. The anchor uses the upsampling providing misaligned upsampled BL and EL pictures, and the comparing method uses the upsampling providing aligned upsampled BL and EL pictures. The {Y, U, V} gain is {-7.0%, -5.9%, -5.6%} on average with IntraBL approach.

The implementation in the software generally matches the description in the document, except one place. In the document, luma_phase_flag is signaled in SPS extension. In the software, luma_phase_flag is not signaled in anywhere of the bitstream. Instead, a macro, accessible to both encoder and decoder, is used to enable/disable luma_phase_flag.

The second version of the proposal is to address possible luma/chroma relative positions other than the default relative luma/chroma position used in CTC sequences. However, due to the lack of explanation and software, the cross-checker is not able to confirm technical correctness of the second proposal.

# Experimental Results

The cross-check experiments on the first version of the proposal were conducted on 64-bit Linux cluster and the results match what were provided by the proponent.

# Reference

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Test Model 1 (SHM 1),” JCT-VC document, JCTVC-L1007, Geneva, Switzerland, Jan. 2013.

- J. Dong, Y. He, Y. Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m24499, 100th MPEG meeting, Geneva, Switzerland, Apr. 30-May 4, 2012.

- J. Dong, Y. He, Y. Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m23485, 99th MPEG meeting, San Jose, USA, Feb. 1-10, 2012.