| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0357 |
| --- | --- |

| Title: | Non-SCE1: Cross-check of JCTVC-M0139 on Inter-layer Intra Mode Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross verification results for JCTVC-M0139 on Inter-layer Intra Mode Prediction. Cross-check simulation confirms that results match with those from proponents. Study of software reports that implementation matches the description in the contribution.

# Introduction

JCTVC-M0139 proposed to modify MPM derivation for EL in SHVC. The algorithm consists of two steps. First step will fill intra mode for neighbors (ModeLeft or ModeTop) with its co-located BL block intra mode if neighbors are IntraBL mode. Second step is applied to co-located BL is intra coded for current CU, which is elaborated as follows

- When ModeBL is intra and is not planar/dc, it is used to derive the MPM list of the enhancement layer block as follows:If all of ModeLeft, ModeTop and ModeBL are the same, signed as MODE3:

  - then 	MPM[0] = MODE3,

MPM[1] = MODE3 + 1,

MPM[2] = MODE3 - 1.

- If two of ModeLeft, ModeTop and ModeBL are the same, signed as MODE2, the rest different one is signed as MODE1:

  - If MODE2 > 1, then MPM[0] = MODE2,

MPM[1] = MODE2 + 1

MPM[2] = MODE2 - 1.

  - Otherwise, MPM[0] = MODE2,

MPM[1] = MODE1

MPM[2] = ( MODE1 > 1) ? !MODE2 : vertical

- If ModeLeft, ModeTop and ModeBL are different from each other,

MPM[0] = ModeBL,

MPM[1] = ModeLeft

MPM[2] = ModeTop.

- Otherwise, derive the MPM list the same way as the base layer.

# Simulation and results

The code is compiled using MS Visual Studio 2010. The released source code is compiled under macro settings as follows.

Test1: MDCS on

#define SVC_BL_CAND_INTRA_STEP1          1      ///< set the neighbor block intra mode to that of its colaocated base layer block if its intraBL

#define SVC_BL_CAND_INTRA_NEW            1      ///< the proposed  inter-layer intra mode prediction method

#define SCAN_DIAG_ONLY                   0      ///< true: turn off the MDCS of enhancement layer.

Test2: MDCS off

#define SVC_BL_CAND_INTRA_STEP1          1      ///< set the neighbor block intra mode to that of its colaocated base layer block if its intraBL

#define SVC_BL_CAND_INTRA_NEW            1      ///< the proposed  inter-layer intra mode prediction method

#define SCAN_DIAG_ONLY                   1      ///< true: turn off the MDCS of enhancement layer.

RA and LP for 2x, 1.5x and SNR configurations are cross verified. The simulation is conducted under a cluster. So encoding and decoding time is not available for this cross-check report. Simulation results match with results from proponents. Detailed results are available in accompanying excel files.