| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0357r1 |
| --- | --- |

| Title: | Non-SCE1: Cross-check of JCTVC-M0139 on Inter-layer Intra Mode Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross verification results for JCTVC-M0139 on Inter-layer Intra Mode Prediction. Cross-check simulation confirms that results match with those from proponents. Study of software reports that implementation matches the description in the contribution. A new method is added in JCTVC-M0139 on April 14 and cross verified in this document. Study confirms that the implementation of the new method matches the description in JCTVC-M0139r2.doc and simulation results match with results from proponents.

# Introduction

JCTVC-M0139 proposed to modify MPM derivation for EL in SHVC. The algorithm(“Method 1”) consists of two steps. First step will fill intra mode for neighbors (ModeLeft or ModeTop) with its co-located BL block intra mode if neighbors are IntraBL mode. Second step is applied to co-located BL is intra coded for current CU, which is elaborated as follows

- When ModeBL is intra and is not planar/dc, it is used to derive the MPM list of the enhancement layer block as follows:

- If all of ModeLeft, ModeTop and ModeBL are the same, signed as MODE3:

  - then 	MPM[0] = MODE3,

MPM[1] = MODE3 + 1,

MPM[2] = MODE3 - 1.

- If two of ModeLeft, ModeTop and ModeBL are the same, signed as MODE2, the rest different one is signed as MODE1:

  - If MODE2 > 1, then MPM[0] = MODE2,

MPM[1] = MODE2 + 1

MPM[2] = MODE2 - 1.

  - Otherwise, MPM[0] = MODE2,

MPM[1] = MODE1

MPM[2] = ( MODE1 > 1) ? !MODE2 : vertical

- If ModeLeft, ModeTop and ModeBL are different from each other,

MPM[0] = ModeBL,

MPM[1] = ModeLeft

MPM[2] = ModeTop.

- Otherwise, derive the MPM list the same way as the base layer.

- On April 14, 2013, a software with a new method of JCTVC-M0139 (“Method 2”) is received for crosschecking. Study of software shows that the implementation matches with the description in JCTVC-M0139. The difference between Method 2 and Method 1 is as follows

- In the first step ModeLeft and/or ModdTop is set to ModeBL, if left and/or top neighbor is IntraBL mode and current co-located BL block is intra coded.

# Simulation and results

The code is compiled using MS Visual Studio 2010. The released source code is compiled under macro settings as follows.

Test1: MDCS on, Method 1

#define SVC_BL_CAND_INTRA_STEP1          1      ///< set the neighbor block intra mode to that of its colaocated base layer block if its intraBL

#define SVC_BL_CAND_INTRA_NEW            1      ///< the proposed  inter-layer intra mode prediction method

#define SCAN_DIAG_ONLY                   0      ///< true: turn off the MDCS of enhancement layer.

Test2: MDCS off, Method 1

#define SVC_BL_CAND_INTRA_STEP1          1      ///< set the neighbor block intra mode to that of its colaocated base layer block if its intraBL

#define SVC_BL_CAND_INTRA_NEW            1      ///< the proposed  inter-layer intra mode prediction method

#define SCAN_DIAG_ONLY                   1      ///< true: turn off the MDCS of enhancement layer.

Test3: MDCS on, Method 2

#define SCAN_DIAG_ONLY                   0      ///< true: turn off the MDCS of enhancement layer.

#define SVC_BL_CAND_INTRA_NEW_R1         1      ///< M0139 improve

Test4: MDCS off, Method 2

#define SCAN_DIAG_ONLY                   1      ///< true: turn off the MDCS of enhancement layer.

#define SVC_BL_CAND_INTRA_NEW_R1         1      ///< M0139 improve

AI for 2x, 1.5x and SNR configurations are cross verified. The simulation is conducted under a cluster. So encoding and decoding time is not available for this cross-check report. Simulation results match with results from proponents. Detailed results are available in accompanying excel files.