| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 13th Meeting: Incheon, KR, 18–26 April 2013 | Document: JCTVC-M0361 |
| --- | --- |

| Title: | Cross-check of motion field compression in RefIdx mode (JCTVC-M0135) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Patrice Onno Canon Research France | Tel: Email: | patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This document provides the cross-check results of the proposal JCTVC-M0135 on motion filed compression in RefIdx mode. All the BD-rate results are reported to match exactly those provided by the proponent.

# Introduction

The motion fields of the reference pictures are used for temporal collocated motion vector (MV) derivation in HEVC version 1. To reduce the buffer size, the motion fields are compressed with the unit size of 16x16 samples. For each 16x16 block, the above-left MV of the 16x16 block is used to represent the MV of the 16x16 block after motion field compression.

Regarding the SHVC approach and in particular the Ref_Idx mode, the motion fields of the inter-layer reference picture (ILRP) are filled with the upsampled motion field from the decoded base layer. To reduce the buffer size, the motion field of the ILRP is compressed with the unit size of 16x16 samples after the motion mapping. The center MV of each 16x16 block is used to represent the MV of the 16x16 block after compression.

In the JCTVC-M0135[1], it is proposed to unify the position of the representative MV in motion field compression for both HEVC and ILRP and it is proposed to select the above-left MV of the 16x16 block to represent the MV of the 16x16 block for ILRP motion field compression

# Cross-checking description

## Implementation verification

The software has been delivered by MediaTek on April 09, 2013. The software implementation has been verified and is confirmed to be in line with the JCTVC-M0135 description.

Here is the corresponding patch over SHM1.0 which confirms the selection of the up-left position in for the memory summarization.

diff -ur SHM-1.0/source/Lib/TLibCommon/TComPic.cpp M0135/source/Lib/TLibCommon/TComPic.cpp

--- /SHM-1.0/source/Lib/TLibCommon/TComPic.cpp

+++ M0135/source/Lib/TLibCommon/TComPic.cpp

@@ -627,7 +627,11 @@

UInt	uiPelX = pcCUDes->getCUPelX() + g_auiRasterToPelX[ g_auiZscanToRaster[uiAbsPartIdx] ];

UInt	uiPelY = pcCUDes->getCUPelY() + g_auiRasterToPelY[ g_auiZscanToRaster[uiAbsPartIdx] ];

UInt uiBaseCUAddr, uiBaseAbsPartIdx;

-     		pcPicBase->deriveUnitIdxBase(uiPelX + 8, uiPelY + 8, upSampleRatio, uiBaseCUAddr, uiBaseAbsPartIdx);

+#if REF_IDX_MFM_USING_UL

+      pcPicBase->deriveUnitIdxBase(uiPelX , uiPelY , upSampleRatio, uiBaseCUAddr, uiBaseAbsPartIdx);

+#else

+      pcPicBase->deriveUnitIdxBase(uiPelX + 8, uiPelY + 8, upSampleRatio, uiBaseCUAddr, uiBaseAbsPartIdx);

+#endif

## Simulations

Tests were done using the common test conditions presented in [2].

The source code was compiled successfully using VC2008 and using Linux 2.6.18-194.el5, gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

The full cross-check results are in the attached xls files. Results provided in JCTVC-M0135 are confirmed.

# Conclusions

The implementation and BDR results of JCTVC-M0135 presented by MediaTek are confirmed by the present document.

# References

- T.-D. Chuang, S. Liu, Y.-W. Huang, S. Lei, “On motion field compression in RefIdx mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0135, April 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1009, January 2013.