| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 13th Meeting: Incheon, KR, 18–26 April 2013 | Document: JCTVC-M0364 WG 11Number : m28849 |
| --- | --- |

| Title: | Non-SCE1: Cross-check report of document JCTVC-M0285 ‘Chroma-like coding of enhancement layer luma intra mode in SHVC’ |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard François Canon Research France | Tel: Email: | Edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This document provides the cross-check results of the Non-SCE1 proposal JCTVC-M0285 on Chroma-like coding of enhancement layer luma intra mode in SHVC from Sony. All the BD-rate results are reported to match exactly those provided by the proponent.

# Introduction

JCTVC-M0285 relates to intra mode coding in SHVC. In this proposal, the luma intra mode coding of enhancement layer is simplified and aligned with chroma intra coding. The number of allowed modes is reduced from 35 to 5. The 5 modes include co-located base layer luma intra mode as one candidate. The coding of luma intra mode is similar as chroma intra mode coding. A flag indicates if the mode is the BL mode or not. If not, a 2-bit FLC is encoded for remaining 4 modes.

# Cross-checking description

## Implementation verification

The software has been delivered by Sony on April 09, 2013.

The following macro is used to enable the proposed tool:

#define SONY_EL_DM                  1      ///< chroma-like EL luma intra mode coding

#define SONY_MDCS_OFF               1

There are two tested configurations.

Test 1:

#define SONY_EL_DM                 1           ///< chroma-like EL luma intra mode coding

#define SONY_MDCS_OFF              0

Test 2:

#define SONY_EL_DM                  1           ///< chroma-like EL luma intra mode coding

#define SONY_MDCS_OFF               1

The software implementation has been verified and is confirmed to be in line with the tool description.

## Simulations

Tests were done using the SCE1 test conditions for AI-2x and AI-1.5x.

The source code was compiled successfully using windows32, VC2008 and using Linux 2.6.18-194.el5, gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

The full cross-check results are in the attached xls files. Results provided in JCTVC-M0285 are confirmed.

When MDCS in On, an average gain of 0.19% Y, 0.09% U and 0.10% V is obtained with an encoding time decrease of 14.7% and a decoding time increase of 2.6%.

When MDCS in Off, an average gain of 0.13% Y, 0.04% U and 0.01% V is obtained with an encoding time decrease of 14.8% and a decoding time increase of 2.4%.

# Conclusions

The implementation and BDR results of JCTVC-M0285 presented by Sony are confirmed by the cross-checker.