| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0366 |
| --- | --- |

| Title: | AhG8: Simplified update of the coefficient level Rice parameter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents an update process of the Rice parameter of the syntax coeff_abs_level_remaining to reportedly improve HEVC RExt lossless and visually lossless coding efficiency for screen content coding. The proposed modification removes one condition check while reportedly improving bit rate savings under RCE2 lossless and range extensions common conditions. For the lossy case, results are also provided when the proposed update is only applied to transform skip.

# Introduction

A target application for HEVC Range Extensions 4:4:4 is wireless display. This is a consumer application that requires visually lossless coding without the increase in encoder/decoder complexity of professional applications. In this application it is necessary to code screen content, natural content and a mix of both.

Prediction error of screen content has different statistics than the natural content case. This difference can be exploited at the residue coding stage by setting a fast update process for the Rice parameter (cRiceParam) as shown in [1]. However, proposal [1] introduces additional checks and a multiplication in the critical loop of the entropy coding of the transform coefficient data.

This contribution proposes a fast update process for screen content that improves coding efficiency while reducing the complexity of the HEVC method by removing one condition. For the lossy case, the proposed method can be applied to transformed skip residual only, thus it is effectively exercised more on screen content and not for natural content.

# Technical Description

## Draft text

Changes are in section, 9.3.3.9 Binarization process for coeff_abs_level_remaining.

Original:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-1)

Proposed:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  <<  cLastRiceParam ) ), 4 )	(9-2)

Proposed with max cRiceParam = 5:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  <<  cLastRiceParam ) ), 5 )	(9-3)

## Source code

Original:

if(absCoeff[idx] > 3*(1<<uiGoRiceParam))

{

uiGoRiceParam = min<UInt>(uiGoRiceParam + 1, 4);

}

Proposed:

uiGoRiceParam = min<UInt>((uiGoRiceParam + ( absCoeff[idx] >> (2<<uiGoRiceParam))), 4);

Proposed with max cRiceParam = 5:
  uiGoRiceParam = min<UInt>((uiGoRiceParam + ( absCoeff[idx] >> (2<<uiGoRiceParam))), 5);

## Method 2: update applied only for non-transformed coefficients

There is no transform in HEVC lossless. For the lossy case, as described above, the method is applied for both, transform and transform skip.

As a variant of the method, the proposed Rice parameter update is applied only to transform skip for lossy. Therefore, for lossless and for transform skip the proposed update is used. Otherwise, the HEVC version 1 update is used. The source code for this case is included with this document.

For maximum Rice parameter equals to 4, the proposed WD text is as follows:

Draft text:

- If transform_skip_flag is equal to 1 or cu_transquant_bypass_flag is equal to 1, the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  <<  cLastRiceParam ) ), 4 )	(9-4)

- Otherwise,  the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-5)

- Simulation results

Results are provided on the top of HM10_Rext2.0 for three cases:

- Table 1: RCE2 lossless common conditions

- Table 2: Range extensions common conditions (JCTVC-L1006) for intra using screen content to show results closer to “visually lossless” performance

- Table 3: Range extensions common conditions for intra for method 2

Detailed results are provided in the accompanying spread-sheets files.

Table 1. Lossless results for maximum of cRiceParam 4 and 5.

|  | Max(cRiceParam) = 4 | Max(cRiceParam) = 5 |
| --- | --- | --- |
| All Intra Main |  |  |
| Class F | -0.9% | -0.9% |
| Class B | -0.1% | 0.2% |
| SC(GBR) | -4.3% | -6.8% |
| RangeExt | -0.3% | 0.1% |
| Overall (w/o SC) | -0.5% | -0.3% |
| Overall (w/ SC) | -2.4% | -3.5% |
| Random Access Main |  |  |
| Class F | -0.4% | -0.5% |
| Class B | -0.1% | 0.0% |
| SC(GBR) | -3.5% | -5.6% |
| RangeExt | -0.3% | -0.2% |
| Overall (w/o SC) | -0.3% | -0.3% |
| Overall (w/ SC) | -1.9% | -3.0% |
| Low delay B Main |  |  |
| Class F | -0.2% | -0.1% |
| Class B | -0.1% | -0.1% |
| SC(GBR) | -3.4% | -5.2% |
| RangeExt | -0.3% | -0.3% |
| Overall (w/o SC) | -0.2% | -0.2% |
| Overall (w/ SC) | -1.8% | -2.7% |

Table 2. Results for range extensions common conditions for intra and max cRiceParam 4 and 5

| Max | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam 4 | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% | -0.4% | -0.3% |
| Class B | 0.2% | 0.2% | 0.2% | 0.3% | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% |
| RGB 4:4:4 SC | -0.7% | -0.6% | -0.6% | -1.4% | -1.4% | -1.3% | -2.5% | -2.7% | -2.6% |
| YCbCr 4:2:2 | 0.1% | 0.2% | 0.1% | 0.2% | 0.3% | 0.3% | 0.4% | 0.4% | 0.4% |
| YCbCr 4:4:4 | 0.1% | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% | 0.4% | 0.4% |
| Overall | -0.3% | -0.3% | -0.3% | -0.7% | -0.6% | -0.6% | -1.2% | -1.3% | -1.2% |
|  | -0.3% | -0.3% | -0.2% | -0.7% | -0.6% | -0.6% | -1.2% | -1.3% | -1.3% |

| Max | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam 5 | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% |
| Class B | 0.2% | 0.2% | 0.2% | 0.3% | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% |
| RGB 4:4:4 SC | -0.4% | -0.4% | -0.4% | -1.0% | -0.9% | -0.9% | -2.3% | -2.5% | -2.5% |
| YCbCr 4:2:2 | 0.1% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% | 0.4% | 0.5% | 0.5% |
| YCbCr 4:4:4 | 0.1% | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% | 0.4% | 0.4% |
| Overall | -0.2% | -0.1% | -0.1% | -0.4% | -0.4% | -0.4% | -1.0% | -1.2% | -1.1% |
|  | -0.2% | -0.2% | -0.1% | -0.4% | -0.4% | -0.4% | -1.0% | -1.2% | -1.1% |

Table 3. Results for proposed update applied only for transform skip

| Max | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam 4 | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.2% | -0.2% | -0.4% | -0.4% | -0.4% | -0.6% | -0.6% | -0.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.8% | -0.8% | -0.8% | -1.7% | -1.7% | -1.7% | -2.9% | -3.1% | -3.1% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | -0.4% | -0.4% | -0.4% | -0.9% | -0.9% | -0.9% | -1.6% | -1.7% | -1.7% |
|  | -0.4% | -0.4% | -0.4% | -0.9% | -0.9% | -0.9% | -1.6% | -1.7% | -1.7% |
| Enc Time[%] | 104% |  |  | 103% |  |  | 104% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  | 104% |  |  |

| Max | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam 5 | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.4% | -0.5% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.6% | -0.6% | -0.6% | -1.3% | -1.3% | -1.3% | -2.7% | -3.0% | -3.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | -0.3% | -0.3% | -0.3% | -0.7% | -0.7% | -0.7% | -1.4% | -1.6% | -1.6% |
|  | -0.3% | -0.3% | -0.3% | -0.7% | -0.7% | -0.7% | -1.4% | -1.6% | -1.6% |
| Enc Time[%] | 105% |  |  | 105% |  |  | 106% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  | 89% |  |  |

# References

[1] M. Budagavi, “AHG8: Coefficient level cRiceParam updates for screen content coding”, JCTVC-M0316, JCT-VC 13th Meeting: Incheon, KR, April 2013.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).