| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0367 |
| --- | --- |

| Title: | RCE1: Cross-check of Tests 3.1, 3.2 and 3.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole, Woo-Shik Kim 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the results of the cross-check of the Range Extensions Core Experiment 1 Tests 3.1, 3.2 and 3.3. It is reported that the cross-check performance results match those provided by the proponents.

# Description

Software integrating all the methods in RCE1 was released by the proponents. The combination is described in JCTVC-L1121 [1]. The software integrates 4 methods on top of the HM10.0_RExt2.

RCE1 tests the following tools independently:

- Test 1.1: LM Chroma  (JCTVC-M0097)

- Test 1.2: Inter-plane intra coding of residual signals (RM)

- Test 2.1: In-loop color-space transformation of residual signals for range extensions

- Test 2.2: Fixed colour space transforms (JCTVC-M0048)

1.	YCbCr	= [.213, .715 .072; -.117, -.394, .511; .511, -.464, -.047]

2.	YCoCg	= [.25 .5 .25; 1/2 0 -1/2;-.25 .5 -.25]

3.	YFbFr	= [5/16 3/8 5/16; -1/2 1 -1/2;1 0 -1]

4.	YFbFr2	= [3/16 5/8 3/16; -1/2 1 -1/2;1 0 -1]

And some combinations of these methods in test 3:

- Test 3: Combination of methods (JCTVC-M0049)

- Test 1.1 + Test 1.2

- Test 1.1 + Test 1.2 + Test 2.2 (YFbFr2)

- Test 1.1 + Test 1.2 + Test 2.1 + Test 2.2

  - YCbCr

  - YFbFr2

This cross-check is for the combinations in test 3. The implemented algorithms agree with what is described in the corresponding proposals, except discrepancy between the description of YCbCr/YCgCo/YFbFr/YFbFr2 transforms usage and the software implementation.

We note the following remarks:

- BD-rate gains of tools reducing the colour correlation on different domain (pixel/residual) are not additive

  - LM chroma (test 1.1) and RM (1.2) gains are not additive (3.1)

- The implemented colour transforms in M0048 are slightly different from the descriptions. First, the selected transform is not signaled in the bitstream. Second, instead of the lossless transform specified above, lossy transforms of YFbFr/YFbFr2 are used of which the equations are as below.

3.	YFbFr	= [5/16 3/8 5/16; -1/4 1/2 -1/4; 1/2 0 -1/2]

4.	YFbFr2	= [3/16 5/8 3/16; -1/4 1/2 -1/4; 1/2 0 -1/2]

# Results

Coding results are obtained with the Range Extensions common conditions in JCTVC-L1006 [2] for the all the cases for Test 3.1 (see Table 1) and for the RGB sequences for Test 3.2 and 3.3 (see Table 2). Results match those provided by the proponents. Complete results are provided in the accompanying spread-sheets files.

Table 1. Results of Test 3.1

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -21.0% | -19.4% | -20.2% | -16.4% | -15.6% | -16.3% | -12.3% | -11.7% | -12.4% |
| YCbCr 4:4:4 | -2.4% | -9.5% | -9.7% | -2.7% | -6.2% | -8.5% | -2.7% | -4.3% | -6.1% |
| YCbCr 4:2:2 | -1.9% | -7.0% | -6.1% | -1.6% | -3.6% | -3.7% | -1.1% | -2.1% | -2.1% |
| Overall | -9.0% | -12.3% | -12.4% | -7.3% | -8.8% | -9.8% | -5.7% | -6.3% | -7.1% |
|  | -9.0% | -12.3% | -12.4% | -7.4% | -9.0% | -9.9% | -5.7% | -6.4% | -6.9% |
| Enc Time[%] | 116% |  |  | 116% |  |  | 117% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -11.8% | -10.6% | -12.7% | -8.4% | -6.8% | -9.1% |  |  |  |
| YCbCr 4:4:4 | -1.0% | -9.6% | -8.1% | -1.1% | -6.5% | -7.0% |  |  |  |
| YCbCr 4:2:2 | -0.9% | -8.1% | -6.0% | -0.7% | -3.4% | -3.8% |  |  |  |
| Overall | -4.9% | -9.5% | -9.1% | -3.6% | -5.6% | -6.7% |  |  |  |
|  | -4.9% | -9.7% | -9.0% | -3.7% | -7.1% | -7.1% |  |  |  |
| Enc Time[%] | 108% |  |  | 108% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -4.5% | -3.8% | -4.8% | -3.1% | -2.2% | -3.2% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -4.7% | -3.1% | -0.2% | -2.7% | -2.5% |  |  |  |
| YCbCr 4:2:2 | -0.3% | -5.1% | -4.0% | -0.3% | -1.8% | -2.3% |  |  |  |
| Overall | -1.8% | -4.5% | -4.0% | -1.3% | -2.2% | -2.7% |  |  |  |
|  | -1.8% | -4.1% | -4.0% | -1.3% | -2.5% | -2.7% |  |  |  |
| Enc Time[%] | 108% |  |  | 109% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |  |  |  |

Table 2. Results for RGB sequences for Tests 3.2 and 3.3

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R | G | B | R | G | B | R | G | B |
| Test 3.2 - YFbFr2 | -20.5% | -46.0% | -11.4% | -7.1% | -39.2% | 0.0% | -3.4% | -29.6% | 6.6% |
| Test 3.3 - YCbCr | -23.6% | -51.0% | -1.6% | -14.7% | -45.7% | 5.5% | -7.7% | -36.9% | 11.7% |
| Test 3.3 - YFbFr2 | -21.3% | -45.7% | -12.2% | -10.7% | -38.9% | -5.1% | -4.7% | -29.8% | 1.4% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | R | G | B | R | G | B |  |  |  |
| Test 3.2 - YFbFr2 | -28.1% | -49.4% | -18.4% | -11.4% | -47.2% | 5.8% |  |  |  |
| Test 3.3 - YCbCr | -32.3% | -54.4% | -8.7% | -21.1% | -54.3% | 12.9% |  |  |  |
| Test 3.3 - YFbFr2 | -28.7% | -49.4% | -18.8% | -14.1% | -47.1% | -0.7% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | R | G | B | R | G | B |  |  |  |
| Test 3.2 - YFbFr2 | -28.7% | -51.7% | -12.6% | -12.2% | -48.7% | 11.6% |  |  |  |
| Test 3.3 - YCbCr | -34.5% | -57.6% | -4.1% | -22.1% | -56.9% | 16.7% |  |  |  |
| Test 3.3 - YFbFr2 | -30.6% | -52.4% | -15.3% | -14.8% | -49.1% | 2.8% |  |  |  |

# Conclusion

In this contribution the Range Extensions Core Experiment 1 Test 3.1, 3.2, and 3.3 are cross-checked. It is confirmed that the test results exactly match to those provided by the proponent. Also the discrepancy between the descriptions of YFbFr/YFbFr2 and their implementations are noted.

# Reference

- J. Sole, K. Kawamura, and J. Kim, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1121, Jan. 2013.

- D. Flynn and K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1006, Jan. 2013.