| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0372 |
| --- | --- |

| Title: | AHG5: Cross-check on Unified intra prediction angles for 4:2:2 chroma format (JCTVC-M0127) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Akira Minezawa Shun-ichi Sekiguchi 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on the unified intra prediction angles schemes for 4:2:2 chroma format (JCTVC-M0127). The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent. Similar tendency on encoding and decoding time was also observed.

# Introduction

Performance verification on several proposed schemes by JCTVC-M0127 was conducted. The verified methods propose to unify intra prediction angles of version1 for YUV4:2:2 chroma case[1]. This work was conducted using modified HM10.0_RExt2.0 software provided by the proponent.

# Simulations & Results

Verification tests are conducted on 64bit Linux operating system (more specifically Fedora 10 x86_64). The performance evaluation has been conducted using the common test configurations for HEVC Range Extensions[2] on AI, RA and LDB configurations. From Table 1 to Table 4 show the results of “Proposal 1A”, “Proposal 1B”, “Proposal 3A” and “Proposal 3B” compared with HEVC Range Extensions anchor, respectively. All simulation results are perfectly matched the results from the proponent. The results are also shown that the increase of both encoding and decoding time was not observed.

Table 1. BD bitrate savings of “Proposal 1A”

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:2:2 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 2. BD bitrate savings of “Proposal 1B”

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 3. BD bitrate savings of “Proposal 3A”

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:2:2 | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 4. BD bitrate savings of “Proposal 3B”

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |

# Software analysis

The modified HM10.0_RExt2.0 software was provided by the proponent. It is confirmed that the source code has been modified according to the description.

# Conclusions

This contribution reports cross-verification results on JCTVC-M0127 that proposes the unified intra prediction angles schemes for YUV4:2:2. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent. Similar tendency on encoding and decoding time was also observed.

# References

- Hiroya Nakamura, “AHG5: Unified intra prediction angles for 4:2:2 chroma format”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-L1005, 13th Meeting: Incheon, KR, April, 2013.

- David Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-L1006, 12th Meeting: Geneva, CH, January, 2013.