| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0374 |
| --- | --- |

| Title: | Non-SCE1: Cross-check for weighted intra prediction (JCTVC-M0117) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: | +1-214-479-1797 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In this document, the cross-check results of the weighted intra prediction are reported in this document, where reconstructed base-layer samples are used to improve enhancement layer sample predictors. Source codes and test results provided by the proponents have been verified.

# Introduction

JCTVC-M0117 [1] proposes the weighted intra prediction. Reconstructed base-layer samples are used to improve enhancement-layer sample predictors, and a CU-level flag is signaled to switch on/off WIP. Complex and simple RD-cost calculation methods for WIP are employed in encoder.

# Experimental Results

Table 1 and Table2 show average BD-rate gains of WIP by using complex and simple RD-cost calculation methods, respectively. WIP results in luma BD-rate gain of AI 2x: 0.3% and AI 1.5x: 0.1% by using complex RD-cost calculation and AI 2x: 0.2% and AI 1.5x: 0.1% by using simplex RD-cost calculation. Encoding and decoding time in the tables may not be accurate. It was confirmed that the cross-check results exactly match to the results provided by the proponents.

Table 1. BD-rate results by using complex RD-cost calculation compared to SHM-1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.2% |  |  |  |
| Class B | -0.3% | -0.7% | -0.8% | -0.1% | -0.3% | -0.4% |
| Overall (Test vs Ref) | -0.3% | -0.6% | -0.6% | -0.1% | -0.3% | -0.4% |
| Overall (Test vs single layer) | 12.1% | 13.0% | 12.7% | 10.2% | 10.0% | 9.3% |
| EL only (Test vs Ref) | 0.0% | -0.3% | -0.3% | 0.4% | 0.1% | 0.0% |
| Enc Time[%] | 197.7% |  |  | 201.8% |  |  |
| Dec Time[%] | 108.7% |  |  | 107.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 1. BD-rate results by using complex RD-cost calculation compared to SHM-1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.2% |  |  |  |
| Class B | -0.2% | -0.6% | -0.6% | -0.1% | -0.2% | -0.3% |
| Overall (Test vs Ref) | -0.2% | -0.5% | -0.5% | -0.1% | -0.2% | -0.3% |
| Overall (Test vs single layer) | 12.2% | 13.1% | 12.8% | 10.2% | 10.0% | 9.4% |
| EL only (Test vs Ref) | -0.1% | -0.4% | -0.4% | 0.2% | 0.0% | -0.1% |
| Enc Time[%] | 114.2% |  |  | 114.5% |  |  |
| Dec Time[%] | 106.2% |  |  | 108.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Reference

- C. K. Kim and B. Jeon, “Non-CE1: Weighted intra prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M0117, Apr. 2013.