| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0375 |
| --- | --- |

| Title: | Non-SCE1: Cross-check for inter-layer intra mode prediction (JCTVC-M0312) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: | +1-214-479-1797 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This documents reports the cross-check results of inter-layer intra mode prediction in JCTVC-M0312, by which MPM is set using the collocated base-layer intra mode. Source codes and test results provided by the proponents have been verified.

# Introduction

Based on the observation that enhancement-layer intra mode is correlated with co-located base-layer intra mode regardless of whether or not it is angular, a couple of new MPM setting processes are proposed in JCTVC-M0312 [1].

Method 1

The MPM is generated as follows:

- If CorDir is angular,

MPM0 = CorDir, MPM1 = CorDir + 1, MPM2 = CorDir - 1.

- Otherwise, if CorDir != LeftDir and CorDir != AboveDir and LeftDir != AboveDir,

MPM0 = CorDir, MPM1 = LeftDir, MPM2 = AboveDir. MPM setting is completed.

- Otherwise,

The same MPM setting process as in SHM-1.0 is applied.

Method 2

By the Method 2, the MPM is generated as follows:

- If CorDir is angular (>1),

MPM0 = CorDir, MPM1 = CorDir + 1, MPM2 = CorDir - 1.

- Otherwise, if LeftDir = = AboveDir,

AboveDir is set to CorDir, and the same MPM process as in SHM1.0 is applied

- Otherwise,

The same MPM setting process as in SHM-1.0 is applied.

Both methods also are tested with MDCS on and off.

# Experimental Results

Tables 1 to 4 show average BD-rate gains by the inter-layer intra mode prediction methods described in the previous section. It was confirmed that the cross-check results exactly match to the results provided by the proponents.

Table 1. BD-rate results by Method 1 compared to SHM-1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.2% | -0.1% |  |  |  |
| Class B | -0.4% | -0.3% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.4% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.0% | 13.4% | 13.1% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.5% | -0.4% | -0.4% | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 103.6% |  |  | 106.4% |  |  |
| Dec Time[%] | 101.4% |  |  | 104.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2. BD-rate results by Method 1 with MDCS off compared to SHM-1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | 0.0% |  |  |  |
| Class B | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.1% | 13.6% | 13.3% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.4% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 103.6% |  |  | 106.2% |  |  |
| Dec Time[%] | 102.3% |  |  | 104.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 3. BD-rate results by Method 2 compared to SHM-1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.2% | -0.1% |  |  |  |
| Class B | -0.4% | -0.3% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.4% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.0% | 13.4% | 13.1% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.5% | -0.4% | -0.4% | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 104.8% |  |  | 108.0% |  |  |
| Dec Time[%] | 101.1% |  |  | 103.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 4. BD-rate results by Method 2 with MDCS off compared to SHM-1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | 0.0% |  |  |  |
| Class B | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.1% | 13.6% | 13.3% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.4% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 105.7% |  |  | 107.8% |  |  |
| Dec Time[%] | 99.1% |  |  | 107.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Reference

- J. Min and E. Alshina, “Non-SCE1: Inter-layer intra mode prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M0312, Apr. 2013.