| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0376 |
| --- | --- |

| Title: | Cross-check of complexity reduction in intra prediction of enhancement layer in SHVC (JCTVC-M0290) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: | +1-214-479-1797 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of JCTVC-M0290, which proposes to disable reference sample filtering and predicted sample filtering in intra prediction of EL to reduce complexity. Source codes and test results by the proponents have been verified.

# Introduction

In JCTVC-M0290 [1], it is proposed to disable reference sample filtering and predicted samples of DC, horizontal and vertical modes in intra prediction of EL, based on the observation that such filtering does not provide noticeable coding gain since inter-layer sample prediction replaces normal intra prediction modes in EL. Therefore, it is claimed that complexity can be reduced without coding gain loss.

# Proposed method

Table 1 shows an average BD-rate gain by the proposed method in [1]. It was confirmed that the cross-check results exactly match to the results provided by the proponents. Encoding and decoding time comparison in this table is kind of reliable since SHM-1.0 anchor and the proposed method are tested in same machines. Coding efficiency is slightly improved and decoding and encoding time is also slightly reduced.

Table 1. BD-rate results by the proposed method compared to SHM-1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.01% | -0.04% |  |  |  |
| Class B | -0.05% | -0.11% | -0.12% | -0.03% | -0.04% | -0.04% |
| Overall (Test vs Ref) | -0.03% | -0.08% | -0.09% | -0.03% | -0.04% | -0.04% |
| Overall (Test vs single layer) | 12.48% | 13.74% | 13.19% | 10.27% | 10.13% | 9.26% |
| EL only (Test vs Ref) | -0.09% | -0.14% | -0.15% | -0.07% | -0.08% | -0.08% |
| Enc Time[%] | 98.7% |  |  | 99.7% |  |  |
| Dec Time[%] | 99.2% |  |  | 99.4% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Reference

- C. Auyeung and J. Xu, “Complexity reduction in intra prediction of enhancement layer in SHVC,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M0290, Apr. 2013.