| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0386 |
| --- | --- |

| Title: | Cross-check of ‘AHG5: using zero-depth chroma transform unit for 4:2:2 format’ (M0099) by LGE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of proposal JCTVC-M0099 ‘AHG5: using zero-depth chroma transform unit for 4:2:2 format‘ by LGE.  The simulation result for all configurations were reported by LGE and tested by Canon.  The cross check result confirms the simulation result reported by LGE.

# Cross Check Procedure

This is a cross-check of proposal JCTVC-M0099 [1].

The source code for JCTVC-M0099 was provided by LGE.  Canon compiled the source code using GCC 3.4.6 and the simulation was performed on a Linux cluster under Range Extensions common test conditions [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code was studied found to be consistent with the description in [1].

# Coding Performance Measurement

Table 1 provides the cross-check result for JCTVC-M0099.  The simulation result for both cases matches the result provided by LGE.

Table 1. M0099 simulation results.

# Conclusion

This cross-check includes results for JCTVC-M0099 source code provided by LGE.  The provided software matches the description in the contribution.  The cross-check result confirms the simulation result reported by LGE.

# References

- J. Kim, B. Jeon (LG), “AHG5: using zero-depth chroma transform unit for 4:2:2 format”, JCTVC-M0099, 13th JCT-VC Meeting, Incheon, KR, Apr. 2013.

- D. Flynn, C. Rosewarne “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, 12th JCT-VC Meeting, Geneva, CH, Jan 2013.