| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0387 |
| --- | --- |

| Title: | RCE1 Test 2.1: Cross-check of ‘In-loop color-space transformation of residual signals for range extensions’ by KDDI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of proposal JCTVC-Mxxxx.  The simulation result for all configurations were reported by KDDI and tested by Canon.  The cross check result confirms the simulation result reported by KDDI.

# Cross Check Procedure

This is a cross-check of proposal JCTVC-Mxxxx [3] for ‘HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods’ [1].

The source code for JCTVC-Mxxxx was provided by KDDI.  Canon compiled the source code using GCC 3.4.6 and the simulation was performed on a Linux cluster under Range Extensions common test conditions [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Coding Performance Measurement

. provides the cross-check result for JCTVC-Mxxxx.  The simulation result for both cases matches the result provided by KDDI.

Table 1. RCE1 Test 2.1 simulation results.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -28.8% | -19.2% | -27.8% | -21.3% | -13.7% | -20.8% | -14.9% | -9.2% | -14.5% |
| YCbCr 4:4:4 | -1.9% | -7.4% | -10.3% | -2.6% | -3.5% | -9.5% | -2.0% | -1.9% | -6.9% |
| YCbCr 4:2:2 | 4.1% | -8.4% | -8.2% | 7.0% | -3.0% | -5.6% | 11.0% | -0.8% | -3.0% |
| Overall | -9.8% | -12.0% | -16.0% | -6.4% | -7.1% | -12.4% | -2.6% | -4.2% | -8.4% |
|  | -9.8% | -12.3% | -15.9% | -6.4% | -7.5% | -12.5% | -2.9% | -4.4% | -8.1% |
| Enc Time[%] | 108% |  |  | 108% |  |  | 107% |  |  |
| Dec Time[%] | 109% |  |  | 108% |  |  | 106% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -34.4% | -13.7% | -29.1% | -27.4% | -3.1% | -22.3% |  |  |  |
| YCbCr 4:4:4 | -1.5% | -12.2% | -13.0% | -2.3% | -6.2% | -13.3% |  |  |  |
| YCbCr 4:2:2 | 2.4% | -13.5% | -11.7% | 3.8% | -5.3% | -8.9% |  |  |  |
| Overall | -12.2% | -13.2% | -18.4% | -9.5% | -4.8% | -15.2% |  |  |  |
|  | -11.7% | -14.5% | -18.0% | -10.9% | -6.7% | -15.7% |  |  |  |
| Enc Time[%] | 106% |  |  | 106% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -35.6% | -8.4% | -27.0% | -27.5% | 0.2% | -21.0% |  |  |  |
| YCbCr 4:4:4 | -3.2% | -12.0% | -15.0% | -4.3% | -4.6% | -15.1% |  |  |  |
| YCbCr 4:2:2 | 1.6% | -13.2% | -12.9% | 2.6% | -3.9% | -9.6% |  |  |  |
| Overall | -13.5% | -11.1% | -18.7% | -10.6% | -2.6% | -15.5% |  |  |  |
|  | -12.3% | -8.1% | -16.4% | -11.1% | -3.3% | -15.7% |  |  |  |
| Enc Time[%] | 105% |  |  | 105% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |  |  |  |

# Conclusion

This cross-check includes results for JCTVC-Mxxxx source code provided by KDDI. The cross-check result confirms the simulation result reported by KDDI.

# References

- J. Sole, K. Kawamura, J. Kim “HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods”, JCTVC-L1121, 12th JCT-VC Meeting, Geneva, CH, Jan 2013.

- D. Flynn, C. Rosewarne “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, 12th JCT-VC Meeting, Geneva, CH, Jan 2013.

- K. Kawamura, T. Yoshino “In-loop color-space transformation of residual signals for range extensions”, JCTVC-Mxxxx, 13th JCT-VC Meeting, Incheon, KR, Apr. 2013.