| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0389 |
| --- | --- |

| Title: | Non-SCE3: Crosscheck of JCTVC-M0090 on low-pass filter for combined inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC-M0090 on low-pass filter for combined inter prediction proposed by Samsung. The proposed methods are implemented on top of SCE3.2, combined prediction in JCTVC-M0294, proposed by Qualcomm. The proposed methods add an additional term in the combined prediction. The combined prediction can be formulated as CombinedPredEL = (PredBL + PredEL + 1) >> 1. The proposed method can be formulated as CombinedPredEL = clip( PredBL + PredEL + Lowpass[PredBL - PredEL] + 1) >> 1, where the Lowpass is a 2D separable symmetrical 5-taps low-pass filter. Two tests are verified. One is the proposed combined prediction without any restriction. The other one is to apply the proposed combined prediction only on 2Nx2N CU and disallow the proposed combined prediction on 8x8 CU with bi-prediction. The verification task was done successfully, and the results matched exactly those provided by Samsung. The proposed methods can provide 0.5% additional gains on top of JCTVC-M0294. The bandwidths and computations of the proposed method are also evaluated. The worst case and average case of the bandwidths and computations matched exactly those provided by Samsung.

# Verification

Samsung provided the SHM-1.0 source code of their proposed method described in JCTVC-M0090 [1]. Two tests are evaluated. The verifications are carried out by setting the following parameters in the provided code:

1) Default mode

#define FDP_COMBINED_INTER_MODE              	1
#define FDP_COMBINED_INTER_MODE_2Nx2N       	0
#define FDP_COMBINED_INTER_MBW_HEVC      	0

2) Restriction mode

#define FDP_COMBINED_INTER_MODE              	1
#define FDP_COMBINED_INTER_MODE_2Nx2N       	1
#define FDP_COMBINED_INTER_MBW_HEVC      	1

Night configurations (RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, LDP-SNR, LDB-2x, LDB-1.5x, and LDB-SNR) were run for the proposed modified combined prediction with the provided code. The analyses on bandwidths and computations are also evaluated. Experiments were conducted according to the methodology defined in JCTVC-L0440 [3].

# Results

The BD-rates, bandwidths, and computations from our experimental results match exactly those provided by Samsung. The detailed results are listed in JCTVC-M0389_*.xls.

# Conclusion

The results presented by Samsung in JCTVC-M0090 are confirmed.

# References

- A. Alshin, E. Alshina, “Non SCE3: Low-pass filter for combined inter prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0090, April, 2013.

- V. Seregin, J. Chen, M. Karczewicz, “SCE3: Combined inter mode (test 3.2),” Document of Joint Collaborative Team on Video Coding, JCTVC-M0294, April, 2013.

- E. François, A. Tabatabai, E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0440, January, 2013.