| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0391 |
| --- | --- |

| Title: | Non-SCE1: Crosscheck of JCTVC-M0123 on constrained intra prediction at enhancement layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC-M0123 on the constrained intra prediction (CIP) modification proposed by LG. In SHM-1.0 IntraBL mode enhancement layer coding, when constrained_intra_pred_flag is set to 1, only the intra coded blocks can be used for the intra prediction of neighbouring blocks, and the inter-layer intra prediction coded blocks cannot be used. In the proposed method, the inter-layer intra prediction coded blocks can be used for the intra prediction of neighbouring blocks at the enhancement layer. The verification task was done successfully, and the results matched exactly those provided by LG.

# Verification

LG provided the SHM-1.0 source code of their proposed codec described in JCTVC-M0123 [1]. The SHM-1.0 anchor sets constrained_intra_pred_flag to 1. The verifications are carried out by setting the following parameters in the provided code:

Anchor: CIP_on

#define LG_CIP_ON                        1
#if LG_CIP_ON
#define LG_CIP_EN_LAYER                  0
#endif

Test: CIP_on with the proposed method

#define LG_CIP_ON                        1
#if LG_CIP_ON
#define LG_CIP_EN_LAYER                  1
#endif

Eight configurations (AI-2x, AI-1.5x, RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR) were run for the proposed constrained intra prediction at enhancement layer with the provided code.

# Results

The BD-rates from our experimental results match exactly those provided by LG. The detailed results are listed in JCTVC-M0391.xls.

# Conclusion

The results presented by LG in JCTVC-M0123 are confirmed.

# References

- C. Kim, B. Jeon, “Non-SCE 1: Constrained intra prediction at enhancement layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0123, April, 2013.