| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0295 |
| --- | --- |

| Title: | SCE5: Crosscheck of JCTVC-M0295 on result of SCE5.1.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC-M0295 on SCE5.1.1, simplified merging candidate derivation in SHM-1.0 IntraBL mode, proposed by Qualcomm. In the proposed method, a completed merging candidate list is firstly generated by using the HEVC version 1 derivation. Then the upsampled base layer motion vector is inserted in front of the constructed merging candidates as a new merging candidate list. The total number of merging candidates keeps the same. The proposed method can reuse merge candidate list generation process in HEVC. The verification task was done successfully, and the results matched exactly those provided by Qualcomm.

# Verification

Qualcomm provided the SHM-1.0 source code of their proposed method described in JCTVC-M0295 [1]. One test is evaluated. The verification is carried out by setting the following parameters in the provided code:

#define SVC_MVP  			0
#define EL_MERGE_COMPLETE    1

Six configurations (RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR) were run for the proposed simplified merging candidate derivation with the provided code.

# Results

The BD-rates from our experimental results match exactly those provided by Qualcomm. The detailed results are listed in JCTVC-M0395.xls.

# Conclusion

The results presented by Qualcomm in JCTVC-M0295 are confirmed.

# References

- V. Seregin, J. Chen, M. Karczewicz, “SCE5: Simplified base layer MV candidate for merge mode (test 5.1.1),” Document of Joint Collaborative Team on Video Coding, JCTVC-M0295, April, 2013.