| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0396 |
| --- | --- |

| Title: | Non-SCE3: Crosscheck of JCTVC-M0297 on bandwidth reduction for combined inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC-M0297 on bandwidth reduction for combined inter mode proposed by Qualcomm, which is an extension of SCE 3.2 combined prediction in JCTVC-M0294. In JCTVC-M0297, two methods are proposed to reduce the worst case bandwidth. This report verifies the second method, where only uni-directional prediction can be used with the combined mode for all CU sizes and the combined mode is disallowed for 8x8 CU with non 2Nx2N partition mode. The verification task was done successfully, and the results matched exactly those provided by Qualcomm. The bandwidths and computations of the proposed method are also evaluated. The worst case and average case of the bandwidths and computations matched exactly those provided by Qualcomm.

# Verification

Qualcomm provided the SHM-1.0 source code of their proposed method described in JCTVC-M0297. One test, the method 2, is evaluated. The verification is carried out by setting the following parameters in the provided code:

#define COMBINED_INTER_MODE              	1  // combined inter mode
#define COMBINED_INTER_MODE_RESTRICTION	1
#define COMBINED_INTER_MODE_UNI          	1

Nine configurations (RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, LDP-SNR, LDB-2x, LDB-1.5x, and LDB-SNR) were run for the proposed bandwidth reduction for combined inter mode with the provided code. The analyses on bandwidths and computations are also evaluated. Experiments were conducted according to the methodology defined in JCTVC-L0440 [3].

# Results

The BD-rates from our experimental results match exactly those provided by Qualcomm. The detailed BD-rate results are listed in JCTVC-M0396.xls. The bandwidths and computations of the proposed method are also evaluated. The worst case and average case of the bandwidths and computations matched exactly those provided by Qualcomm. The worst case bandwidth and computations are the same as the SHM-1.0 anchor. The detailed analyses are listed in JCTVC-M0396_complexity_analysis.xls.

# Conclusion

The results presented by Qualcomm in JCTVC-M0297 are confirmed.

# References

- V. Seregin, M. Karczewicz, “Non-SCE3: Bandwidth reduction for combined inter mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0297, April, 2013.

- V. Seregin, J. Chen, M. Karczewicz, “SCE3: Combined inter mode (test 3.2),” Document of Joint Collaborative Team on Video Coding, JCTVC-M0294, April, 2013.

- E. François, A. Tabatabai, E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0440, January, 2013.