| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0397 |
| --- | --- |

| Title: | Non-SCE1: Crosscheck of JCTVC-M0311 on supplementary results on SCE1 category 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang} mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC-M0311 on the supplementary results on SCE1 category 2 proposed by LG, Sony, and Qualcomm. In the proposed method, intra prediction based on difference samples between base layer (BL) and enhancement layer (EL) is proposed. The proposed signaling can be implicit or explicit. In this contribution, the methods with implicit signaling are evaluated. The verification task was done successfully, and the results matched exactly those provided by LG, Sony, and Qualcomm.

# Verification

LG, Sony, and Qualcomm provided the SHM-1.0 source code of their proposed method described in JCTVC-M0311 [1]. Three tests are evaluated. The verifications are carried out by setting the following parameters in the provided code:

1.	Residual TextureBL with implicit signaling

#define INTRA_BL_ENH_DIFF                1
#define INTRA_DC_ENH_DIFF                0
#define INTRA_PL_ENH_DIFF                0

2.	Residual DC with implicit signaling.

#define INTRA_BL_ENH_DIFF                0
#define INTRA_DC_ENH_DIFF                1
#define INTRA_PL_ENH_DIFF                0

3.	Residual planar with implicit signaling

#define INTRA_BL_ENH_DIFF                0
#define INTRA_DC_ENH_DIFF                0
#define INTRA_PL_ENH_DIFF                1

Two configurations (AI-2X and AI-1.5x) were run for the proposed modifications with implicitly signaling with the provided code.

# Results

The BD-rates from our experimental results match exactly those provided by LG, Sony, and Qualcomm. The detailed results are listed in JCTVC-M0397_*.xls.

# Conclusion

The results presented by LG, Sony, and Qualcomm in JCTVC-M0311 are confirmed.

# References

[1] J. Park, B. Jeon, C. Auyeung, A. Tabltabai, K. Rapaka, J. Chen, X. Li, M. Karczewicz, “Non-SCE1: supplementary results on category 2,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0311, April, 2013.