| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0400 |
| --- | --- |

| Title: | Non-SCE4: crosscheck of JCTVC-M0233 on Inter-layer switchable upsampling filters for spatial scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross verification results for JCTVC-M0233 on Inter-layer switchable upsampling filters for spatial scalability. Study of software reports that implementation matches the description in the contribution. BD-rate numbers match with proponents’ results. Encoding and decoding time is not accurate in this crosscheck report. Complexity assessment results are also cross verified and results match with proponents’ results.

# Introduction

JCTVC-M0233 proposed to use two up-sampling filters for CU with IntraBL mode. The switching is applied to luma and chroma and signaled with one flag coded with one dedicated context model. For luma, either existing 8-tap filter or a new 4-tap filter is used. For chroma, either existing 4-tap filter or a new 2-tap filter is used. A 3-tap filter [7,50,7] is applied to zero phase position.

# Simulation and results

The code is compiled using MS Visual Studio 2010. The released source code is compiled under macro settings as follows.

For the coding efficiency performance:

#define INTER_LAYER_FILTER               1

#define SHVC_COMPLEXITY_ASSESSMENT           0

For the complexity assessment:

#define INTER_LAYER_FILTER               1

#define SHVC_COMPLEXITY_ASSESSMENT           1

RA, LD-P and LD-B for 2x, 1.5x and SNR configurations are cross verified. The simulation is conducted in a heterogeneous cluster. So encoding and decoding time is not accurate for this cross-check report. Simulation results (BD-rate numbers and complexity assessment numbers) match with results from proponents. Detailed results are available in accompanying excel files.