| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0401 |
| --- | --- |

| Title: | SCE1 : Crosscheck of Test 4.2.2 and 4.2.3 on Difference Intra prediction by Qualcomm (M0331) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Junghye Min | Email: | elena_a.alshina@samsung.com jh643.min@samsung.com |
| Source: | Samsung Electronics Co, Ltd |  |  |

_____________________________

# Abstract

This report is verification of test 4.2.2 and 4.2.3 from the proposal M0331 (SCE1: Results of Test 4.2.2 and 4.2.3 on Difference Intra prediction). The source code was reviewed and it was confirmed that implementation is consistent with algorithm descriptions in the proposal. BD-rate and PSNR results from our tests exactly match the results provided by proponents. The proposed approach provides performance gains -0.8% and -0.6 in luma component for AI 2x and AI 1.5 respectively. Encoding/Decoding time increment is about 60%/9% and 51%/6%  for AI 2x and AI 1.5.

# Introduction

In M0331, Qualcomm presented test results of the Difference domain Intra Prediction (DIP). The DIP uses the difference between the pixels of current neighbors and those of collocated BL neighbors for intra prediction. Mode dependent intra sample smoothing process is not applied for DIP. On CU level, 1 bit flag indicating whether DIP is applied is signaled.

# Test results

The tests were performed under the conditions defined in JCTVC-L1101 [2] with AI 2x and AI 1.5x configurations. The bit rate and PSNR of our test results match with results proponents provided. Encoding/Decoding run times are similar with our results.  Simulation results are summarised in following tables.

# Conclusion

Performance of difference domain intra prediction was verified. The DIP provides performance gains of -0.8 and -0.6 in luma component for AI 2x and AI 1.5 respectively. Encoding time increment is about 55% where decoding time is increased by about 8%.

# References

[1] K. Rapaka, J.Chen, X.Li, M.Karczewicz, SCE1 : Results of Test 4.2.2 and 4.2.3 on Difference Intra Prediction, JCTVC-M0331, 2013

[2] A. Tabatabai, K. Rapaka, A. Saxena and S. Liu, Description of Core Experiment SCE 1: Intra Prediction Improvements in SHVC, JCTVC-L1101, 2013

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).