| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0403 |
| --- | --- |

| Title: | Cross-check of Motion-based adaptive partition technique with an application on SHVC (JCTVC-M0070). |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Alexander Alshin Elena Alshina 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 { alexander_b.alshin, elena_a.alshina }@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This is verification of performance adaptive partition technique proposed by Huawei (JCTVC-M0070) [1]. In this contribution, an adaptive CU partition algorithm was studied to improve motion compensation performance along the moving boundaries.

# Introduction

CU partition is a important part of modern video codec scheme. In HEVC for inter CU following cases is possible : PART_2Nx2N, PART_NxN, PART_2NxN, or PART_Nx2N; and if amp_enabled_flag equal to 1, also PART_2NxnU, PART_2NxnD, PART_nLx2N, or PART_nRx2N. However only these partitions are not enough to obtain an accurate motion estimation sometimes. That is why in JCTVC-M0070 adaptive partition technique was proposed. The adaptive partition for the current CU could be derived in the following steps:

- A reference block is located

- Available motion information from neighbouring locations, either spatial or temporal, applies onto the reference block

- The statistics of the difference block after motion compensation are analyzed to generate the adaptive partition

- Apply the adaptive partition from 3 onto the current CU for coding

The number of partitions is limited to 2. The adaptive partition, denoted as PART_APxAP, is defined as a new mode.

Two new syntax elements, part0_merge_idx[ x0 ][ y0 ] and part1_merge_idx[ x0 ][ y0 ] are introduced.

Mean value of the residuel block is used as the threshold for partition generation.

# Test results for verified algorithm

The verified algorithm is evaluated under the SHVC common test conditions [2], and the results are provided in Table 1 below.

Table 1. Test results summary.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Test vs Ref) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Test vs single layer) | 19,3% | 32,0% | 33,1% | 16,6% | 28,5% | 30,9% | 14,9% | 28,9% | 32,7% |
| EL only (Test vs Ref) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 101,0% |  |  | 100,5% |  |  | 100,3% |  |  |
| Dec Time[%] | 102,1% |  |  | 100,6% |  |  | 101,9% |  |  |
| Enc Mem[%] | 169,7% |  |  | 169,5% |  |  | 184,4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,0% |
| Overall (Test vs Ref) | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,0% |
| Overall (Test vs single layer) | 26,1% | 36,6% | 38,2% | 22,6% | 33,1% | 36,0% | 22,4% | 34,5% | 39,1% |
| EL only (Test vs Ref) | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,0% |
| Enc Time[%] | 100,8% |  |  | 100,3% |  |  | 100,1% |  |  |
| Dec Time[%] | 102,0% |  |  | 101,4% |  |  | 101,2% |  |  |
| Enc Mem[%] | 166,6% |  |  | 174,3% |  |  | 186,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusions

The performance of adaptive partition technique is verified in this report. In spite of on the current stage of development proposed algorithm didn’t show significant performance improvement it has interesting potential in the future for improving motion compensation algorithm module and reduction of computational complexity. Complexity assessment study is needed.

# References

[1] J. Zan Motion-based adaptive partition technique with an application on SHVC,  JCTVC-M0070  document, April, 2013.

[2] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013