| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0405 |
| --- | --- |

| Title: | AHG5: Crosscheck of JCTVC-M0335 on offset scaling in SAO for high bit-depth video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {cw.hsu, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC-M0335 on offset scaling in SAO for high bit-depth video coding proposed by Qualcomm. In JCTVC-M0335, offset scaling methods in SAO are proposed when bit-depth is larger than 10 to improve coding efficiency. We verified the method 1 with the internal bit-depth set to input bit-depth plus 2. The verification task was done successfully, and the results matched exactly those provided by Qualcomm.

# Verification

Qualcomm provided the source code of their proposed method described in JCTVC-M0335 [1] based on HM10.0-range-extensions verison2. The verification is carried out by setting the following parameters in the provided code:

#define SAO_OFFSET_SCAL                           1

There are two methods proposed in JCTVC-M0335, where each method should be tested with the internal bit-depth set to input bit-depth plus 2 and 4. We only test the method 1 with the internal bit-depth set to input bit-depth plus 2. According to the common test conditions of HEVC range extension [2], seven configurations (AI-Main-Tier, AI-High-Tier, AI-SuperHigh-Tier, RA-Main-Tier, RA-High-Tier, LB-Main-Tier, and LB-High-Tier,) were run for the proposed method 1 with the provided source code.

# Results

The BD-rates from our experimental results match exactly those provided by Qualcomm. The detailed results are listed in JCTVC-M0405.xls.

# Conclusion

The results for the proposed method 1 with the internal bit-depth set to input bit-depth plus 2 presented by Qualcomm in JCTVC-M0335 are confirmed.

# References

- W.-S. Kim, J. Sole, M. Karczewicz, “AhG5: Offset Scaling in SAO for High Bit-depth Video Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0335, April, 2013.

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1006, January, 2013.