| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0407_r2 |
| --- | --- |

| Title: | Cross-check of modified in-loop colour-space transformation (JCTVC-M0072) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution reports the cross-check of modified in-loop colour-space transformation, which is proposed in JCTVC-M0072-v2.

# Introduction

The modified in-loop colour space transformation of residual signals is presented in the document JCTVC-M0072-v2 [1]. The proposed method controls the degradation of coding gain by the conventional method JCTVC-L0371while it reduces run-times. This contribution reports the cross-check of this proposal. The revision two of this contribution reports the results by the revised software which was provided by the proponent.

The software version was HM10.0 RExt2.0. The reference was generated by using the common test condition [2]. Provided software was compiled on 64-bit Windows 7 environment. The experiments were performed on the cluster PCs.

Table 1 Summary results of All Intra conditions

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -21.8% | -15.1% | -20.9% | -16.9% | -11.5% | -16.7% | -12.3% | -8.4% | -12.4% |
| YCbCr 4:4:4 | -1.6% | -7.1% | -6.8% | -1.6% | -4.1% | -5.6% | -1.2% | -2.3% | -4.1% |
| YCbCr 4:2:2 | 0.7% | -5.9% | -5.7% | 1.5% | -2.1% | -3.1% | 2.4% | -0.5% | -1.1% |
| Overall | -8.2% | -9.6% | -11.6% | -6.2% | -6.2% | -8.8% | -4.1% | -4.0% | -6.2% |
|  | -8.0% | -9.9% | -11.7% | -6.3% | -6.4% | -8.9% | -4.3% | -4.1% | -6.3% |
| Enc Time[%] | 105% |  |  | 104% |  |  | 103% |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  | 103% |  |  |

Table 2 Summary results of Random Access conditions

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -24.4% | -13.4% | -20.7% | -19.8% | -7.9% | -17.1% |
| YCbCr 4:4:4 | -1.2% | -10.0% | -8.1% | -1.3% | -6.9% | -7.6% |
| YCbCr 4:2:2 | 0.6% | -9.3% | -7.5% | 1.3% | -3.6% | -4.9% |
| Overall | -9.1% | -11.0% | -12.5% | -7.2% | -6.2% | -10.2% |
|  | -8.8% | -12.2% | -11.8% | -8.5% | -7.6% | -11.0% |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |

Table 3 Summary results of Low delay B conditions

|  | # Low delay B HE Main-tier |  |  | # Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -25.2% | -10.3% | -19.7% | -19.4% | -5.3% | -16.5% |
| YCbCr 4:4:4 | -1.5% | -10.1% | -9.1% | -1.7% | -6.3% | -8.3% |
| YCbCr 4:2:2 | 0.4% | -9.5% | -8.6% | 1.0% | -3.2% | -5.6% |
| Overall | -9.5% | -10.0% | -12.8% | -7.3% | -5.0% | -10.4% |
|  | -9.0% | -8.0% | -11.2% | -8.5% | -5.0% | -10.6% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |

# Conclusions

We have verified results reported in JCTVC-M0072. The results of the cross-check matched with proponent’s report. The encoding / decoding run-times also matched closely to those reported by the proponent.

# References

- S. Matsuo, et al., “AHG7: Modification of in-loop colour-space transformation”, JCTVC-M0072, Incheon, KR, 18–26 Apr. 2013.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, Geneva, CH, 14–23 January, 2013.