| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0408 |
| --- | --- |

| Title: | Cross-check of post filter for colour-space transformation (JCTVC-M0080) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution reports the cross-check of post filter for colour-space transformation, which is proposed in JCTVC-M0080.

# Introduction

The post filter for colour-space transformation is presented in the document JCTVC-M0080 [1]. In the proposed method, the transform matrix signals to a decoder as a SEI message. This contribution reports the cross-check of this proposal.

The software version was HM10.0 RExt2.0. The reference was generated by using the common test condition [2] with Visual Studio 2007. Provided software was compiled on 64-bit Windows 7 environment with Visual Studio 2010 due to the library issue. The experiments were performed on the cluster PCs. Encoding / decoding run-times are unreliable.

Table 1 Summary results of All Intra conditions

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_12bit_444_crop.rgb | -34.8% | -16.0% | -37.9% | -26.2% | -8.5% | -31.3% | -17.2% | -2.6% | -24.4% |
| Kimono1_1920x1080_24_10bit_444.rgb | -27.1% | -11.2% | -27.8% | -10.1% | 18.0% | -16.3% | 9.1% | 19.3% | 0.3% |
| EBULupoCandlelight_1920x1080_50_10bit_444.rgb | -42.9% | -39.3% | -40.3% | -42.4% | -33.7% | -41.7% | -39.8% | -22.3% | -41.8% |
| EBURainFruits_1920x1080_50_10bit_444.rgb | -49.1% | -34.5% | -45.5% | -46.0% | -30.2% | -41.7% | -41.5% | -23.3% | -37.1% |
| VenueVu_1920x1080_30_8bit_444.rgb | -27.0% | -24.0% | -25.8% | -23.3% | -20.7% | -22.1% | -18.8% | -16.8% | -18.1% |
| DucksAndLegs_1920x1080_30_10bit_444.rgb | -14.2% | 10.2% | -13.1% | 13.7% | 14.7% | 11.1% | 28.5% | 17.5% | 22.5% |
| OldTownCross_1920x1080_50_10bit_444.rgb | -28.3% | -2.2% | -25.8% | -3.8% | 3.9% | -8.1% | 3.1% | 1.4% | -0.8% |
| ParkScene_1920x1080_24_10bit_444.rgb | -23.6% | -13.5% | -21.6% | -15.8% | 0.8% | -14.8% | -2.4% | 3.9% | -6.0% |
| Overall | -30.9% | -16.3% | -29.7% | -19.2% | -7.0% | -20.6% | -9.9% | -2.9% | -13.2% |
| Enc Time[%] | 88% |  |  | 88% |  |  | 89% |  |  |
| Dec Time[%] | 83% |  |  | 81% |  |  | 80% |  |  |

Table 2 Summary results of Random Access conditions

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_12bit_444_crop.rgb | -40.4% | -13.9% | -42.7% | -34.8% | 0.0% | -38.7% |
| Kimono1_1920x1080_24_10bit_444.rgb | -32.9% | -35.9% | -35.2% | -33.6% | 5.0% | -37.7% |
| EBULupoCandlelight_1920x1080_50_10bit_444.rgb | -37.5% | -35.1% | -36.3% | -42.9% | -36.9% | -43.3% |
| EBURainFruits_1920x1080_50_10bit_444.rgb | -46.0% | -30.0% | -46.8% | -47.9% | -29.3% | -47.9% |
| VenueVu_1920x1080_30_8bit_444.rgb | -22.8% | -22.3% | -25.2% | -21.8% | -21.2% | -23.7% |
| DucksAndLegs_1920x1080_30_10bit_444.rgb | -36.2% | 2.4% | -35.1% | -8.2% | 42.8% | -5.2% |
| OldTownCross_1920x1080_50_10bit_444.rgb | -40.3% | -28.8% | -52.7% | -9.3% | 16.2% | -15.8% |
| ParkScene_1920x1080_24_10bit_444.rgb | -25.2% | -24.5% | -27.4% | -25.5% | 1.9% | -28.6% |
| Overall | -35.2% | -23.5% | -37.7% | -28.0% | -2.7% | -30.1% |
| Enc Time[%] | 84% |  |  | 79% |  |  |
| Dec Time[%] | 86% |  |  | 81% |  |  |

Table 3 Summary results of Low delay B conditions

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_12bit_444_crop.rgb | -45.2% | -11.2% | -48.7% | -39.0% | 5.1% | -45.4% |
| Kimono1_1920x1080_24_10bit_444.rgb | -33.8% | -32.2% | -33.7% | -40.1% | 15.3% | -42.0% |
| EBULupoCandlelight_1920x1080_50_10bit_444.rgb | -34.7% | -31.5% | -32.6% | -46.5% | -32.6% | -45.0% |
| EBURainFruits_1920x1080_50_10bit_444.rgb | -46.9% | -26.9% | -44.7% | -50.0% | -25.1% | -46.5% |
| VenueVu_1920x1080_30_8bit_444.rgb | -23.8% | -20.8% | -24.3% | -22.6% | -20.0% | -23.0% |
| DucksAndLegs_1920x1080_30_10bit_444.rgb | -46.9% | 8.5% | -44.7% | -12.7% | 42.8% | -6.3% |
| OldTownCross_1920x1080_50_10bit_444.rgb | -55.0% | -29.3% | -58.0% | -21.2% | 14.8% | -16.4% |
| ParkScene_1920x1080_24_10bit_444.rgb | -27.9% | -22.4% | -27.8% | -32.5% | 8.1% | -31.6% |
| Overall | -39.3% | -20.7% | -39.3% | -33.1% | 1.1% | -32.0% |
| Enc Time[%] | 85% |  |  | 82% |  |  |
| Dec Time[%] | 80% |  |  | 77% |  |  |

# Conclusions

We have verified results reported in JCTVC-M0080. The results of the cross-check matched with proponent’s report. The encoding / decoding run-times are unreliable due to the library issue.

# References

- M. Matsumura, et al., “AHG7: Post filter for colour-space transformation”, JCTVC-M0080, Incheon, KR, 18–26 Apr. 2013.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, Geneva, CH, 14–23 January, 2013.