| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0411 |
| --- | --- |

| Title: | RCE1: Results of in-loop color-space transformation of residual signals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution reports results of an in-loop color-space transformation for residual signals, which is included in RCE1 Test 2.1. This method transforms prediction error signals in RGB/YUV chroma format into those in a sub-optimal color space. Chroma sampling is aligned into 4:4:4 by up-sampling if chroma format is not 4:4:4. The transformation matrix is derived from pixel domain for each coding unit by a singular-value-decomposition (SVD). The color-space transformation is applied to prediction error of both intra and inter mode. No additional information is necessary because residual signals are always transformed. The Y/G BD-rate gains of the All Intra / Random Access / Low delay B are 9.8% / 12.2% / 13.5% for the HE Main-tier and 6.4% / 9.5% / 10.6% for the HE High-tier, respectively.

# Introduction

This contribution proposes an additional in-loop color-space transformation. Transformation is applied to  4:4:4 format after that chroma is up-sampled if the chroma format is not 4:4:4. Transformed target is intra / inter residual signals. Transformation matrices are derived from neighboring samples / reference samples by using a simplified singular-value-decomposition with limited iterations.

# Proposed method

## Overview

Block diagram of the proposed in-loop color-space transformation is illustrated in Fig. 1. Compared to the existing HEVC architecture, matrix derivation and color-space transformation / inverse transformation are added. The input of the matrix derivation is either local decoded image or frame buffer. The output of the matrix derivation is a transformation matrix for each coding unit. After intra / inter prediction, residual signals are transformed by the derived matrix. Resulting signals are coded by the existing tools in the HEVC. When a chroma format is not 4:4:4, chroma samples are up-sampled so that the transformation is applied. In the following section, we describe the each component in Fig. 1.

Figure 1 Block diagram of the proposed in-loop color-space transformation. Matrix derivation is applied to signals in pixel domain. Resulting matrix is applied to signals in residual domain.

## Transform and inverse transform

In the forward operation, color-space transformation matrix is applied to three planes G, B, and R as follows,

.

Resulting values are clipped within the range of the HEVC specification because values are enlarged of  times in the worst case.

In the inverse operation, color-space transformation matrix is applied the three components P’, Q’, and R’ as follows,

.

## Derivation algorithm of transformation matrix

Transform matrix is derived from the reference sample values. Difference reference samples are utilized for the intra case and inter case.

For the case of intra block, target block and reference samples are shown in Fig. 2. In this figure, target block consists of 8x8 gray samples and references are orange and purple samples.

Figure 2 An example of the target block and reference sample for the case of intra 8x8 block.

Actual reference samples depend on the intra prediction mode. Orange pixels are referred to when the intra prediction mode is the range from two to ten, while purple pixels are referred to when the intra prediction mode is in the rage from 26 to 34. Otherwise, both orange and purple pixels are referred to. In any case, the number of reference samples becomes the power-of-two. Therefore upper left region is not included due to the violation of shift operation to compute an average.

For the case of inter block, reference samples for the matrix derivation is the same as that for motion compensation. In order to realize the shift operation, reference samples in the AMP block is sub-sampled such that the number of samples becomes the power-of-two. For example, the number of reference samples in a 12x16 block is reduced to 2/3.

After getting the reference samples, an average of sample values in the same plane is subtracted from the reference sample values. A covariance matrix of three planes is computed from the subtracted values. For example, when the number of reference pixels is 32, a covariance matrix is computed by . The covariance matrix is then normalized by the shit operation so that maximum value of diagonal elements become within the range from 4096 to 8191.

After generating normalized covariance matrix, SVD is applied based on Jacob algorithm. The maximum number of main iterations is decided to three. A square root computation of arbitrary number is also realized by integer operation with two iterations. Finally, each column is sorted so that values of diagonal elements are decreased and then transposed. The obtained matrix is named . When the reference samples are not available,  is a unit matrix.

## Coding structure restriction and syntax

Coding structure is required to be identical among all planes because of the color space transformation. For instance, following items need the same properties; the type of prediction unit, the mode of intra prediction. Although this restriction can reduce the some syntax elements, such syntax modification is not applied in order to keep the existing syntax. As a result, any syntax modification is not required for this scheme.

# Experimental results

The proposed mode is implemented to HM10.0-RExt2.0. A test condition follows RCE1 test condition [2] and the common test condition for HEVC range extensions [3]. Following tables show the summary of BD-rate gain and codec runtime.

Table 1 Results of the all intra conditions

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -28.8% | -19.2% | -27.8% | -21.3% | -13.7% | -20.8% | -14.9% | -9.2% | -14.5% |
| YCbCr 4:4:4 | -1.9% | -7.4% | -10.3% | -2.6% | -3.5% | -9.5% | -2.0% | -1.9% | -6.9% |
| YCbCr 4:2:2 | 4.1% | -8.4% | -8.2% | 7.0% | -3.0% | -5.6% | 11.0% | -0.8% | -3.0% |
| Overall | -9.8% | -12.0% | -16.0% | -6.4% | -7.1% | -12.4% | -2.6% | -4.2% | -8.4% |
|  | -9.8% | -12.3% | -15.9% | -6.4% | -7.5% | -12.5% | -2.9% | -4.4% | -8.1% |
| Enc Time[%] | 108% |  |  | 108% |  |  | 107% |  |  |
| Dec Time[%] | 109% |  |  | 108% |  |  | 106% |  |  |

Table 2 Results of the random access conditions

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -34.4% | -13.7% | -29.1% | -27.4% | -3.1% | -22.3% |
| YCbCr 4:4:4 | -1.5% | -12.2% | -13.0% | -2.3% | -6.2% | -13.3% |
| YCbCr 4:2:2 | 2.4% | -13.5% | -11.7% | 3.8% | -5.3% | -8.9% |
| Overall | -12.2% | -13.2% | -18.4% | -9.5% | -4.8% | -15.2% |
|  | -11.7% | -14.5% | -18.0% | -10.9% | -6.7% | -15.7% |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |

Table 3 Results of the low delay B conditions

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -35.6% | -8.4% | -27.0% | -27.5% | 0.2% | -21.0% |
| YCbCr 4:4:4 | -3.2% | -12.0% | -15.0% | -4.3% | -4.6% | -15.1% |
| YCbCr 4:2:2 | 1.6% | -13.2% | -12.9% | 2.6% | -3.9% | -9.6% |
| Overall | -13.5% | -11.1% | -18.7% | -10.6% | -2.6% | -15.5% |
|  | -12.3% | -8.1% | -16.4% | -11.1% | -3.3% | -15.7% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |

# Conclusion

This contribution proposed in-loop color-space transformation of residual signals. The proposed method reduced the redundancy of color space in 4:4:4 chroma format. Since transformation matrix was derived from reconstructed samples on the both encoder and decoder side, no side information was necessary. On the other hand, actual transformation target is signals in residual domain. Computation of matrix derivation is completely realized by integer multiplication, addition, and shift operation with limited iterations.

Experimental results showed that the proposed method achieved 9.8% / 12.2% / 13.5% BD-rate reduction respectively for the All Intra, Random Access and Low delay B of HE Main-tier.

Since the in-loop color-space transformation achieves solid coding gain with inter-plane correlation, we recommend adopting it in HEVC range extensions.

# References

- K. Kawamura, et al., “AHG7: In-loop color-space transformation of residual signals for range extensions”, JCTVC-L0371, Geneva, CH, 14–23 January, 2013.

- J. Sole, et al., “HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods”, JCTVC-L1121, Geneva, CH, 14–23 January, 2013.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1121, Geneva, CH, 14–23 January, 2013.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).