| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0412 |
| --- | --- |

| Title: | AHG5: CU based chroma intra prediction with reduced reference |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution proposes the CU-based chroma prediction by reduced references which predicts chroma samples using linear combination of luma samples for non 4:2:0 format. When a block size is large, a load of the parameter derivation process for each coding unit is reduced by using the limited reference samples in the worst case. The Y/G BD-rate gains of HE Main / High / Super-High tiers are 7.0% / 5.5% / 4.2% for all intra conditions. Compared with the top of CU-based chroma prediction in JCTVC-M0097, the Y/G BD-rate loss is less than 0.01% for whole AI conditions while the number of reference samples are limited.

# Introduction

HEVC range extensions study the chroma intra prediction based on the linear combination of luma samples which was called LM chroma in short. Due to the hardware complexity issue, the LM mode was dropped at the profile discussion of HEVC version 1.

Non-4:2:0 chroma format consists of larger amount of chroma pixels than that of 4:2:0 chroma format. Therefore, contribution by the LM mode to BD-rate becomes large.

In the RCE1, LM chroma is investigated deeply. The complexity of LM chroma is disclosed. For instance, computation cost of the parameter derivation is described in the JCTVC-M0097. The top of the LM chroma mode reduces such the cost by using CU based derivation.

This contribution proposes a further simplification to the top of LM chroma mode proposed in the JCTVC-M0097. A key approach is a reduction of the number of reference samples when a block size is large.

# Proposed method and test set

## Description

A tool of a luma-chroma intra prediction (LM mode) can reduce an inter component redundancy. The LM mode being applied to each TU block, a prediction coefficient derivation occurs for each TU block. In JCTVC-M0097 method 2, the derivation load for each CU is reduced by using CU based parameter derivation. This approach requires the heavy load for a large block because the all neighboring samples are utilized for parameter derivation.

The proposed method reduces the reference samples in the parameter derivation by using sub-sampling. In the viewpoint of the worst case, the sub-sampling is utilized when the block size is larger than 4x8 or 8x8 for 4:2:2 and 4:4:4 chroma format. Actual sub-sampling methods are shown in Fig. 1 and Fig. 2 for 4:2:2 and 4:4:4 chroma format. There are no parsing dependency to the size of CU.

In this contributions, the CU based method is named proposal 1 and the TU based method is named proposal 2.

Figure 1 Reference sample positions for 4:2:2 chroma format.

Figure 2 Reference sample positions for 4:2:2 chroma format.

## Test 1

Test 1 compares the CU based proposal (proposal 1) with HM10.0_RExt2.0 anchor.

## Test 2

Test 2 compares the CU based proposal (proposal 1) with CU based luma-chroma prediction such as JCTVC-M0097 method 2.

## Test 3

Test 3 compares the TU based proposal (proposal 2) with TU based luma-chroma prediction such as JCTVC-M0097 method 1.

# Number of computation in a CU

In this section, a derivation load like the number of add/mul operation is clarified. One architecture considers the load for each LCU, while another architecture considers the cost for each CU. Result by other condition can be obtained from the attached Excel file, JCTVC-M0412.xlsx.

Table 1 The worst case for each CU

| LCU | 64 | JCTVC-M0097 |  | JCTVC-M0412 |  |
| --- | --- | --- | --- | --- | --- |
| CU | 64 | Method 1 | Method 2 | Proposal 2 | Proposal 1 |
| TU | 32 | TU base | CU base | TU base | CU base |
| L & C | add | 124 | 252 | 60 | 124 |
| LL & LC | add | 124 | 252 | 60 | 124 |
|  | mult | 128 | 256 | 64 | 128 |
| misc | add | 14 | 14 | 14 | 14 |
|  | mult | 9 | 9 | 9 | 9 |
| per CU | derivation | 4 | 1 | 4 | 1 |
|  | add | 1048 | 518 | 536 | 262 |
|  | mult | 548 | 265 | 292 | 137 |
| per LCU | derivation | 4 | 1 | 4 | 1 |
|  | add | 1048 | 518 | 536 | 262 |
|  | mult | 548 | 265 | 292 | 137 |

Table 2 The worst case for each CU/LCU

| LCU | 64 | JCTVC-M0097 |  | JCTVC-M0412 |  |
| --- | --- | --- | --- | --- | --- |
| CU | 64 | Method 1 | Method 2 | Proposal 2 | Proposal 1 |
| TU | 4 | TU base | CU base | TU base | CU base |
| L & C | add | 12 | 252 | 12 | 124 |
| LL & LC | add | 12 | 252 | 12 | 124 |
|  | mult | 16 | 256 | 16 | 128 |
| misc | add | 14 | 14 | 14 | 14 |
|  | mult | 9 | 9 | 9 | 9 |
| per CU | derivation | 256 | 1 | 256 | 1 |
|  | add | 9728 | 518 | 9728 | 262 |
|  | mult | 6400 | 265 | 6400 | 137 |
| per LCU | derivation | 256 | 1 | 256 | 1 |
|  | add | 9728 | 518 | 9728 | 262 |
|  | mult | 6400 | 265 | 6400 | 137 |

Table 3 The worst case for each LCU

| LCU | 64 | JCTVC-M0097 |  | JCTVC-M0412 |  |
| --- | --- | --- | --- | --- | --- |
| CU | 8 | Method 1 | Method 2 | Proposal 2 | Proposal 1 |
| TU | 4 | TU base | CU base | TU base | CU base |
| L & C | add | 12 | 28 | 12 | 28 |
| LL & LC | add | 12 | 28 | 12 | 28 |
|  | mult | 16 | 32 | 16 | 32 |
| misc | add | 14 | 14 | 14 | 14 |
|  | mult | 9 | 9 | 9 | 9 |
| per CU | derivation | 4 | 1 | 4 | 1 |
|  | add | 152 | 70 | 152 | 70 |
|  | mult | 100 | 41 | 100 | 41 |
| per LCU | derivation | 256 | 64 | 256 | 64 |
|  | add | 9728 | 4480 | 9728 | 4480 |
|  | mult | 6400 | 2624 | 6400 | 2624 |

# Experimental results

The proposed methods are implemented to HM10.0-RExt2.0. A test condition follows RCE1 test condition [2] and the common test condition for HEVC range extensions [3]. Following tables show the summary of BD-rate gain and codec runtime.

Table 4 Results of the all intra conditions on the test 1

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -17.7% | -16.2% | -16.7% | -13.4% | -12.8% | -13.1% | -9.9% | -9.6% | -9.7% |
| YCbCr 4:4:4 | -1.8% | -6.3% | -5.9% | -1.9% | -4.0% | -5.2% | -1.8% | -2.6% | -3.7% |
| YCbCr 4:2:2 | -0.1% | -1.2% | -0.9% | -0.1% | -0.7% | -0.6% | -0.1% | -0.4% | -0.4% |
| Overall | -7.0% | -8.3% | -8.2% | -5.5% | -6.1% | -6.6% | -4.2% | -4.4% | -4.8% |
|  | -7.0% | -8.3% | -8.3% | -5.5% | -6.2% | -6.7% | -4.2% | -4.5% | -4.7% |
| Enc Time[%] | 107% |  |  | 107% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 5 Results of the all intra conditions on the test 2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.04% | 0.05% | 0.05% | 0.04% | 0.05% | 0.05% | 0.03% | 0.04% | 0.04% |
| YCbCr 4:4:4 | -0.01% | 0.00% | -0.08% | 0.00% | 0.01% | -0.04% | 0.00% | 0.02% | 0.00% |
| YCbCr 4:2:2 | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.01% | 0.02% | -0.01% | 0.01% | 0.02% | 0.00% | 0.01% | 0.02% | 0.02% |
|  | 0.02% | 0.02% | -0.01% | 0.01% | 0.02% | 0.01% | 0.01% | 0.02% | 0.02% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 6 Results of the all intra conditions on the test 3

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.03% | 0.04% | 0.03% | 0.03% | 0.04% | 0.04% | 0.02% | 0.03% | 0.04% |
| YCbCr 4:4:4 | -0.02% | -0.04% | -0.10% | -0.01% | 0.00% | -0.05% | 0.00% | 0.02% | 0.00% |
| YCbCr 4:2:2 | 0.01% | 0.03% | 0.02% | 0.01% | 0.03% | 0.02% | 0.01% | 0.02% | 0.02% |
| Overall | 0.01% | 0.01% | -0.01% | 0.01% | 0.02% | 0.01% | 0.01% | 0.02% | 0.02% |
|  | 0.01% | 0.01% | -0.01% | 0.01% | 0.02% | 0.00% | 0.01% | 0.02% | 0.02% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

# Conclusion

This contribution proposes the CU based chroma intra prediction by reduced reference. The load of the parameter derivation process for each CU is reduced at the worst case while the BD-rate loss is less than 0.01%. We recommend adopting LM mode with reduced reference in HEVC range extensions.

# References

- J. Kim, “RCE1: The performance of extended chroma mode for non 4:2:0 format”, JCTVC-M0097, Incheon, KR, 18–26 Apr. 2013.

- J. Sole, et al., “HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods”, JCTVC-L1121, Geneva, CH, 14–23 January, 2013.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1121, Geneva, CH, 14–23 January, 2013.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).