| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0413 |
| --- | --- |

| Title: | SCE5: Results of test 5.1.4 on inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution reports the results of inter-layer motion-vector prediction using HEVC base layer, which is included in SCE5 Test 5.1.4. This method consists of two parts. One is to change the order of motion vector candidates for the inter-layer predicted motion vector. The other is refinement of the predicted motion vector, which is similar manner of SVC. The Y BD-rate gains of RA 2x / 1.5x and LD-P 2x / 1.5x are 0.03% / -0.17% (degraded) and 0.22% / 0.06%, respectively.

# Introduction

This contribution proposes the inter-layer motion-vector prediction for HEVC scalable extensions. The motion vector candidate is generated by using similar manner of SVC. For instance, a motion vector in the base layer becomes that in the enhancement layer. Generated motion vector is inserted to motion vector candidates in the merge mode. Results by a combination with uncompressed motion vector are also provided.

# Proposed method

## Proposal 1

Proposal 1 is only to change the order of motion vector candidate in the merge mode. The number of MV candidates in the merge mode is kept to five. Motion vector derivation is same in SHM1.0. Motion vector compression is enabled.

## Proposal 2

Proposal 2 is combination of the proposal 1 and the MV refinement described as follows. Motion vector compression is enabled.

A procedure for motion vector refinement is similar manner of SVC. The collocated base layer is specified for each 4x4 pixels’ block of an enhancement layer. When the collocated base layer is coded by an inter mode, RefIdx and up-sampled MV of the collocated base layer is assigned to the 4x4 block of the enhancement layer. When the collocated base layer is coded by intra mode, RefIdx and up-scaled MV in the neighboring blocks of the base layer is assigned to the 4x4 block. If the all neighboring block is coded by an intra mode, zero MV is assigned to the 4x4 block. Four MVs in the 8x8 pixels’ block of the enhancement layer are replaced by the MV which is associated with the smallest RefIdx. This operation is applied for each L0 and L1 list. If two or four 4x4 blocks in the 8x8 block have similar L0 MV and L1 MV, such 4x4 blocks are merged and get new L0 MV and L1 MV based on the average of MVs.

This refinement is effective in the 1.5x cases because the each 8x8 pixels’ block has the same motion vector and RefIdx in the 2.0x case. A motion-vector up-scaling bug in SHM1.0 cases difference in the experimental results. This bug is fixed in SHM1.1.

## Proposal 3

Proposal 3 is the order change (proposal 1) with uncompressed motion vector.

# Experimental results

The proposed method is implemented to the SHM1.0. A test condition follows the SCE5 description [2]. AI condition has no impact because the proposed method is applied to the inter frame. Following tables show the summary of BD-rate gain and codec runtime.

Table 1 Results by the proposal 1

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% | 0.3% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.2% | 0.3% | 0.3% |
| Overall (Test vs single layer) | 19.3% | 32.0% | 33.0% | 16.7% | 28.8% | 31.2% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.6% | 0.7% | 0.7% |
| Enc Time[%] | 100.2% |  |  | 100.2% |  |  |
| Dec Time[%] | 100.1% |  |  | 99.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.2% | -0.1% |  |  |  |
| Class B | -0.2% | 0.2% | 0.1% | -0.1% | 0.2% | 0.2% |
| Overall (Test vs Ref) | -0.2% | 0.1% | 0.0% | -0.1% | 0.2% | 0.2% |
| Overall (Test vs single layer) | 25.8% | 36.6% | 38.2% | 22.4% | 33.4% | 36.2% |
| EL only (Test vs Ref) | -0.3% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 100.1% |  |  | 100.0% |  |  |
| Dec Time[%] | 99.9% |  |  | 99.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2 Results by the proposal 2

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.3% | 0.5% | 0.5% |
| Overall (Test vs Ref) | -0.1% | 0.0% | 0.0% | 0.3% | 0.5% | 0.5% |
| Overall (Test vs single layer) | 19.2% | 32.0% | 33.0% | 16.9% | 29.1% | 31.5% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.9% | 1.0% | 1.0% |
| Enc Time[%] | 100.2% |  |  | 100.3% |  |  |
| Dec Time[%] | 106.2% |  |  | 105.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.2% | -0.1% |  |  |  |
| Class B | -0.2% | 0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| Overall (Test vs Ref) | -0.2% | 0.0% | -0.1% | 0.0% | 0.2% | 0.2% |
| Overall (Test vs single layer) | 25.8% | 36.5% | 38.0% | 22.5% | 33.4% | 36.1% |
| EL only (Test vs Ref) | -0.3% | -0.1% | -0.2% | 0.1% | 0.3% | 0.2% |
| Enc Time[%] | 100.1% |  |  | 100.0% |  |  |
| Dec Time[%] | 105.3% |  |  | 105.3% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 3 Results by the proposal 3

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.4% | -2.6% | -2.5% |  |  |  |
| Class B | -0.6% | -1.1% | -1.2% | -0.2% | -0.3% | -0.3% |
| Overall (Test vs Ref) | -0.8% | -1.5% | -1.6% | -0.2% | -0.3% | -0.3% |
| Overall (Test vs single layer) | 18.3% | 29.9% | 30.9% | 16.3% | 28.1% | 30.4% |
| EL only (Test vs Ref) | -1.1% | -1.9% | -1.9% | 0.2% | 0.0% | -0.1% |
| Enc Time[%] | 99.5% |  |  | 99.9% |  |  |
| Dec Time[%] | 100.3% |  |  | 99.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -1.3% | -2.0% | -1.9% |  |  |  |
| Class B | -0.7% | -0.6% | -0.7% | -0.4% | -0.2% | -0.2% |
| Overall (Test vs Ref) | -0.9% | -1.0% | -1.1% | -0.4% | -0.2% | -0.2% |
| Overall (Test vs single layer) | 24.9% | 35.1% | 36.7% | 22.1% | 32.9% | 35.6% |
| EL only (Test vs Ref) | -1.3% | -1.4% | -1.5% | -0.4% | -0.2% | -0.3% |
| Enc Time[%] | 99.4% |  |  | 99.8% |  |  |
| Dec Time[%] | 100.0% |  |  | 99.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

This contribution proposed inter-layer syntax prediction for HEVC scalable extensions. The order change is effective for some sequences, while the MV refinement causes the degradation.

# References

- K. Kawamura, et al., “TE5: Results of test 5.2.8 on inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer”, JCTVC-L0376, Geneva, CH, 14–23 January, 2013.

- V. Seregin, et al., “Description of Core Experiment SCE5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-L1105, Geneva, CH, 14–23 January, 2013.

- X. Li, et al., “Common Test Conditions and Software Reference Configurations for the Scalable Test Model”, JCTVC-L1009, Geneva, CH, 14–23 January, 2013.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).