| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0414 |
| --- | --- |

| Title: | AHG15: Inter-layer motion-vector prediction using AVC base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution reports the investigation of inter-layer motion-vector prediction using AVC base layer. This prediction consists of two parts. One part is to change of MV candidates order for the predicted MV in the merge mode. The other part is refinement of the predicted MV, which is similar manner of SVC. Additional results by combinations with MV compression and POC issues are also provided.

# Introduction

This contribution proposes the inter-layer motion-vector prediction for HEVC scalable extensions using AVC base layer [1]. In this contribution, a inter-layer motion vector prediction and a related technologies are investigated. Results by introducing IL_MV is provided with and without motion vector compression.  A POC issue is addressed and affects coding gain. The motion vector candidate is generated by using similar manner of SVC. For instance, a motion vector in the base layer becomes that in the enhancement layer. Generated motion vector is inserted to motion vector candidates in the HEVC architecture. Six test cases for five proposals are described as follows.

# Proposed methods and test cases

## Test 1

The proposal 1 is introduction of an inter layer MV into SHM1.1 with a AVC base layer. The motion vector is uncompressed.

## Test 2

The proposal 2 is introduction of a compressed motion vector into the proposal 1. The anchor is the test 1.

## Test 3

SHM1.1 does not take care of the reference frame difference between a base layer and an enhancement layer. This situation occurs at the last GOP because a POC is different between AVC and HEVC when a length of the last GOP is not equal to 8.

The proposal 3 is correction of the above POC alignment issue. When the reference frame is not valid, a reference list is filled in zero. The anchor is the test 1. Alternative options is to change the test condition regarding the sequence length. For instance, the sequence length should be the multiple of eight.

## Test 4

Proposal 4 is to change the order of motion vector candidate in the merge mode. The number of MV candidates in the merge mode increase from five to six. This is the same as the proposal 1 in the JCTVC-M0414. The anchor is the test 1.

## Test 5

Proposal 5 is the MV refinement described in the proposal 2 in the JCTVC-M0414. Test 5 is combination of the proposal 4 and the proposal 5. The anchor is the test 1.

The refinement is no effective in the case of 2x.

## Test 6

Test 6 is combination of proposal 2 and proposal 4. The anchor is the test 1.

# Experimental results

The proposed method is implemented to the SHM1.1. A test condition follows the common test conditions for the scalable test model [2]. AI condition has no impact because the proposed method is applied to the inter frame. Following tables show the summary of BD-rate gain and codec runtime.

Table 1 Results by the test 1

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.9% | -4.5% | -4.0% |  |  |  |
| Class B | -1.2% | -2.7% | -2.9% | -1.3% | -2.5% | -2.8% |
| Overall (Test vs Ref) | -1.4% | -3.2% | -3.2% | -1.3% | -2.5% | -2.8% |
| Overall (Test vs single layer) | 30.9% | 41.8% | 42.4% | 38.4% | 48.4% | 51.8% |
| EL only (Test vs Ref) | -2.0% | -3.8% | -3.8% | -1.4% | -2.8% | -3.1% |
| Enc Time[%] | 96.8% |  |  | 96.3% |  |  |
| Dec Time[%] | 109.5% |  |  | 113.2% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -1.1% | -2.4% | -2.2% |  |  |  |
| Class B | -1.0% | -2.0% | -1.8% | -0.7% | -1.2% | -1.3% |
| Overall (Test vs Ref) | -1.0% | -2.1% | -1.9% | -0.7% | -1.2% | -1.3% |
| Overall (Test vs single layer) | 42.6% | 54.9% | 57.4% | 50.4% | 62.0% | 65.6% |
| EL only (Test vs Ref) | -1.6% | -2.8% | -2.6% | -0.8% | -1.4% | -1.6% |
| Enc Time[%] | 94.8% |  |  | 93.3% |  |  |
| Dec Time[%] | 110.5% |  |  | 115.0% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.7% | -1.7% | -1.5% |  |  |  |
| Class B | -0.5% | -1.0% | -0.8% | -0.5% | -0.8% | -0.8% |
| Overall (Test vs Ref) | -0.6% | -1.2% | -1.0% | -0.5% | -0.8% | -0.8% |
| Overall (Test vs single layer) | 585.2% | 624.3% | 635.6% | 989.0% | 1040.7% | 1064.6% |
| EL only (Test vs Ref) | -1.6% | -2.3% | -2.1% | -0.6% | -0.9% | -1.0% |
| Enc Time[%] | 94.9% |  |  | 94.0% |  |  |
| Dec Time[%] | 108.6% |  |  | 113.3% |  |  |

Table 2 Results by the test 2

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.3% | 3.0% | 2.6% |  |  |  |
| Class B | 0.6% | 1.1% | 1.2% | 0.4% | 0.7% | 0.8% |
| Overall (Test vs Ref) | 0.8% | 1.6% | 1.6% | 0.4% | 0.7% | 0.8% |
| Overall (Test vs single layer) | 31.9% | 44.3% | 44.8% | 39.0% | 49.5% | 53.0% |
| EL only (Test vs Ref) | 1.2% | 2.2% | 2.1% | 0.6% | 1.0% | 1.1% |
| Enc Time[%] | 100.7% |  |  | 100.2% |  |  |
| Dec Time[%] | 99.8% |  |  | 102.7% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.1% | 2.0% | 1.9% |  |  |  |
| Class B | 0.7% | 1.2% | 1.2% | 0.3% | 0.4% | 0.5% |
| Overall (Test vs Ref) | 0.8% | 1.4% | 1.4% | 0.3% | 0.4% | 0.5% |
| Overall (Test vs single layer) | 43.8% | 57.1% | 59.4% | 50.9% | 62.7% | 66.4% |
| EL only (Test vs Ref) | 1.4% | 2.1% | 2.1% | 0.5% | 0.7% | 0.7% |
| Enc Time[%] | 101.4% |  |  | 100.5% |  |  |
| Dec Time[%] | 102.4% |  |  | 102.4% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.7% | 1.4% | 1.2% |  |  |  |
| Class B | 0.3% | 0.6% | 0.6% | 0.2% | 0.3% | 0.4% |
| Overall (Test vs Ref) | 0.4% | 0.8% | 0.8% | 0.2% | 0.3% | 0.4% |
| Overall (Test vs single layer) | 587.8% | 629.8% | 641.0% | 991.2% | 1043.6% | 1069.0% |
| EL only (Test vs Ref) | 1.5% | 2.0% | 1.9% | 0.5% | 0.5% | 0.7% |
| Enc Time[%] | 101.3% |  |  | 100.4% |  |  |
| Dec Time[%] | 101.5% |  |  | 101.0% |  |  |

Table 3 Results by the test 3

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.3% |  |  |  |
| Class B | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% |
| Overall (Test vs Ref) | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |
| Overall (Test vs single layer) | 30.8% | 41.6% | 42.2% | 38.3% | 48.2% | 51.6% |
| EL only (Test vs Ref) | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |
| Enc Time[%] | 99.6% |  |  | 99.5% |  |  |
| Dec Time[%] | 100.1% |  |  | 100.6% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 42.6% | 54.9% | 57.4% | 50.4% | 62.0% | 65.6% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99.9% |  |  | 99.8% |  |  |
| Dec Time[%] | 97.4% |  |  | 97.2% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 585.2% | 624.3% | 635.6% | 989.0% | 1040.7% | 1064.6% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99.8% |  |  | 99.8% |  |  |
| Dec Time[%] | 96.6% |  |  | 94.7% |  |  |

Table 4 Results by the test 4

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.3% | 0.2% |  |  |  |
| Class B | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% |
| Overall (Test vs Ref) | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| Overall (Test vs single layer) | 30.8% | 41.8% | 42.4% | 38.2% | 48.2% | 51.6% |
| EL only (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.6% | -0.6% | -0.5% |
| Enc Time[%] | 100.0% |  |  | 99.9% |  |  |
| Dec Time[%] | 100.2% |  |  | 101.4% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.5% | 0.5% |  |  |  |
| Class B | -0.1% | 0.4% | 0.3% | -0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | 0.0% | 0.5% | 0.4% | -0.1% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 42.6% | 55.5% | 57.9% | 50.2% | 62.1% | 65.6% |
| EL only (Test vs Ref) | 0.1% | 0.6% | 0.5% | -0.3% | -0.1% | -0.1% |
| Enc Time[%] | 100.3% |  |  | 100.1% |  |  |
| Dec Time[%] | 101.8% |  |  | 102.8% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.3% | 0.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Overall (Test vs Ref) | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.1% |
| Overall (Test vs single layer) | 585.1% | 625.1% | 636.4% | 988.9% | 1040.8% | 1065.1% |
| EL only (Test vs Ref) | 0.1% | 0.3% | 0.3% | -0.3% | -0.2% | -0.2% |
| Enc Time[%] | 100.4% |  |  | 100.1% |  |  |
| Dec Time[%] | 100.0% |  |  | 100.4% |  |  |

Table 5 Results by the test 5

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.2% | 2.8% | 2.5% |  |  |  |
| Class B | 0.3% | 0.8% | 0.9% | 0.1% | 0.5% | 0.5% |
| Overall (Test vs Ref) | 0.6% | 1.4% | 1.4% | 0.1% | 0.5% | 0.5% |
| Overall (Test vs single layer) | 31.7% | 44.0% | 44.5% | 38.6% | 49.2% | 52.6% |
| EL only (Test vs Ref) | 0.8% | 1.7% | 1.6% | -0.1% | 0.3% | 0.4% |
| Enc Time[%] | 100.7% |  |  | 100.2% |  |  |
| Dec Time[%] | 99.0% |  |  | 102.2% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.9% | 2.0% | 1.8% |  |  |  |
| Class B | 0.4% | 1.2% | 1.1% | 0.1% | 0.5% | 0.5% |
| Overall (Test vs Ref) | 0.6% | 1.4% | 1.3% | 0.1% | 0.5% | 0.5% |
| Overall (Test vs single layer) | 43.4% | 57.1% | 59.4% | 50.6% | 62.8% | 66.4% |
| EL only (Test vs Ref) | 1.0% | 2.0% | 1.9% | 0.0% | 0.5% | 0.5% |
| Enc Time[%] | 101.6% |  |  | 100.7% |  |  |
| Dec Time[%] | 102.2% |  |  | 103.0% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.6% | 1.4% | 1.1% |  |  |  |
| Class B | 0.2% | 0.6% | 0.6% | 0.1% | 0.3% | 0.2% |
| Overall (Test vs Ref) | 0.3% | 0.8% | 0.7% | 0.1% | 0.3% | 0.2% |
| Overall (Test vs single layer) | 587.3% | 629.7% | 640.7% | 990.6% | 1044.0% | 1067.2% |
| EL only (Test vs Ref) | 1.1% | 1.7% | 1.6% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 101.7% |  |  | 100.7% |  |  |
| Dec Time[%] | 99.2% |  |  | 98.1% |  |  |

Table 6 Results by the test 6

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.3% | 0.2% |  |  |  |
| Class B | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% |
| Overall (Test vs Ref) | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| Overall (Test vs single layer) | 30.8% | 41.8% | 42.4% | 38.3% | 48.3% | 51.8% |
| EL only (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.4% | -0.4% | -0.3% |
| Enc Time[%] | 100.2% |  |  | 100.3% |  |  |
| Dec Time[%] | 104.5% |  |  | 107.5% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.5% | 0.5% |  |  |  |
| Class B | -0.1% | 0.4% | 0.3% | 0.0% | 0.1% | 0.2% |
| Overall (Test vs Ref) | 0.0% | 0.5% | 0.4% | 0.0% | 0.1% | 0.2% |
| Overall (Test vs single layer) | 42.6% | 55.5% | 57.9% | 50.4% | 62.2% | 65.8% |
| EL only (Test vs Ref) | 0.1% | 0.6% | 0.5% | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100.6% |  |  | 100.5% |  |  |
| Dec Time[%] | 102.2% |  |  | 100.9% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.3% | 0.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall (Test vs Ref) | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 585.1% | 625.1% | 636.4% | 989.4% | 1041.2% | 1065.6% |
| EL only (Test vs Ref) | 0.1% | 0.3% | 0.3% | -0.2% | -0.1% | -0.1% |
| Enc Time[%] | 100.6% |  |  | 100.5% |  |  |
| Dec Time[%] | 106.5% |  |  | 107.3% |  |  |

Table 7 Summary results

| # | Short description | RA-2x | RA-1.5x | LP-2x | LP-1.5x | LB-2x | LB-1.5x |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | BL MV + uncompressed MV | -1.40% | -1.33% | -1.03% | -0.70% | -0.57% | -0.54% |
| Test 2 | compressed MV | 0.77% | 0.39% | 0.81% | 0.32% | 0.40% | 0.19% |
| Test 3 | POC alignment | -0.10% | -0.08% | 0.00% | 0.00% | 0.00% | 0.00% |
| Test 4 | BL MV as a 3rd candidate | -0.09% | -0.16% | -0.03% | -0.13% | 0.02% | 0.00% |
| Test 5 | Test 4 + compressed MV | 0.56% | 0.15% | 0.55% | 0.11% | 0.32% | 0.14% |
| Test 6 | Test 4 + MV refinement | -0.09% | -0.10% | -0.03% | -0.04% | -0.02% | 0.03% |

# Conclusion

This contribution proposed inter-layer syntax prediction for HEVC scalable extensions. The proposed method predicted motion vector of the enhancement layer from that of the collocated base layer. Resulting motion vector was inserted to the merge mode.

# References

- K. Kawamura, et al., “TE5: Results of test 5.2.8 on inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer”, JCTVC-L0376, Geneva, CH, 14–23 January, 2013.

- X. Li, et al., “Common Test Conditions and Software Reference Configurations for the Scalable Test Model”, JCTVC-L1009, Geneva, CH, 14–23 January, 2013.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).