| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0416 |
| --- | --- |

| Title: | Cross-check of intra motion compensation by TI (JCTVC-M0350) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunil Lee Chanyul Kim | Email: | sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of the intra motion compensation for lossless coding of screen content sequences. The coding efficiency of the proposed method is independently evaluated by Samsung, and it was confirmed that the test results match those provided by the proponent.

# Introduction

This contribution reports the cross-verification results of the intra motion compensation for lossless coding of screen content sequences proposed by TI in JCTVC-M0350 [1]. In this contribution, the intra motion estimation and compensation along either vertical or horizontal direction is proposed. For the complexity reduction at the encoder, the test results with the constraint on the search range along vertical direction are also provided. The propose method is combined with SAP_HV [2] and cRiceParam update [3], and their test results are provided either.

# Test results

The proposed method is implemented on top of HEVC range extension software and tested under the common test condition of RCE2 defined in [4]. Please note that the coding efficiency of the proposed method is only evaluated for 10 screen content sequences. The cross-verification results are summarized in Table 1. The test results exactly match those provided by the proponent. The encoding/decoding time was not provided by the proponent, but it was observed that the encoding time increases significantly even in Test 2 (in which the search range along the vertical direction is restricted to LCU boundary) in our test results.

Table 1. Cross-verification results of JCTVC-M0350

| Test |  | AI | RA | LDB |
| --- | --- | --- | --- | --- |
| Test 1 | Bit-rate saving | -24.0% | -18.9% | -15.7% |
|  | Enc/Dec Time[%] | 138% / 85% | 113% / 109% | 109% / 99% |
| Test 2 | Bit-rate saving | -21.0% | -16.6% | -14.1% |
|  | Enc/Dec Time[%] | 136% / 87% | 112% / 110% | 108% / 102% |
| Test 3 | Bit-rate saving | -29.1% | -22.6% | -19.8% |
|  | Enc/Dec Time[%] | 134% / 85% | 113% / 110% | 108% / 101% |
| Test 4 | Bit-rate saving | -32.8% | -25.9% | -22.9% |
|  | Enc/Dec Time[%] | 133% / 84% | 113% / 108% | 109% / 100% |

# Conclusion

This contribution reports the cross-verification results of JCTVC-M0350. It was confirmed that the test results match those provided by the proponent. The encoding time increases in all the tests and all the configurations. The cross-checker suggests that the coding efficiency of this method for the other natural video sequences need to be checked and the trade-off between the coding efficiency and the complexity (especially encoder-side complexity) needs to be carefully investigated.

# References

- M. Budagavi and D.-K. Kwon, “AHG8: Video coding using Intra motion compensation,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0350, Apr. 2013.

- M. Zhou and M. Budagavi, “RCE2: Experimental results on Test 3 and Test 4,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0056, Apr. 2013.

- M. Budagavi, “AHG8: Coefficient level cRiceParam updates for screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0316, Apr. 2013.

- W. Gao, M. Zhou, P. Amen, and S. Lee, “HEVC Range Extension Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1122, Jan. 2013.