| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0418 |
| --- | --- |

| Title: | Cross-check of adaptive inter-plane prediction for RGB content by HHI (JCTVC-M0230) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd. Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This document reports the cross-check results for JCTVC-M0230 low-cost adaptive inter-plane prediction scheme for RGB contents. The proponents provided source with changes implemented within the recommended reference software HM-10.0_RExt2.0. The modified source code was studied and found to be consistent with the description in JCTVC-M230. Runtime and Complexity analysis was performed by counting the number of addition and shift operation used for the prediction.

# Introduction

The introduced technique in JCTVC-M0230 operates in a closed in-loop on the prediction residuals. In order to achieve the low-cost property, the scheme in JCTVC-M0230 employs a predefined number of predictors. The complexity in decoder is negligible. The predictions consist of one addition and one shift operation in maximum and the additional memory usage is needed by one bit for the chroma components. At the encoder, additional modes have to be tested, and hence, the more predictors are allowed the more extended the run time. The detailed complexity analysis are as follows.

- Prediction requires one addition and one shift operation in maximum

- Additional memory usage in increased by one bit for the chroma components

- Encoder complexity depends on the number of available prediction

- Additional one bit for the chroma components is required for weighting process

# Experimental Results

Test conditions as outlined in JCTVC-L1006 and JCTVC-L1122 “Common Range Extension test conditions and software reference configurations” were used. The all-Intra and random access configurations were tested

## Range Extension Sequence

The experiment has performed as mentioned in [1].  The following tables show the performance for the different color spaces relative to native RGB input using different weighting formulas for the RGB sequences recommended for the Range Extension development.

|  | All Intra |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Tier |  |  | High Tier |  |  | Super High Tier |  |  |
|  | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 |
| YCoCg | -34.87% | -33.73% | -27.28% | -25.54% | -24.13% | -16.08% | -16.97% | -15.84% | -9.22% |
| GRbRr | -35.88% | -40.51% | -20.73% | -28.27% | -33.84% | -12.64% | -20.85% | -26.81% | -7.00% |
| Fixed | -38.07% | -40.53% | -23.42% | -32.13% | -34.31% | -16.02% | -24.92% | -27.17% | -9.81% |
| N=1 | -39.40% | -41.85% | -25.18% | -33.98% | -36.20% | -18.39% | -27.30% | -29.53% | -12.68% |
| N=2 | -40.51% | -42.84% | -26.66% | -35.06% | -37.24% | -19.82% | -28.24% | -30.44% | -13.94% |

|  | Random Access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Tier |  |  | High Tier |  |  | Super High Tier |  |  |
|  | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 |
| YCoCg | -40.12% | -39.21% | -34.65% | -32.68% | -30.94% | -20.98% | -22.56% | -20.65% | -8.27% |
| GRbRr | -40.55% | -44.50% | -25.79% | -34.63% | -40.14% | -13.75% | -26.60% | -32.83% | -4.01% |
| Fixed | -42.20% | -44.58% | -28.76% | -38.53% | -40.60% | -18.42% | -30.85% | -32.67% | -7.64% |
| N=1 | -43.85% | -46.17% | -30.09% | -41.97% | -44.13% | -22.27% | -35.72% | -37.84% | -13.18% |
| N=2 | -44.32% | -46.55% | -30.64% | -42.45% | -44.56% | -22.98% | -36.22% | -38.28% | -13.96% |

|  | Low Delay |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Tier |  |  | High Tier |  |  | Super High Tier |  |  |
|  | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 |
| YCoCg | -42.35% | -41.33% | -35.43% | -34.33% | -32.51% | -21.41% | -23.08% | -21.20% | -8.42% |
| GRbRr | -43.87% | -47.42% | -27.23% | -38.02% | -43.07% | -15.38% | -27.91% | -34.05% | -4.49% |
| Fixed | -42.80% | -45.69% | -27.13% | -39.20% | -41.59% | -17.35% | - | - | - |
| N=1 | -45.02% | -47.86% | -28.68% | -43.43% | -46.06% | -21.52% | -35.78% | -38.27% | -12.05% |
| N=2 | -45.09% | -47.91% | -28.77% | - | - | - | - | - | - |

## Screen contents (Lossless)

For RGB screen content sequences, the lossless experiment was conducted as described in RCE2 [2].

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Compression Ratio |  | Savings |
| YCoCg | 8.01 | 9.11 | -12.30% |
| GRbRr | 8.01 | 9.46 | -15.32% |
| Fixed | 8.01 | 9.16 | -12.60% |
| N=1 | 8.01 | 9.82 | -18.01% |
| N=2 | 8.01 | 9.87 | -18.47% |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Compression Ratio |  | Savings |
| YCoCg | 98.28 | 104.99 | -11.16% |
| GRbRr | 98.28 | 108.77 | -13.31% |
| Fixed | 98.28 | 105.39 | -10.71% |
| N=1 | 98.28 | 111.28 | -13.28% |
| N=2 | 98.28 | 111.91 | -13.77% |

|  | Low Delay |  |  |
| --- | --- | --- | --- |
|  | Compression Ratio |  | Savings |
| YCoCg | 335.11 | 347.25 | -11.00% |
| GRbRr | 335.11 | 356.36 | -12.16% |
| Fixed | 335.11 | 346.73 | -9.70% |
| N=1 | 335.11 | 383.83 | -11.81% |
| N=2 | 335.11 | 385.00 | -12.09% |

# Conclusion

It is confirmed that the source implementation is consistent with the description in the proposal document JCTVC-M0230. The rate-distortion performance of the proposed tool has been verified.

# References

- D. Flynn, and K. Sharman, “Common Test Conditions and Software Reference,” JCTVC-L1006, Geneva, Switzerland, January 2013.

- W. Gao, M. Zhou, P. Amon, and S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” JCTVC-L1122, Geneva, Switzerland, January 2013.

- T. Nguyen, A. Khairat, D. Marpe, “RCE1/RCE2/AHG5/AHG8: Adaptive Inter-Plane Prediction for RGB Content,” JCTVC-M0230, Incheon, Korea, April 2013.