| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0419 |
| --- | --- |

| Title: | Cross-check of JCTVC-M0091 (Non SCE1-Combined Intra Prediction with low-pass filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jani Lainema | Tel: Email: | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides a cross-check for JCTVC-M0091 (Non SCE1-Combined Intra Prediction with low-pass filter). It is confirmed that both objective performance and run-times provided in the original proposal are accurately reported.

It is pointed out though that in r0 and r1 versions of JCTVC-M0091 there appears to be a typo in the equation defining the proposed operation. Correct form is believed to be Pred’EL= PredEL +Â·[PredBL - PredEL] which would correspond to the software implementation provided.

# Objective results

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -1.3% | -1.2% |  |  |  |
| Class B | -0.6% | -2.0% | -2.1% | -0.2% | -0.8% | -0.8% |
| Overall (Test vs Ref) | -0.6% | -1.8% | -1.8% | -0.2% | -0.8% | -0.8% |
| Overall (Test vs single layer) | 11.8% | 11.6% | 11.3% | 10.1% | 9.4% | 8.8% |
| EL only (Test vs Ref) | -0.5% | -1.8% | -1.9% | 0.4% | -0.4% | -0.4% |
| Enc Time[%] | 117.9% |  |  | 114.5% |  |  |
| Dec Time[%] | 105.0% |  |  | 103.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).