| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0421 |
| --- | --- |

| Title: | SCE1 : Cross check of 2.4 and 2.5 on Modification on textureRL, DC, Planar prediction (M0313) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Junghye Min Elena Alshina | Email: | jh643.min@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification of performance from three modifications in M0313 (Modifications on textureRL, DC, Planar predictions). The source code was reviewed and it was confirmed that implementation is consistent with algorithm descriptions in the proposal. BD-rate and PSNR results from our tests exactly match the results provided by proponents. The proposed approach provides performance gains of -0.5% and -0.1 for AI 2x and AI 1.5x test. Encoding/Decoding times are 97.7%/108.8% and 96.7%/104.7% for AI 2x and AI 1.5x compared to anchor.

# Introduction

LG, Sony and Qualcomm proposed several intra prediction methods using difference samples. In M0313, test results of following three modifications were presented. These modifications replace the texture RL, planar, DC modes. .

- Modified textureRL : Boundary process is performed to inter-layer texture prediction. Boundary process is performed using difference between samples of base layer and enhancement layer.

- Modified Planar : Right-top and bottom-left reference samples are set to zero for planar prediction.

- Modified DC : Similar to Modified textureRL. Only offset values for boundary process are different.

# Test results

The tests were performed under the conditions defined in JCTVC-L1101 [2] with AI 2x and AI 1.5x configurations. The bit rate and PSNR of our test results match with results proponents provided. Encoding/Decoding run times are similar with our results.  Simulation results are summarised in following tables.

# Conclusion

Performance from a combination of three intra tools with difference samples was verified. The proposed approach provides performance gains of -0.5% and -0.1 for AI 2x and AI 1.5x test. Encoding/Decoding times are 97.7%/108.8% and 96.7%/104.7% for AI 2x and AI 1.5x compared to anchor.

# References

[1] J. Park, J. Kim, B. Jeon, C. Auyeung, A.Tabatabai, K. Rapaka, J.Chen, X.Li, M.Karczewicz, SCE1 Category 2.4 and 2.5 : Modifications on texture RL, dc, planar prediction, JCTVC-M0313, 2013

[2] A. Tabatabai, K. Rapaka, A. Saxena and S. Liu, Description of Core Experiment SCE 1: Intra Prediction Improvements in SHVC, JCTVC-L1101, 2013

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).