| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0422 |
| --- | --- |

| Title: | SCE1 : Crosscheck of test 4.2.2 and 4.2.6 on Difference Intra Prediction (M0324) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Junghye Min Elena Alshina | Email: | jh643.min@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report is verification of performance from combined results of test 4.2.2 AND 4.2.6 on difference intra prediction in M0324. The source code was reviewed and it was confirmed that implementation is consistent with algorithm descriptions in the proposal. BD-rate and PSNR results from our tests exactly match the results provided by proponents. The proposed approach provides performance gains of -0.8% and -0.6 for AI 2x and AI 1.5x tests. Encoding/Decoding time increments for AI 2x and AI 1.5x are 64.8%/10.5% and 55.8%/8.2%.

# Introduction

LG, Sony and Qualcomm proposed Difference domain Intra Prediction (DIP) which uses the difference between the pixels of current neighbors and those of collocated BL neighbors for intra prediction in M0324. Mode dependent intra sample smoothing process is not applied for DIP. On CU level, 1 bit flag indicating whether DIP is applied is signaled. For prediction and reconstruction values in difference domain, clipping into [-128,127] is applied to keep the range size the same with the original domain.

# Test results

The tests were performed under the conditions defined in JCTVC-L1101 [2] with AI 2x and AI 1.5x configurations. The bit rate and PSNR of our test results match with results proponents provided. Encoding/Decoding run times are similar with our results.  Simulation results are summarised in following tables.

# Conclusion

Performance of Difference domain Intra Prediction was verified. The proposed approach provides performance gains of -0.8% and -0.6 for AI 2x and AI 1.5x tests. Encoding/Decoding time increments for AI 2x and AI 1.5x are 64.8%/10.5% and 55.8%/8.2%.

# References

[1] K. Rapaka, J.Chen, X.Li, M.Karczewicz, J. Park, J. Kim, B. Jeon, C. Auyeung, A.Tabatabai, SCE1 combined results of Test 4.2.2 and 4.2.6 on Difference Intra Prediction, M0324, 2013

[2] A. Tabatabai, K. Rapaka, A. Saxena and S. Liu, Description of Core Experiment SCE 1: Intra Prediction Improvements in SHVC, JCTVC-L1101, 2013

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).