| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0425 |
| --- | --- |

| Title: | About phase calculation and up-sampling filter coefficients in JCTVC-M0188 and JCTVC-M0322 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshina | Email: | elena_a.alshina@samsung.com alexander_b.alshin@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report presents spectral characteristics analysis for new up-sampling filters proposed by Inter Digital and Qualcomm for fractional positions not specified yet in SHVC Test Model 1/ SHVC Working Draft 1. Phase calculation bug in SHVC Test Model 1/ SHVC Working Draft 1 is reported and possible fixes are discussed.

# The bug in phase derivation in SHVC Test Model 1/ SHVC Working Draft 1

First, specification text bug was reported in JCTVC-M0188.

During up-sampling process the phase calculation is needed. Up-sampling filter coefficients are specified only for fractional positions which happen under common test condition (for scalability ratios 1:1, 2:1 and 3:2 only). Fractional positions and filter coefficients were agree during Shanghai meeting [1] and later in Geneva Chroma up-sampling filter coefficients were rounded to 5-bits precision in order to follow HEVC MC filter design.

Regarding phase calculation during Shanghai meeting [1] it was agreed to follow SVC design (including accurate Chroma alignment, which as was shown give no objective gain in SHVC).

Exactly following this agreement up-sampling process was implemented in the reference s/w. Even SVC code for phase derivation was re-used. The benefits of usage SVC phase derivation process and code are following:

- there is pixel-wise division operation;

- arbitrary scalability ratio is supported.

But unfortunately phase derivation process description in SHVC Test Model 1/ SHVC Working Draft 1 [3],[4] has nothing similar with implementation in a reference s/w:

===== SHVC Test Model 1/ SHVC Working Draft 1 text start=========

The variable xRef16 is derived as follow:

xRef16 = ( xP * PicWRL * 16 + ScaledW / 2) / ScaledW                       (G-3)

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

yRef16 = (yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH                        (G-4)

–	Otherwise, the variable yRef16 is derived as follow:

yRef16 = ( yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH – offset                 (G-5)

where the value of offset is derived as follows:

if (ScaledH  is equal to PicHRL )
			offset = 0
		otherwise if (ScaledH  is equal to 1.5 * PicHRL )
			offset = 1
		otherwise if (ScaledH  is equal to 2.0 * PicHRL )
			offset = 2.

....

xPhase = ( xRef16 ) % 16

....

yPhase = ( yRef16 ) % 16

===== SHVC Test Model 1/ SHVC Working Draft 1 text end=========

The first concern is division operation for each sample in (G-4) and (G-5). S/w implementation doesn’t have this division and it is not needed.

The second concern is not-general enough representation. Only scalability ratios 1:1, 2:1 and 3:2 are described in formulas above. Generalization for arbitrary scalability ratio is not clear.

The third concern is mismatch between text and s/w. The results of phase calculation (yPhase ) in Chroma vertical interpoaltion process  for yP=1, 4, 7, 10, ...,  is yPhase = 10; but in s/w impelmentation result of phase calulation for tehse positions is 9. Results of phase calcualtion for other cases under common test conditions are teh same for [3],[4] text and s/w.

In orderr to resolve these concerns the following modificaiton of secificaiton text is proposed.

===== Proposed SHVC Test Model 1/ SHVC Working Draft 1 text start=========

The variables xRef16 and yRef16 are derived as follow:

xRef16 = ((xP *iScaleX + iAddX) >> 12) - 4 * iPhaseX;  (G-3)

yRef16 = ((yP *iScaleY + iAddY) >> 12) - 4 * iPhaseY;  (G-4)

where

iPhaseX = 0

for cIdx =0,..,2;

iPhaseY = 0

for cIdx =0, but

iPhaseY = 1

for cIdx =1,2;

iAddX       = ( ( (PicWRL * iPhaseX ) <<  14 ) + (ScaledW >> 1 ) ) / ScaledW  + ( 1 << 11 );

iAddY       = ( ( (PicHRL * iPhaseY ) <<  14 ) + (ScaledH >> 1 ) ) / ScaledH + ( 1 << 11 );

iScaleX     = ( (PicWRL << 16 ) + (ScaledW>> 1 ) ) / ScaledW ;

iScaleY     = ( (PicHRL << 16 ) + (ScaledH >> 1 ) ) / ScaledH;

===== Proposed SHVC Test Model 1/ SHVC Working Draft 1 text end=========

Proposed text change match s/w implementation, works for arbitrary scalability ratio, doesn’t have division operation for each samples. Proposed change is identical to modification suggested in [2], but slightly shorter.

# Up-sampling filter analysis

Positions and phases derivation process in up-sampling module of SHVC Test Model 1/ SHVC Working Draft 1 assumes that there is no phase shift between enhancement and base layer picture. Currently used test sequences were generated using 0 phase shift.

The key idea of changes suggested in [1] and [2] is modification of Positions and phases derivation process in up-sampling module for different relative locations of samples on base and enhancement layers. The difference is location shall be signalled in a bit-stream.

Suggested in [1]-[2] modification doesn’t not have effect for currently used test sequences. The goal of these contribution is to extend SHVC flexibility for another variants of down-sampler (which is on-normative).

For modifications proposed in [1]-[2] up-sampling filter for all fractional positions is needed. Some of these filters are not specified yet. Table 1 below shows variants of Luma u-sampling filters suggested in [1] and [2]. In several cases suggestions are different. So we have analysed spectral characteristics for both variants and in several cases we suggest alternative variants.

Table 1. Luma up-sampling filters.

| Phase | Inter Digital | Qualcomm | Samsung |
| --- | --- | --- | --- |
| 0 | {0,0,0,64,0,0,0,0} | {0,0,0,64,0,0,0,0} | {0,0,0,64,0,0,0,0} |
| 1 | {0,1,-3,63,4,-2,1,0} | {0,1,-3,63,4,-2,1,0} | {0,1,-3,63,4,-2,1,0} |
| 2 | {-1,2,-6,62,9,-3,1,0} | {-1,2,-6,61,9,-3,1,0} | {-1, 2, -5, 62, 8, -3, 1,0} |
| 3 | {-1,3,-8,60,13,-5,2,0} | {-1,3,-8,60,13,-4,1,0} | {-1,3,-8,60,13,-4,1,0} |
| 4 | { -1, 4, -10, 57, 19, -7, 3,-1 } | { -1, 4, -10, 58, 17, -5, 1,0 } | { -1, 4, -10, 58, 17, -5, 1, 0 } |
| 5 | { -1, 4, -11,52, 26, -8, 3, -1,0} | { -1, 4, -11,52, 26, -8, 3, -1,0} | { -1, 4, -11,52, 26, -8, 3, -1,0} |
| 6 | { -1, 4, -11, 49, 29, -9, 4,-1 } | { -1, 4, -11, 50, 29, -9, 3,-1 } | { -1, 4, -11, 50, 28, -8, 2,0} |
| 7 | { -1, 4, -12, 45, 35, -10, 4,-1 } | { -1, 4, -11, 45, 34, -10, 4,-1 } | { -1, 4, -11, 44, 35, -10, 4,-1} |
| 8 | { -1,4,-11,40,40, -11, 4,-1 } | { -1,4,-11,40,40, -11, 4,-1 } | { -1,4,-11,40,40, -11, 4,-1 } |

For fractional positions 0,5 and 8 there is no disagreement and coefficients match with [3]-[4] (and so there is no performance change from [1] and [2] under common test conditions).

For fractional position 4 (phase shift =1/4) variant suggested by Qualcomm matches HEVC with MC interpolation filter. We support this variant rather than variant suggested by Inter Digital.

Phase =2. Target Phase shift is 1/8 = 0,125 here. In Qualcomm’s variant the sum of coefficients in not equal to 63. Most probably there is a typo. For variant presents by Inter Digital amplitude and Phase domain frequency responses are shown on Figures 1-2. We also show alternative variant which has smaller deviation from ideal response both in Amplitude and Phase domains.

| Fig. 1. Amplitude domain frequency responses for target phase shift 1/8. | Fig. 2. Phase domain frequency responses for target phase shift 1/8. |
| --- | --- |

Phase =3. Target Phase shift is 1/16here. For variant presents by Qualcomm Inter Digital amplitude and Phase domains frequency responses are shown on Figures 3-4. The one from Qualcomm’s contribution is preferable since has smaller deviation from ideal one.

| Fig. 3. Amplitude domain frequency responses for target phase shift 3/16. | Fig. 4. Phase domain frequency responses for target phase shift 3/16. |
| --- | --- |

Phase =4. Target Phase shift is ¼ here. This filter exists in HEVC specification. This is 7-taps DCTIF. The one proposed by Inter Digital was studied in HEVC CE3 (interpolation MC filter), this is 8 taps DCTIF. It was replaced by 7-taps due to the bomb in frequency response (Fig.5)  which was a reason for visual artefacts in LD-P tests. This is preferable to re-use existing MC Interpolation filter here.

| Fig. 5. Amplitude domain frequency responses for target phase shift 1/4. | Fig. 6. Phase domain frequency responses for target phase shift 1/4. |
| --- | --- |

Phase =6. Target Phase shift is 3/8 here. Both filters suggested by Inter Digital and Qualcomm have significant deviation from ideal frequency response (Fig.7-8). We would suggest the variant {-1, 4, -11, 50, 28, -8, 2,0} which is essentially 7-taps and closer to the ideal for this target phase shift.

| Fig. 7. Amplitude domain frequency responses for target phase shift 1/4. | Fig. 8. Phase domain frequency responses for target phase shift 1/4. |
| --- | --- |

Phase =7. Target Phase shift is 7/16 here. The filter suggested by Qualcomm has lower deviation from ideal frequency response (Fig.9-10). We would suggest the variant { -1, 4, -11, 44, 35, -10, 4,-1} which has slightly less error accumulation.

| Fig. 9. Amplitude domain frequency responses for target phase shift 1/4. | Fig. 10. Phase domain frequency responses for target phase shift 1/4. |
| --- | --- |

Among two proposals JCTVC-M0188 [1] and JCTVC-M0322 [2] filters proposed by Qualcomm have less deviation from ideal filter frequency response. This results in better performance shown by Qualcomm’s contribution [2]. At the same time suggested below alternative variants for up-sampling filters are even close to the ideal (especially for target phases 2 and 6). It means that potentially even higher gain could be achieved with accurate up-sampling filter design.

# Conclusion

Mismatch between SHVC Test Model 1/ SHVC Working Draft 1 and s/w needs to be fixed. We would suggest fixing it according to the s/w implementation which follows SVC, generically applied for arbitrary scalability ratio and doesn’t have division operation for each sample.

Up-sampling filters for fractional positions not specified in SHVC Test Model 1/ SHVC Working Draft 1 should be carefully studied. We would suggest establishing AhG which will create and make available share test content, specify test conditions and evaluation criteria.

# References

[1] E. Alshina, H. Lakshman, J.Dong, J.Chen, A.Luthra, “Suggested up-sampling filter design for tool experiments on HEVC scalable extension”, JCTVC-K0378, Shanghai, China, Oct. 2012.

[2] J. Chen, R. Krishna, M. Karczewicz “TE-A1: Results of test 4.3 on fixed up-sampling filter”, JCTVC-L0335, Geneva, CH, Jan. 2013.

[3] J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Test Model 1 (SHM 1)”, JCTVC-L1007, Shanghai, China, Oct. 2012.

[4] J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela “SHVC Working Draft 1”, JCTVC-L1008, Geneva, CH, Jan. 2013.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).