| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0429 |
| --- | --- |

| Title: | Update on Cross-segment Decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiangtao Wen and Spencer Cheng | Tel: Email: Tel: Email: | +86-15101679256 jtwen@tsinghua.edu.cn +1.613.276.2560 scycheng@gmail.com |
| Source: | Tsinghua University and Nanjing Yuyan Information Technology Ltd. |  |  |

_____________________________

# Abstract

In this paper, we present an improved algorithm for decoding video bitstreams with time-varying visual quality. The algorithm extracts information available to the decoder from a high visual quality segment of the clip that has already been received and decoded, but was encoded independently from the current lower quality segment. The proposed decoder is capable of significantly improving the Quality of Experience of the user without incurring significant delays and overhead to the storage and computational complexities of both the encoder and the decoder, or loss of coding efficiency. We present simulation results using the HEVC reference encoder and standard test clips, and discuss areas of improvements to the algorithm.

[Note that the associated paper was submitted to IEEE JSTSP.]

# Introduction

Video encoding and communications systems have traditionally been designed under the assumption that the encoder has a much higher computational power and much larger storage than the decoder. With the ever widening popularity of mobile multimedia applications, especially with user generated content, this assumption is no longer valid. Many widely watched clips on YouTube were captured on mobile phones, but are played back not only on mobile phones or tablets but also on smart TVs, smart set-top boxes, as well as laptop and desktop computers, all of which may possess much more computational and storage resources than the mobile phone on which the video clip was originally captured and encoded.

Due to the high complexity associated with implementing a state-of-the-art video encoding system using the H.264/AVC and especially the up-coming HEVC standard, encoders found in mobile devices, although compliant to the standard syntaxes, were often designed with sub-optimal implementations of only small subsets of the numerous encoding tools supported by the standard. Typical video encoding tools that are not fully implemented include sub-pel motion estimation (ME), many Inter partition sizes and/or Intra prediction directions, and etc. The number of reference frames used in the ME is often limited. In addition, the algorithms for selecting the tools that are supported, for conducting rate-distortion optimized ME, quantization, mode decision, frame prediction type decision and group of picture (GOP) reference structures are also drastically simplified in many encoding systems. Various implementation constrains (e.g. cache sizes) as well as application requirements (e.g. video conferencing or live streaming) make it very difficult to optimize bit rate allocation globally for the entire clip.

Furthermore, when the bitstreams generated by such encoders are streamed over the network, in response to network bandwidth variations, the streaming servers will often adjust the rate at which the clip is encoded downward so as to prevent the playback on the receiving device from stalling.

All of the above factors contribute to losses in coding efficiency as well as visual quality variations in the bitstream that the playback device receives. It is desirable, therefore, to come up with a system that can improve the quality of experience (QoE) of the end user given all the limitations, constraints and quality variations present in the content encoding, transcoding, streaming and playback processes.

In this submission, we present an improved algorithm for decoding video bitstreams with time-varying qualities. The algorithm utilizes information received by the decoder in a segment of the clip that 1) has already been received and decoded, but 2) was encoded independently from the current segment, and 3) has a higher visual quality than the current segment. By extracting information contained in such a segment that is available to the decoder but was not taken advantaged by the encoder, the proposed decoder is capable of significantly improve the QoE of the user without incurring significant overhead to the storage and computational complexities of both the encoder and the decoder, or introducing significant delays or losses to coding efficiency.

The full description of the improved algorithm compared to JVT-L0455 is described in the attached JSTSP paper along with the results.

# Proposal

A PSNR_Y SEI message is proposed, which may be associated with individual coded pictures.  It indicates the Y PSNR (in dB) for the current access unit. It has a payload of 2 bytes.

## No display SEI message syntax and semantics.

| psnr_y ( payloadSize ) { | Descriptor |
| --- | --- |
| psnr_y | u(16) |
| } |  |

PSNR_Y SEI message semantics

The PSNR_Y SEI message indicates that PSNR of the Y component of the current picture.

Table 1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| .. |  |
| PSNR_Y | - The access unit containing the SEI message |

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| … |  |
| else if( payloadType = = 134 ) |  |
| region_refresh_info( payloadSize ) |  |
| else if( payloadType = = 135 ) |  |
| psnr_y( payloadSize ) | u(16) |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| … |  |

# Patent rights declaration(s)

Tsinghua University and Nanjing Yuyan Information Technology Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).